3GPP TSG-RAN WG4 Meeting #115	R4-2506888

Malta, 19th – 23rd May, 2025

Agenda item:	7.1.2.3.4

Source:	Intel Corporation

Title:	Views on SRS insertion loss compensation and reporting enhancements

Document for:	Discussion

# Introduction

The Rel-19 WID on UE RF enhancements for NR FR1/FR2 and EN-DC Phase 4 was approved in RAN#103 with the following objectives related to 6Rx for handheld and FWA UEs [1]:

### 4.1 Objective of Core part WI The objectives of core part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: … 6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. …

The topics of SRS insertion loss imbalance were discussed in previous meetings with agreements provided in [2-8].  In this contribution we provide further views on the topic.

# Discussion

In RAN4 #115 the following further agreements on SRS IL compensation solutions were reached [8]:

Topic #4: SRS IL imbalance ### Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: - No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions. - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19 Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx Agreement - FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs Issue 4-1-3: SRS IL imbalance solution as optional feature Agreement - If any new requirements or procedures for SRS IL handling are defined in Rel-19, they will be defined as optional UE features ### Sub-topic 4-2: SRS IL imbalance issue solutions Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

## SRS IL imbalance UE self-compensation and SRS output power testing

The UE self-compensation is expected to be the baseline solution to handle the SRS IL issue, and the general approach should be to make sure that UE transmits SRS at the maximum power level whenever possible to make sure receiver side SNR is maximized. We recommend clarifying the principles for SRS IL compensation as follows:

- For SRS transmissions Rel-19 UE shall perform self-compensation of SRS IL up to its configured maximum output power

  - UE shall perform full or partial self-compensation of the SRS IL on every SRS resource (SRS antenna port) whenever it has power headroom relative to the configured maximum Tx power.

  - The compensation shall be performed for any SRS transmission power level in TS 38.213.

Proposal #1:	Specify solution for UE to perform SRS IL pre-compensation based on the following principles

  - For SRS transmissions Rel-19 UE shall perform self-compensation of SRS IL up to its configured maximum output power

    - UE shall perform full or partial self-compensation of the SRS IL on every SRS resource (SRS antenna port) whenever it has power headroom relative to the configured maximum Tx power.

    - The compensation shall be performed for any SRS transmission power level in TS 38.213.

    - No RAN1/4 specification impacts

## SRS output power testing

The existing specifications generally require that UE performs SRS IL self-compensation. However, some current UEs may not follow this behavior in scenarios with PSRS < PCMAX. Thus, we propose new Rel-19 optional requirements and a testing framework to ensure proper UE implementation for legacy UEs. The framework aims to:

- Ensure UEs adhere to current RAN1/4 power control equations for consistent SRS transmission power across different transmissions and Tx antennas.

- Cover a wide range of output transmit power configurations, specifically scenarios with PSRS,PC ≤ PCMAX,L.

- Define optional new requirements in the Rel-19.

Proposal #2:	Introduce SRS output power testing framework to guarantee proper UE behavior based on existing specifications at least for PSRS < PCMAX.

## SRS IL imbalance UE assistance and reporting

Regarding signalling aspects, the following general approaches were identified in the previous meeting:

- Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation

  - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed

- Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility

  - Further check whether solution 2 is in the scope or not

Both mechanisms allow the network to obtain more precise information about the actual SRS transmission power imbalance between different transmit branches. The Solution 1 provides network a basic information on UE capability to perform SRS IL compensation. In our view the Solution 1 approach may need to be extended to provide information on the UE SRS IL (∆TRxSRS,UE), which may include either a static value (maximum SRS IL for the given UE) or more dynamic information on the actual SRS IL used by the UE. The respective information can assist the NW to assess the power imbalance and perform the compensation on its side to derive DL channel estimates. To simplify the signalling framework it is recommended to consider semi-static reporting.

The Solution 2 approach would imply dynamic power headroom reporting and can provide more precise information on the actual power imbalance / headroom to the NW side. The Type 3 SRS PHR reporting proposed in the previous meetings is already available in RAN1 specifications, but limited to carriers without PUSCH transmissions and additional impacts to introduce reporting for generic case are considered minimal.

In our view, both Solution 1 and Solution 2 are feasible and can be considered for standardization as alternative solutions (semi-static and dynamic signalling methods).

Proposal #3:	Agree on the following UE assistance and reporting framework

    - Introduce UE capability indication on whether UE performs SRS IL self-compensation

    - Introduce semi-static reporting of UE-specific SRS IL value

    - Extend Type 3 SRS PHR reporting to carriers with PUSCH

## Enhancements to PCmax equations to improve SRS IL compensation

As explained in our paper [9], when the effective SRS IL (∆TRxSRS,UE) is better than the minimum requirement (∆TRxSRS,UE < ∆TRxSRS), the UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities if PCMAX < PCMAX,H. To ensure that the UE performs more optimal SRS IL pre-compensation, we recommend adjusting the existing SRS PCMAX equations on PCMAX_L,f,c so that the UE applies the compensation considering UE-specific (or SRS port specific) SRS IL value ∆TRxSRS,UE. In the latter case, UE will be expected to perform self-compensation optimized for the UE-specific insertion loss.

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Proposal #4:	Adjust the PCMAX_L,f,c equations to include UE-specific SRS-IL component:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

## SRS IL applicability to 2Rx/4Rx/6Rx/8Rx UEs.

The considered framework is generic, and we do not see a motivation to limit it to 8Rx cases only. It is recommended to make it applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Furthermore, given the potential impact on UE implementation it is recommended to introduce new Rel-19 features as optional.

Proposal #5:	Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

# Conclusion

In this paper we provide views on the insertion loss imbalance across SRS ports. In summary, the following proposals are made:

Proposal #1:	Specify solution for UE to perform SRS IL pre-compensation based on the following principles

  - For SRS transmissions Rel-19 UE shall perform self-compensation of SRS IL up to its configured maximum output power

    - UE shall perform full or partial self-compensation of the SRS IL on every SRS resource (SRS antenna port) whenever it has power headroom relative to the configured maximum Tx power.

    - The compensation shall be performed for any SRS transmission power level in TS 38.213.

    - No RAN1/4 specification impacts

Proposal #2:	Introduce SRS output power testing framework to guarantee proper UE behavior based on existing specifications at least for PSRS < PCMAX.

Proposal #3:	Agree on the following UE assistance and reporting framework

    - Introduce UE capability indication on whether UE performs SRS IL self-compensation

    - Introduce semi-static reporting of UE-specific SRS IL value

    - Extend Type 3 SRS PHR reporting to carriers with PUSCH

Proposal #4:	Adjust the PCMAX_L,f,c equations to include UE-specific SRS-IL component:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Proposal #5:	Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

# References

- RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#103, March 2024

- R4-2406585 “WF on requirements for 6Rx”, AT&T, RAN4#110bis, April 2024

- R4-2410751 “WF on requirements for 6Rx”, AT&T, RAN4#111, May 2024

- R4-2414285 “WF on requirements for 6Rx”, AT&T, RAN4#112, August 2024

- R4-2417209 “WF on requirements for 6Rx”, AT&T, RAN4#112bis, October 2024

- R4-2420376 “WF on requirements for 6Rx”, AT&T, RAN4#113, November 2024

- R4-2503014 “WF on requirements for 6Rx”, AT&T, RAN4#114, February 2025

- R4-2505101 “WF on 6Rx RF requirements and SRS IL imbalance”, Intel Corporation, RAN4#114bis, April 2025

- R4-2504163 “Views on SRS insertion loss compensation and reporting enhancements” , Intel Corporation, RAN4#114bis, April 2025