3GPP TSG-RAN WG4 Meeting #115		R4-2506914
Malta, MT, 19th – 23rd May, 2025

Title:	TP to TR38.792 on MSD definition approach for PC1.5 with HPUE on both FDD and TDD

Source:	OPPO

Agenda item:	6.8.5

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce the MSD definition approach for PC1.5 with HPUE on both FDD and TDD based on the approved TR38.792 v.0.1.0.

- Text proposal

<Start of text proposal>

1	Scope

The present document is a technical report for UE requirements for PC1.5 inter-band UL CA and DC with HPUE in both FDD and TDD bands under Rel-19 time-frame. The purpose is to gather the relevant background information and studies of these 3Tx band combinations.

This TR contains the RF requirements of band combination specific part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

[4]	WI description, RP-231719: "WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC"

[5]	WI description, RP-232674: "New WID: Rel-18 new basket WI for 3Tx inter-band UL CA and EN-DC"

[6]	WI description, RP-241679: "New WID: Rel-19 HPUE (PC1.5 or 2) for NR intra-band CA or NR inter-band CA/DC band combinations with/without NR SUL"

[7]	3GPP R4-2504614 MSD for 3Tx UL CA combinations with 2Tx FDD + 1Tx TDD, T-Mobile USA, RAN4#114bis 2025-04

[8]	3GPP R4-25xxxxx R19 MSD for PC1.5 with HPUE on both FDD and TDD, OPPO, RAN4#115 2025-05

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3Tx: UE transmit with one Tx chain on one band and two Tx chains on the other band in CA or EN-DC band combinations.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CA	Carrier Aggregation

DC	Dual Connectivity

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

4	Background

4.1	Introduction

General requirements for 3Tx feature was introduced in Rel-18 RAN4 specification 38.101-1 and 38.101-3 via WI [4]. After that, some band combinations are proposed and introduced via Rel-18 basket WI [5]. In Rel-19, basket WI [6] will introduce more band combination for PC1.5 with high power on both FDD and TDD bands.

The present document is a technical report for the Rel-19 basket WI and covers band combinations for PC1.5 with high power on both FDD and TDD bands.

It covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination) mainly MSD analysis caused by harmonic mixing, cross band leakage and IMD issue.

For the band combinations whose 2Tx MSD is not specified in the spec yet, 3Tx MSD needs to be evaluated. And it is encouraged to align the requirements between 2Tx and 3Tx band combinations.

4.2	MSD definition approach for IMD

In current spec, the MSD of PC1.5 for some CA/DC band combinations have already been defined. For these band combinations, the power capability in each band is PC3 FDD 1Tx + PC1.5 TDD 2Tx. And the MSD caused by IMD was defined based on the power configuration of 23dBm FDD + 27.8dBm TDD due to the power capability limitation in each band.

In this TR, the band combinations with power capabilities PC2 FDD + PC2/1.5 TDD are targeted. It can be seen that the power capability of each band is different from Rel-18 3Tx CA/DC band combinations, consequently, the MSD needed will be different. Based on analysis in [7,8], the MSD for 26dBm FDD + 26dBm TDD power configuration is 10dB larger than 23dBm FDD + 27.8dBm TDD power configuration which makes it not proper to directly reuse the MSD already defined for PC3 FDD 1Tx + PC1.5 TDD 2Tx even the total power class is same.

Therefore, for the PC1.5 CA/DC band combinations with power capability of PC2 FDD + PC2/1.5 TDD, the MSD caused by IMD needs to be analysed based on the power configuration of 26dBm FDD + 26dBm TDD power configuration in this TR.

4.3	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	PC1.5 FDD+TDD CA band combinations with high power on both FDD and TDD bands

5.x	CA_nX-nY (Example)

5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band nX+nY

| NR CA Band combination | NR Band |
| --- | --- |
| CA_nX-nY | nX, nY |

5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE CA Power Class

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_nXA-nYA | 29a | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE a: The maximum output power in NR band X and band Y are PC2 or PC1.5. |  |  |  |  |  |  |

5.x.3	MSD scenario studies

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ nX-nY.

Table 5.x.3-1: MSD scenarios for CA_nX-nY with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | M order of nY UL | N order of nX DL | Yes or No? |
|  | … | … | … |
| Cross band leakage | nY UL | nX DL | Yes or No? |
|  | … | … | … |
| IMD | IMD order (M order of nX UL + N order of nY UL) | nX DL | Yes or No? |
|  | … | … | … |

Editor's note: only list the MSD scenario that exists for this band combination.

Editor's note: For the IMD scenario, UE Tx power for each band shall be set at min{26dBm, Pcmax,L,f,c}.

5.x.4	REFSENS requirements

Editor's note1: Make it clear whether the existing 2Tx requirements are reused or newly analysed due to for example missing of the 2Tx requirements.

Editor's note2: If reused, just say to reuse the existing 2Tx MSD requirements for MSD type harmonic mixing, or cross band leakage or IMD.

Editor’s note3: If MSD is newly specified, it would be useful to give some key parameters as reference.

5.x.4.1	REFSENS requirements for total power class 1.5

<End of text proposal>