3GPP TSG-RAN WG4 Meeting #115													R4-2506922
Malta, MT, 19th – 23rd May, 2025

Title:	R19 MPR for PC1.5 contig CA with NC RB allocation

Source:	OPPO

Agenda item:	7.1.2.1.2

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 contiguous CA MPR with NC RB allocation based on WF [1].

- Non-Contig RB allocation MRP

In last meeting, the NC RB allocation MPR have been discussed, and in general, companies would like to keep similar difference as contiguous RB allocation for PC1.5 and PC2, i.e. around 0.5dB difference. However, there are some proposals have 1-1.5dB difference between PC1.5 and PC2. Then in the tentative values the results have 1.5dB gap in the table below.

With some offline discussions it was mentioned that the larger values are based on tightened ACLR/SEM for PC1.5 than PC2 with the consideration of leave room for RFIC driving PA. To understand how this tightened ACLR/SEM will impact the MPR results, some simulations have been done as below.

The comparison configurations are:

- CA class C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 30khz

- LO number is 1LO+TxD

- 64QAM modulation in both CCs

- ACLR/SEM requirements tightened with 0.5dB/1dB/1.5dB

The MPR results as shown in Annex A can be summarized as below table 1.

Table 1 max delta MPR between tightened ACLR/SEM and original ACLR/SEM

|  |  | tight 0.5dB ACLR/SEM - original | tight 1dB ACLR/SEM - original | tight 1.5dB ACLR/SEM - original |
| --- | --- | --- | --- | --- |
| DFT-S-OFDM | Inner | 0.3 | 0.7 | 0.9 |
|  | Outer 1 | 0.4 | 0.4 | 0.7 |
|  | Outer 2 | 0.4 | 0.4 | 0.4 |
| CP-OFDM | Inner | 0.1 | 0.5 | 0.6 |
|  | Outer 1 | 0.4 | 0.4 | 0.4 |
|  | Outer 2 | 0.4 | 0.5 | 0.5 |

From this result it can be understood that if tight the PC1.5 ACLR/SEM requirements with 1.5dB in the simulation/measurement, but keep PC2 as original ACLR/SEM, then the MPR difference between PC1.5 and PC2 will be as below table 2.

From this table, it can be seen that if tighten 1.5dB ACLR/SEM, the PC1.5 MPR is around 1~1.5dB comparing to PC2 MPR.

Table 2 delta MPR between PC1.5 and PC2 with tight ACLR/SEM in PC1.5

|  |  | tight 0.5dB ACLR/SEM - original | tight 1dB ACLR/SEM - original | tight 1.5dB ACLR/SEM - original |
| --- | --- | --- | --- | --- |
| DFT-S-OFDM | Inner | 0.8 | 1.2 | 1.4 |
|  | Outer 1 | 0.9 | 0.9 | 1.2 |
|  | Outer 2 | 0.9 | 0.9 | 0.9 |
| CP-OFDM | Inner | 0.6 | 1 | 1.1 |
|  | Outer 1 | 0.9 | 0.9 | 0.9 |
|  | Outer 2 | 0.9 | 1 | 1 |

Observation 1:   If tighten ACLR/SEM by 1.5dB for PC1.5 while keep ACLR/SEM unchanged for PC2, the PC1.5 MPR is around 1~1.5dB higher comparing to PC2 MPR.

One question that has been discussed in last meeting is whether the PC2 MPR simulation/measurement has already considered this 1.5dB ACLR/SEM tighten when MPR requirements were defined at that time. From the offline discussion it seems this is not that obvious.

Observation 2:   It is unclear whether ACLR/SEM tighten has been considered in PC2 MPR evaluation in the past.

At this moment, it might be safe to assume that normal ACLR/SEM has been used in PC2 MPR evaluation. With above analysis, it is clear that if we assume that current PC2 MPR is based on the 3GPP defined ACLR/SEM requirements, while in PC1.5 we leave 1.5dB room for RFIC implementation (i.e. tighten ACLR/SEM by 1.5dB) then the MPR for PC1.5 can be as below table, i.e. the upper value in the WF.

Proposal 1:         Consider 1~1.5dB higher MPR for PC1.5 NC RB allocation than PC2. Values can be found in below table.

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 9.5 | 17 |
|  | QPSK | 5 | 9.5 | 17 |
|  | 16QAM | 5 | 9.5 | 17 |
|  | 64QAM | 7 | 9.5 | 17 |
|  | 256QAM | 8.5 | 9.5 | 17 |
| CP-OFDM | QPSK | 5.5 | 10 | 17 |
|  | 16QAM | 5.5 | 10 | 17 |
|  | 64QAM | 7 | 10 | 17 |
|  | 256QAM | 9.5 | 10 | 17 |

- Conclusion

This paper shared more MPR simulation results for PC1.5 intra-band contiguous CA, got below proposals.

Observation 1:   If tighten ACLR/SEM by 1.5dB for PC1.5 while keep ACLR/SEM unchanged for PC2, the PC1.5 MPR is around 1~1.5dB higher comparing to PC2 MPR.

Observation 2:   It is unclear whether ACLR/SEM tighten has been considered in PC2 MPR evaluation in the past.

Proposal 1:         Consider 1~1.5dB higher MPR for PC1.5 NC RB allocation than PC2. Values can be found in below table.

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 9.5 | 17 |
|  | QPSK | 5 | 9.5 | 17 |
|  | 16QAM | 5 | 9.5 | 17 |
|  | 64QAM | 7 | 9.5 | 17 |
|  | 256QAM | 8.5 | 9.5 | 17 |
| CP-OFDM | QPSK | 5.5 | 10 | 17 |
|  | 16QAM | 5.5 | 10 | 17 |
|  | 64QAM | 7 | 10 | 17 |
|  | 256QAM | 9.5 | 10 | 17 |

- Reference

[1] R4-2505100 WF on HPUE RF requirements, Qualcomm, #114bis 2025-04

- Annex A: MPR comparison for NC RB allocation

Table 1 CP-OFDM non-contig RB allocation: CA Class C

|  | CBW1 | CBW2 | NRB1 | NRB2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS (khz) | Inner/ Outer | delta = tight 0.5dB -original | delta = tight 1dB -original | delta = tight 1.5dB - original |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT | 60 | 100 | 162 | 273 | 1 | 109 | 1 | 52 | 30 | inner | 0.2 | 0.5 | 0.7 |
| DFT | 60 | 100 | 162 | 273 | 1 | 83 | 1 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 |
| DFT | 100 | 100 | 273 | 273 | 1 | 183 | 1 | 52 | 30 | inner | 0.2 | 0.5 | 0.7 |
| DFT | 100 | 100 | 273 | 273 | 1 | 140 | 1 | 1 | 30 | inner | 0.2 | 0.2 | 0.5 |
| DFT | 60 | 100 | 162 | 273 | 1 | 120 | 1 | 118 | 30 | outer1 | 0.3 | 0.3 | 0.3 |
| DFT | 60 | 100 | 162 | 273 | 1 | 82 | 1 | 118 | 30 | outer1 | 0.4 | 0.4 | 0.4 |
| DFT | 60 | 100 | 162 | 273 | 1 | 43 | 1 | 118 | 30 | outer1 | 0.4 | 0.4 | 0.7 |
| DFT | 60 | 100 | 162 | 273 | 1 | 42 | 1 | 118 | 30 | outer1 | 0.0 | 0.4 | 0.4 |
| DFT | 60 | 100 | 162 | 273 | 1 | 1 | 1 | 1 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 182 | 1 | 182 | 30 | outer1 | 0.4 | 0.4 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 138 | 1 | 182 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 110 | 1 | 150 | 30 | outer1 | 0.4 | 0.4 | 0.7 |
| DFT | 100 | 100 | 273 | 273 | 1 | 109 | 1 | 163 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 1 | 1 | 1 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| DFT | 60 | 100 | 162 | 273 | 1 | 41 | 1 | 120 | 30 | outer2 | 0.0 | 0.0 | 0.4 |
| DFT | 60 | 100 | 162 | 273 | 1 | 1 | 1 | 272 | 30 | outer2 | 0.0 | 0.4 | 0.4 |
| DFT | 60 | 100 | 162 | 273 | 1 | 1 | 1 | 180 | 30 | outer2 | 0.0 | 0.0 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 108 | 1 | 180 | 30 | outer2 | 0.0 | 0.4 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 1 | 1 | 272 | 30 | outer2 | 0.4 | 0.4 | 0.4 |
| DFT | 100 | 100 | 273 | 273 | 1 | 1 | 1 | 100 | 30 | outer2 | 0.3 | 0.3 | 0.3 |

|  | CBW1 | CBW2 | NRB1 | NRB2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS (khz) | Inner/ Outer | delta = tight 0.5dB -original | delta = tight 1dB -original | delta = tight 1.5dB - original |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP | 60 | 100 | 162 | 273 | 1 | 110 | 1 | 51 | 30 | inner | 0.1 | 0.5 | 0.6 |
| CP | 60 | 100 | 162 | 273 | 1 | 84 | 1 | 1 | 30 | inner | 0.1 | 0.3 | 0.5 |
| CP | 60 | 100 | 162 | 273 | 1 | 83 | 1 | 1 | 30 | inner | 0.1 | 0.5 | 0.6 |
| CP | 100 | 100 | 273 | 273 | 1 | 184 | 1 | 51 | 30 | inner | 0.1 | 0.3 | 0.6 |
| CP | 60 | 100 | 162 | 273 | 1 | 120 | 1 | 118 | 30 | outer1 | 0.4 | 0.4 | 0.4 |
| CP | 60 | 100 | 162 | 273 | 1 | 82 | 1 | 118 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 60 | 100 | 162 | 273 | 1 | 43 | 1 | 118 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 60 | 100 | 162 | 273 | 1 | 42 | 1 | 118 | 30 | outer1 | 0.0 | 0.4 | 0.4 |
| CP | 60 | 100 | 162 | 273 | 1 | 1 | 1 | 1 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 182 | 1 | 182 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 138 | 1 | 182 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 110 | 1 | 150 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 109 | 1 | 163 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 1 | 1 | 1 | 30 | outer1 | 0.0 | 0.0 | 0.4 |
| CP | 60 | 100 | 162 | 273 | 1 | 41 | 1 | 120 | 30 | outer2 | 0.0 | 0.5 | 0.5 |
| CP | 60 | 100 | 162 | 273 | 1 | 1 | 1 | 272 | 30 | outer2 | 0.0 | 0.4 | 0.4 |
| CP | 60 | 100 | 162 | 273 | 1 | 1 | 1 | 180 | 30 | outer2 | 0.0 | 0.0 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 108 | 1 | 180 | 30 | outer2 | 0.0 | 0.4 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 1 | 1 | 272 | 30 | outer2 | 0.4 | 0.4 | 0.4 |
| CP | 100 | 100 | 273 | 273 | 1 | 1 | 1 | 100 | 30 | outer2 | 0.0 | 0.0 | 0.4 |