3GPP TSG- WG4 Meeting # 115	R4-2506967

# St Julian’s, Malta, 19th – 25th May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 EESS

Agenda Item:	6.17.2

Document for:	Approval

Introduction

In RAN4#113, it is mandatory for new UEs from Rel-15 to support NS_205 and accordingly to report modifiedMPRbehaviour bit for NS_205. In RAN4#114, the draftCR R4-2502851 was endorsed and CR R4-2501408 was agreed to capture the UE RF requirements for Rel-19 EESS, introducing NS_205/CA_NS_205 and updating NS_202/CA_NS_202 with additional spectrum emission -5dBm/200MHz, and the corresponding A-MPR values. In RAN4#114bis, the draftCR R4-2505131 for Rel-15 was endorsed. But there may be some parts in the endorsed draftCR to invite misunderstandings among companies, and they should be fixed in this meeting.

Discussion

Handing of EESS requirements in Rel-19

For modifiedMPR-Behavior corresponding to NS_202/CA_NS_202 and NS_205/CA_NS_205, the TS number and version number listed in [1]. In theory, if the additional spurious emission is unchanged in the future version or release, no need for the engineer to refer to v19.0.0.

For example, in v19.5.0 (as the latest spec version), if the spectrum emission limit for NS_202 is same as that in v19.0.0, v19.0.0 can be referred to by older releases specifications. On the contrary, if the spectrum emission limit for NS_202 is updated in v19.5.0, only the latest version should be referred to.

There is no point to show the version number in modifiedMPR-Behavior table for the requirement updated or introduced in Rel-19. In addition, numbering of “v19.0.0” in the older releases specifications referring to v19.0.0 in the above example shall be corrected whenever future requirements are updated and it invites extra work.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n257 |
|  | 1 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
|  | 2 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

Proposal 1: In modifiedMPR-Behavior table, the version number for the requirement updated (NS_202) or introduced (NS_205) in Rel-19 should be removed. It is straightforward to refer to the requirements in the latest spec version.

In Rel-19, RAN4 has agreed to update the additional spurious emission requirement from +1dBm/200MHz to -5dBm/200MHz to ensure that all the UEs referring to the latest specification from now on meet tighter requirement of -5dBm/200MHz. Accordingly, we excluded +1dBm/200MHz in the latest spec version for Rel-19. Therefore, the A-MPR requirements for NS_202 corresponding to +1dBm/200MHz should not be in the latest spec version. Besides, the modifiedMPR bit for NS_202/CA_NS_202 shall be set to 1. Since there is only one requirement for NS_202, it is impossible for the bit to be set to 0. The following highlighted part in [1] is unnecessary and need to be removed to avoid ambiguity.

##### 6.2.3.3.3 A-MPR for NS_202 for power class 3 For power class 3, A-MPR for NS_202 shall be 1.0 dB if the corresponding bit of modifedMPR-Behavior for bands n257, n258 as defined in Annex H is set to 0 otherwise 2.0 dB.

Observation 1: There is no requirement for additional spurious emission of +1dBm/200MHz for band n257/n258 in the latest Rel-19 spec version.

Proposal 2: The A-MPR requirements for NS_202/CA_NS_202 corresponding to +1dBm/200MHz should not be in the latest spec version.

Handing of EESS requirements in Rel-15

RAN4 has agreed to mandate NS_202/CA_NS_202 and NS_205/CA_NS_205 since Rel-15. For NS_202, the spurious emission limit is tightened in Rel-19 with -5dBm/200MHz. For the new Rel-15 UE, the requirement of -5dBm/200MHz must be satisfied. To avoid ambiguity, the additional spurious emission, as well as the corresponding A-MPR requirements should be updated in the latest spec version of Rel-15.

Things are different for NS_205. In Wuhan meeting, the example for NS_203 was pointed out, and it was proposed to follow the way of NS_205 to repeat the requirements of NS_205 since Rel-15. However, dating back to the discussion on NS_203, the Rel-15 cat-F CR R4-2016785[4] and Rel-16 cat-A CR R4-2014055 [5] were agreed. That is the reason the NS_203 requirements exist start from Rel-15. The procedure is different for NS_205. We have the Rel-19 cat-B CR [2] agreed first. And in the revised WID [6], the NS_205 introduced in Rel-19 is applied from Rel-15 with functional modification of feature, as cat-C CR.

Currently, in Rel-15 specification, more than one network signalling labels are supported for both band n257 and n258. The table in ‘Annex H (Normative): Modified MPR behaviour’ would be referred to determine which A-MPR requirement is needed. And then, NS_205 pointing to spec version 19.1.0 would be found without ambiguity. The NS_205/CA_NS_205 and the corresponding A-MPR requirements as the Rel-19 feature should not be specified in Rel-15.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n257 |
|  | 1 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
|  | 2 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.6 in latest version of Rel-19 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.6 in the latest version of Rel-19 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

Observation 2: The reason that the requirement of NS_203 was specified in Rel-15 is that the Rel-15 cat-F CR and Rel-16 cat-A CR were agreed. Different from NS_205, the Rel-19 cat-B CR was agreed, and the Rel-15 cat-C draftCR with functional modification of feature has been discussed.

Proposal 3: The example that the requirement of NS_203 was specified in Rel-15 should not be followed by NS_205, given that the procedures of the two network signalling labels are different.

Proposal 4: The NS_205 and the corresponding A-MPR requirements as the Rel-19 feature should not be specified in Rel-15.

Conclusion

In this contribution we further discuss how to specify the requirements for EESS, according to the analysis, we have the following observations and proposals:

Observation 1: There is no requirement for additional spurious emission of +1dBm/200MHz for band n257/n258 in the latest Rel-19 spec version.

Observation 2: The reason that the requirement of NS_203 was specified in Rel-15 is that the Rel-15 cat-F CR and Rel-16 cat-A CR were agreed. Different from NS_205, the Rel-19 cat-B CR was agreed, and the Rel-15 cat-C draftCR with functional modification of feature has been discussed.

Proposal 1: In modifiedMPR-Behavior table, the version number for the requirement updated (NS_202) or introduced (NS_205) in Rel-19 should be removed. It is straightforward to refer to the requirements in the latest spec version.

Proposal 2: The A-MPR requirements for NS_202/CA_NS_202 corresponding to +1dBm/200MHz should not be in the latest spec version.

Proposal 3: The example that the requirement of NS_203 was specified in Rel-15 should not be followed by NS_205, given that the procedures of the two network signalling labels are different.

Proposal 4: The NS_205 and the corresponding A-MPR requirements as the Rel-19 feature should not be specified in Rel-15.

- References

[1] R4-2502851, Draft CR to add new requirements for EESS protection from n257 and n258, RAN4#114, February, 2025

[2] R4-2501408, Big CR on UE RF requirements for WI NR_mmWave_protect, RAN4#114, February, 2025

[3] R4-2505131, draft CR to 38.101-2 EESS (Rel-15), RAN4#114bis, April, 2025.

[4] R4-2016785, EESS protection related requirements for FR2 bands, RAN4#97-e, November, 2020

[5] R4-2014055, EESS protection related requirements for FR2 bands, RAN4#97-e, November, 2020

[6] RP-250818, Revised WID: mmWave in NR: UE spurious emissions and EESS (Earth Exploration Satellite Service) protection, RAN#107, March, 2025.