# 3GPP TSG-RAN WG4 Meeting #115		R4-2506971

# Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.28.2

Source:	Samsung

Title:	TP to TR 38.774 on testability of LP-WUR

Document for:	Approval

# 1  Introduction

In this paper, we provide text proposal to TR 38.774 on testability of LP-WUR.

# 2  Text propsoal

## <<Start of Change>>

# 8	Testability

<Editor’s note: discussions and analysis for testability solutions>

## 8.1	Testability for UE Performance verification

### 8.1.1	General Framework

It has been agreed that same LP-WUS RF requirements will apply to all three RRC states (RRC_Idle, RRC_Inactive, RRC_Connected). Further the metric used for conformance testing is going to be the same as for defining the requirements i.e., 1% MDR of LP-WUS. For demodulation requirements FAR will also be used.

LP-WUR is sole receiver without paired transmitter, so there is no direct loopback for TE to obtain UE’s detection results unless relying on MR. To address this issue, several approaches have been discussed for MDR testing.

- One approach is to count the detection by waking up MR to complete the paging process every time. This approach suffers very long test time. Moreover, additional measure is needed to make sure MR is in off or sleep mode before detection.

- Another approach is to count the detection rate without waking up the MR until all detection is completed. This approach not only has the advantage of significant test time saving, but also provides robust and repeatable test results. On the other hand, additional effort on test mode is needed.

- There is also proposal to test MDR with RRC_Connected state only. According to RAN1 agreed UE capabilities for LP-WUS, there could be UEs supporting RRC_Idle/Inactive mode only and RRC_Connected state is not available for MDR testing.

- One more alternative proposal is to perform functional test and obtain the MDR of PDCCH instead of MDR of LP-WUS. For this approach, LP-WUR ON/OFF status and MR ON/OFF status is up to UE implementation, consequently a successful PDCCH detection can be done without LP-WUR. This uncertainty will make the conformance test results not repeatable. Moreover, this approach actually relaxes the RF requirements of LP-WUR.

By considering both test time and conformance test robustness, RAN4 decides to count the detection rate without waking up the MR until all detection is completed. To achieve this target, a test mode is needed. The test mode should at least be able to force the LP-WUR always on during the MDR test, and to loopback test results to TE after MDR test over. Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

### 8.1.2	other

## <<End of Change>>