3GPP TSG-RAN WG4 Meeting #115	R4-2507001

Malta, MT, 19th – 23rd May, 2025

Source:	China Telecom

Title:	Further discuss RF Rx requirements for PC2 RedCap in FDD bands

Agenda Item:	7.6.2

Document for:	Discussion

- Introduction

This contribution further discusses the REFSENS requirements for PC2 RedCap with 1Rx in FDD bands.

- Discussion

By applying ΔR1R to REFSENS (1Tx (26dBm) /2Rx) as expressed by following formula is a propriate way to specify the PC2 1Rx RedCap UE requirement, because only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

REFSENSPC2 RedCap 1Rx=REFSENSPC2 2Rx/1Tx + ΔR1R

Thus, we have the lastest WF [1] approved with reaching the following agreements regarding PC2 RedCap with 1Rx in FDD bands.

Issue 1-1-1: REFSENS for 1Rx in FD-FDD Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD - Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB. - Companies to evaluate REFSENS for other bands. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. - Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1. - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. Way forward: It is encouraged for companies to provide analysis for option 1 and 2. Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

Another advantage is if ΔR1R is defined in a band agnostic way, then all the bands for PC2 non-RedCap UE could be supported directly by PC2 1Rx RedCap UE without basket work to introduce band by band specially. Thus we give our simple consideration to make ΔR1R apply to group of bands which are under some conditions. The parameter to distinguish REFSENS capability for bands for PC2/PC3 non-RedCap UE is RSD or REFSENS. In last meeting, the issue that ΔR1R may not be correctly specified even for PC3 RedCap UE was  raised for some of bands which the REFSENS requirement is worse due to proximity of UL and DL. At this time, the diversity Rx will dominate the REFSENS, if removing diversity antenna like RedCap UE, then the ΔR1R may be larger than 3dB. It is straightforward to limit the application condition for the ΔR1R Table 7.3I.2-1 even for PC3 RedCap UE.

Observation 1: It is reasonable to limit the application condition for the ΔR1R in Table 7.3I.2-1 for PC3 RedCap UE.

Observation 2: The application condition for the ΔR1R in Table 7.3I.2-1 for PC3 RedCap UE can be decided by REFSENS two antenna port.

Below, we will try to find the value for REFSENS two antenna port, based on which, the ΔR1R in Table 7.3I.2-1 is applicable. The ΔR1R for LTE UE category 0 can be obtained by comparing the REFSESNS specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1.

Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
|  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.5 | -94.5 | -92.7 |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
| 111 | -99 | -96.2 | -94.3 | -80.6 |  |  | FDD |

Table 1: ΔR1R for LTE UE category 0

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
|  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| 2 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | FDD |
| 3 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | FDD |
| 4 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | FDD |
| 5 | 2.5 | 2.5 | 2.5 | 2.5 |  |  | FDD |
| 8 | 2.5 | 2.5 | 2.5 | 2.5 |  |  | FDD |
| 13 |  |  | 3 | 3 |  |  | FDD |
| 20 |  |  | 2.5 | 2.5 | 3 | 3 | FDD |
| 25 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | FDD |
| 26 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |  | FDD |
| 28 |  | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | FDD |
| 39 |  |  | 2.5 | 2.5 | 2.5 | 2.5 | TDD |
| 40 |  |  | 2.5 | 2.5 | 2.5 | 2.5 | TDD |
| 41 |  |  | 2.5 | 2.5 | 2.5 | 2.5 | TDD |
| 111 | 2.5 | 2.5 | 2.5 | 2.5 |  |  | FDD |

Observation 3: At least when the REFSENS for normal UE with 2Rx is larger than 89.5dBm, the ΔR1R values can be kept as 2.5 or 3dB for LTE category 0 or RedCap UE with 1Rx

Proposal 1: Add a note to the Table 7.3I.2-1, saying that requirement is applicable when the corresponding requirement for two antenna port in Table 7.3.2-1a is larger than 89.5dBm

For PC2 RedCap UE, when the self-interference is not severe (in the note 1, we give the value of 89.5dBm REFSENS as the threshold), as the RSD has been specified, the ΔR1R can be reused. For the case the self-interference is severe (e.g. Band n8 20MHz CBW), the ΔR1R may be worsen due to higher transmit power producing larger degradation when removing the diversity antenna.

Proposal 2: For PC2 RedCap UE with 1Rx, reuse and flat 3dB ΔR1R when the band self-interference is not severe, specify the value when the band self-interference is severe.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) for PC3 | ΔR1R (dB) for PC2 |
| --- | --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 | 2.5 |
| FDD band | 5 | 2.51 | 3.01 |
| FDD band | 10, 15, 20 | 3.01 | 3.01 |
| Band n8 | 20 | TBD | TBD |
| Note 1: Applicable for the REFSENS requirement for two antenna port in Table 7.3.2-1a is larger than 89.5dBm |  |  |  |

- Conclusions

In this contribution, we discuss the REFSENS for PC2 RedCap with 1Rx in FDD bands and have the following proposals:

Observation 1: It is reasonable to limit the application condition for the ΔR1R in Table 7.3I.2-1 for PC3 RedCap UE.

Observation 2: The application condition for the ΔR1R in Table 7.3I.2-1 for PC3 RedCap UE can be decided by REFSENS two antenna port.

Observation 3: At least when the REFSENS for normal UE with 2Rx is larger than 89.5dBm, the ΔR1R values can be kept as 2.5 or 3dB for LTE category 0 or RedCap UE with 1Rx

Proposal 1: Add a note to the Table 7.3I.2-1, saying that requirement is applicable when the corresponding requirement for two antenna port in Table 7.3.2-1a is larger than 89.5dBm

Proposal 2: For PC2 RedCap UE with 1Rx, reuse and flat 3dB ΔR1R when the band self-interference is not severe, specify the value when the band self-interference is severe.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) for PC3 | ΔR1R (dB) for PC2 |
| --- | --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 | 2.5 |
| FDD band | 5 | 2.51 | 3.01 |
| FDD band | 10, 15, 20 | 3.01 | 3.01 |
| Band n8 | 20 | TBD | TBD |
| Note 1: Applicable for the REFSENS requirement for two antenna port in Table 7.3.2-1a is larger than 89.5dBm |  |  |  |

- References

R4-2505231, WF on PC2 RedCap UE RF requirements, China Telecom