3GPP TSG RAN WG4 Meeting #115	 R4-2507011

Malta, MT, 19 May – 23 May 2025

Agenda Item:	7.24.4.1

Source: 	Ericsson

Title:	Discussion on UE initiated BM requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the UE initiated BM requirements for NR MIMO Phase 5.

# Discussion

Event triggered measurement requirements

Event triggered measurement delay:

For defining event triggered measurement reporting requirements, RAN4 needs to discuss and decide on the start point, end point, measurement period, the event evaluation time, and the measurement report preparation time.

Regarding the start point and end point, following was agreed in previous meeting [R4-2420096].

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Further, regarding the measurement period definition following WF is agreed in last meeting.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

      - the value of current beam and new beams are the same.

  - For M in L1_measurement periods:

    - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam

    - If timeRestrictionForChannelMeasurement is not configured,

      - Further discuss on:

        - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

        - Option 2: No requirements even the NW can configure it.

        - Option 3: FFS

  - For P (sharing factor) in L1_measurement periods:

    - For SSB based or CSI-RS based,

      - The definition is the same as legacy

      - The value can be the same or different for the current beam and new beam

One open issue in the measurement reporting delay is the M value when the timeRestrictionForChannelMeasurement is not configured. When we checked RAN1 agreements there is no discussion on whether timeRestrictionForChannelMeasurement is supported for UE initiated beam report or not. We suggest discussing it when RAN1 introduces this parameter.

- For UE initiated beam reporting M value is 1. RAN4 to revisit the M value when the timeRestrictionForChannelMeasurement is introduced in RAN1.

Further following WF is agreed.

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

In the last meeting, RAN4 agreed that the P value can differ between the current beam and the new beam. In FR2, the P value determines the sharing of RS occasions when they overlap with GAP and/or SMTC. According to the meeting discussion, either the current beam or the new beam may overlap with GAP and/or SMTC. Despite having the same periodicity, the overlap can result from different offset configurations. Therefore, the new beam may have a different sharing factor than the current beam.

Consider a scenario where P = 1 for the current beam and P = 3 for the new beam, as shown in Figure 1. The measurement period would be longer if the maximum approach is used. However, in the alternative approach, the measurement occasions for the new beam and the current beam are closer together, making it reasonable to compare the previous occasion of the new beam with the latest occasion of the current beam. Since the measurement occasions for the new and current beams differ, the UE will need to compare one set of fresh measurements with one set of previous measurements, which means this method is not disadvantageous.

- If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement period_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement period_newbeam.

Measurement window/counter consideration in UE initiated beam report:

Based on the RAN1 agreement described below, we believe its functionality is similar to that of the TTT timer, with one key difference: events do not need to persist throughout the entire window duration but must occur M number of times. In the legacy approach, the UE can assess the event at the end of the TTT to determine if the event criteria have been met.

In the UEI BM context, the UE must evaluate the event N number of times to confirm that it occurs M number of times. This functionality appears more stringent than the TTT for L3, where the UE can simply check at the end of the TTT to see if the event condition remains satisfied. Therefore, we suggest defining RAN4 requirements to ensure that UE measurement behaviour is tested accurately.

RAN1#120bis agreement: Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

- RAN4 to define UEI BM requirements when the measurement window/counter is configured.

Measurement period when time window is configured:

We think we do not need to define the measurement period in terms of the equation and instead we can specify it as a text saying that after the first event entering condition is met, UE starts the timer with T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window. We think we need not specify the requirement in the equation because within the T_window, there could be N number of measurement occasions and M out of N number occasions needs to be satisfying to trigger the UEI BM report.

- RAN4 to agree on following as the method to specify the requirement when the time window is configured: after the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window

Other components:

In our last meeting, we reached the following agreement:

The event-triggered measurement reporting delay includes the TFirst UL channel.

- TFirst UL channel refers to the time from the RS that triggered the L1 reporting to the next first PUCCH.

We suggest one clarification, with the motivation explained below:

Once the event is triggered, the UE must send an indication on the first UL channel to notify the NW that the event has occurred, within "TFirst UL Channel." After the event is triggered, the UE may require time to prepare the new UCI for transmission on the first UL channel. Typically, the preparation time for the PUCCH or PUSCH is defined in the RAN1 specification, but it is crucial to account for this in the RAN4 requirement design. This is important because the UE might miss the first occasion of the first UL channel if the interval between the RS that triggers the event, and the first UL channel grant is insufficient for the UE to prepare and send the new UCI on the first UL channel.

- RAN4 to agree TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1.

Inter-cell scenario for UEIBM

Whether to consider intra-cell and inter-cell scenario for defining event triggered reporting for CSI-RS was discussed in last meeting. We provide our view on the inter-cell scenario in this section.

In Rel-15, SSB based L1-RSRP requirements were defined for intra-cell scenario where all the SSB in a cell are associated with single PCI. In Rel-17, RAN1 introduced inter-cell beam management and inter-cell multi-TRP scenario where SSB of the cell can be associated with different PCI, where the SSB with different PCI are typically transmitted from different TRP associated with same serving cell. In Rel-17, RAN4 defined L1-RSRP measurement requirements for inter-cell multi-TRP scenario where one TRP can be associated with serving cell PCI and other TRP can be associated with different PCI than the serving cell PCI. However, in that WI, only SSB was considered, and CSI-RS based inter-cell L1-RSRP measurements were not considered during that WI.

CSI-RS based L1 measurement requirements are defined in Rel-15 and CSI-RS is assumed to be QCL’ed with SSB. As per the applicability of CSI-RS requirements shown below, SSB which is QCLed with CSI-RS can be intra-cell and inter-cell.

For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON.

One of the first agreement RAN1 made for this work item is shown below. Since supporting intra-cell and inter-cell scenario was one of the first agreement by RAN1, it signifies the importance of considering both the scenarios.

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Hence, we propose

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

When UE is configured with CSI-RS measurements in inter-cell scenario, SSB associated with CSI-RS may be from different TRP (different PCI) and in FR2, they may need to be received with different QCL Type D. Since multi-Rx may not always be assumed, UE may be able to measure only one TRP CSI-RS at a time when the CSI-RS from different TRPs are overlapping in time. RAN4 may need to define sharing or scaling rules for the inter-cell CSI RS measurements. One may argue that SSB based inter-cell L1-RSRP measurements are defined during a dedicated WI in Rel-17 and may contain big workload to define CSI-RS based inter-cell L1-RSRP measurement requirements. We think that RAN4 can reuse the principles of sharing agreed for SSB based measurements to derive the requirements for CSI-RS based inter-cell L1-RSRP measurements.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

- RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

Impact on unified TCI state switching delay

Currently RAN1 supports two alternatives for unified TCI state activation. Based on the use case for which the events are defined, we think the RAN1 defined events support the existing TCI state activation framework listed below.

Alternative 1:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE activating some of the TCI states from the RRC configured TCI states

- Step 3: DCI indicating one of the MAC CE activated TCI states.

Alternative 2:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE directly activating a TCI state from the RRC configured TCI states

In some implementations, after receiving the measurement report when the event 2 trigger conditions are met, if a new beam which was not in the active TCI state list is better than the current beam, then NW may decide to switch the current beam to the new beam. In this scenario, Alternative 1 may cause extra delay for switching the current beam to best beam. However, we think that this can be supported with Alternative 2 as described above. Hence, we do not see a need for new requirements definition.

From the WF, we understand that RAN1 is discussing enhancements for delay reduction in the case of DCI-based TCI state switching. We can revisit this after RAN1 reaches an agreement. Historically, RAN4 has not defined any additional delays beyond those specified by RAN1. Therefore, if RAN1 agrees to enhance the DCI-based TCI state switching delay, it should be straightforward for RAN4 to incorporate that into our specifications.

- For MAC CE based TCI state switching, existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- For DCI based TCI state switching, wait for RAN1 progress.

Other issue discussed in last meeting was whether/how to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time. The proposals discussed are listed below.

- Option 1: (Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 2:

  - For such case, No RAN4 requirements.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Regarding this issue, RAN1 reached following agreement, and we think it is sufficient. We do not see a need to add additional applicability rules in the RAN4 specification.

Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. FFS: Further details on the update (if necessary)

- When the indicated TCI state is changed, counter resetting mechanism is defined in RAN1 and RAN4 not to define additional applicability rules when the indicated TCI state change.

UE capability for UE IBM

Last meeting we had discussion on how many events UE can support and if there should be a capability defined. The proposals discussed were listed below.

Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities. - Option 1: - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

We believe the intention behind option 1 is that, unlike L3 measurement where the number of cells the UE needs to detect and evaluate may be limited, there is no such limitation here. In MIMO, the number of SSB/CSI-RS the UE can measure is already a RAN1 defined capability, and the number of reporting configurations are also a defined capability or limited by specification. Therefore, we do not see the need to define a separate capability for event evaluation criteria, as the number of report configurations and the number of RS the UE needs to measure are limited.

One potential limitation could be the number of timers the UE can maintain for event-triggered reporting. If this is a bottleneck, we believe RAN1 is the appropriate WG to discuss this, as the number of RS and the number of reporting configurations are defined in RAN1. Consequently, we do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification.

- Do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification. If a capability for the number of events to supported is needed, it shall be defined by RAN1.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- For UE initiated beam reporting M value is 1. RAN4 to revisit the M value when the timeRestrictionForChannelMeasurement is introduced in RAN1.

- If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement period_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement period_newbeam.

- RAN4 to define UEI BM requirements when the measurement window/counter is configured.

- RAN4 to agree on following as the method to specify the requirement when the time window is configured: after the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window.

- RAN4 to agree TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1.

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

- RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- For MAC CE based TCI state switching, existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- For DCI based TCI state switching, wait for RAN1 progress.

- When the indicated TCI state is changed, counter resetting mechanism is defined in RAN1 and RAN4 not to define additional applicability rules when the indicated TCI state change.

- Do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification. If a capability for the number of events to supported is needed, it shall be defined by RAN1.

# References

[1]		R4-2420096, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung

[2] 	RP-234007, New WID: NR MIMO Phase 5