3GPP TSG-RAN WG4 Meeting #115	R4-2507029

# Malta, MT, 19 - 23 May, 2025

Title: 	Discussion on IoT NTN Phase3 demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.32.6

Document for:	Discussion

Background

During RAN#106 meeting, revised WID [1] on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 was approved. In this contribution, we share our view on IoT NTN Phase3 demodulation performance requirements.

Discussion

Based on RAN1 agreements, 3.75kHz SCS symbol-level OCC length 2 for NPUSCH format 1 and 15kHz SCS slot-level OCC length 2 for NPUSCH format 1 are introduced for IoT-NTN. Considering that there are some impacts on baseband processing, we propose to define corresponding requirements for above features.

Define corresponding requirements for following features.

- 3.75kHz SCS symbol-level OCC length 2 for NPUSCH format 1

- 15kHz SCS slot-level OCC length 2 for NPUSCH format 1

As a summary, the new defined requirements are proposed as following Table 2-1.

Table 2-1: Minimum requirements for NPUSCH format 1 with OCC length 2

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | TBD | 128 Hz | 256 | 2 | 70% |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | TBD | 128 Hz | 256 | 2 | 70% |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | TBD | 8 Hz | 128 | 8 | 70% |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | TBD | 8 Hz | 128 | 8 | 70% |

Propose to define the new performance requirements as summarized in Table 2-1.

Proposals

In this contribution, we share our view on IoT NTN Phase3 demodulation performance requirements. Our observations and proposals are:

- Define corresponding requirements for following features.

- 3.75kHz SCS symbol-level OCC length 2 for NPUSCH format 1

- 15kHz SCS slot-level OCC length 2 for NPUSCH format 1

Propose to define the new performance requirements as summarized in Table 2-1:

Table 2-1: Minimum requirements for NPUSCH format 1 with OCC length 2

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | TBD | 128 Hz | 256 | 2 | 70% |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | TBD | 128 Hz | 256 | 2 | 70% |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | TBD | 8 Hz | 128 | 8 | 70% |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | TBD | 8 Hz | 128 | 8 | 70% |

Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#106, MediaTek Inc.