3GPP TSG-RAN WG4 Meeting #115	R4-2507039

Malta, MT, 19th-23rd May, 2025

Title: 	Overviews on demodulation requirements for R19 LP-WUS

Source: 	Huawei, HiSilicon

Agenda item:	7.28.7

Document for:	Discussion

Background

According to the TU assign, it’s time to start the performance part for R19 LP-WUS discussion in this meeting. This contribution provides our accordingly general views.

- Discussions

Test applicability

RF testability is still on discussion in RF session, according to [1], the common understanding is that RF performance is testable but test model design as well as the details of test model is up to RAN5:

Issue 4-1-1: Whether LR can be tested directly Agreements: - Down select below two approaches for LP-WUR UE RF requirements: - Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated. - The note depends on RAN1 conclusion - Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR Issue 4-1-4: UE test mode for LP-WUR Agreements: - Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

Our understanding is that demodulation test framework is similar as RF performance test since the common test metric is MDR (Miss detection ratio) and FAR (False alarm ratio). Therefore, it can be concluded that demodulation test is also testable and it’s reasonable to define related requirements.

Observation 1:  Demodulation test framework is similar as RF performance test since the common test framework is MDR and FAR.

From the study in RAN1, LP-WUS can operate in both RRC_IDLE/INACTIVE and RRC_CONNECTED modes. It is noted that the process for LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode are different. In addition, supporting IDLE/INACTIVE mode and CONNECTED mode is based on UE capabilities, which means a UE supporting LP-WUS operation in IDLE/INACTIVE mode does not necessarily to support operation in CONNECTED mode, and vice versa.

Since the demod test is not the functionality test, ideally the test under one mode is sufficient, and most likely UE would support IDLE/INACTIVE mode for better utilizing the WUS mechanism in terms of power saving. However, we are not so sure that RAN1 could provide confirmable feedback that all UE supporting LP-WUS should support the feature under IDLE/INACTIVE mode. Therefore, to cover the completed operation scenarios, test cases should be designed for both IDLE/INACTIVE mode and CONNECTED mode.

Observation 2: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LR, which mode is selected for test should be up to UE declaration.

For demod test, it’s proposed to follow the agreement in RF session that demod test mode design as well as the details of demod test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs. Demod experts could focus on discussion on simulation parameters that impacts on performance.

Proposal 2:  Same as RF test, demod test mode design as well as the details of demod test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

General test setup

Both LP-WUS and LP-SS are introduced, LP-SS has the same function as NR PSS/SS which is used for synchronization and measurement. We propose to only test LP-WUS since RAN4 has never introduced requirements for synchronous signal.

Proposal 3: RAN4 to only define requirements for LP-WUS, not introduce requirements for LP-SS.

Considering on the RAN1 design and traditional RAN4 test setup, following general test setup are proposed:

Frequency range: Prioritize FR1, FFS FR2

Duplex mode: FDD and TDD

SCS: 15kHz for FDD and 30kHz for TDD

Number of RB: 11

Antenna configuration: 1T1R

Channel model: TDLA30-10

Proposal 4: RAN4 to consider following general test setup as start point:

Frequency range: Prioritize FR1, FFS FR2

Duplex mode: FDD and TDD

SCS: 15kHz for FDD and 30kHz for TDD

Number of RB: 11

Antenna configuration: 1T1R

Channel model: TDLC300-100

Two LS-WUS receivers are defined: LP-WUS operation based on OFDM overlaid sequence and LP-WUS based on OOK. Noted that there is a separate capability introduced for the latter one. The former receiver requires UE only perform envelope detection to estimate each encoded bit and apply decoding, it has low power consumption but relatively poor performance. The latter one requires UE perform sequence correlation to estimate each encoded bit and apply decoding, it has better performance and anti-interference capability but relatively high power consumption. We see big difference between two receivers from signal detection algorithm perspective. Therefore, we propose to define requirements for both receivers to cover UE with different capabilities. To reduce the test burden, it’s proposed to define single test for both receivers.

Proposal 5: RAN4 to define demodulation requirements with single test for both receivers:

LP-WUS receiver based on envelop detection. (OOK based receiver)

LP-WUS receiver based on OFDM sequence correlation. (OFDM based receiver)

According to RAN1 design, up to 5 information bits is supported, which means two kind of coding schemes are supported: small block coding for 1 or 2 information bits and RM coding for 3 or more information bits. From our understanding, such two coding schemes have different decoding algorithm. Therefore, from baseband processing view, both decoding algorithms should be tested. Therefore, we propose the following:

Proposal 6: RAN4 to define requirements for LP-WUS with two different coding/decoding algorithms: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding.

Same as PUCCH and PRACH requirements, both MDR and FAR requirements should be defined. Similar as PRACH and PUCCH tests. FAR and MDR requirements could be tested by following approaches:

FAR requirements is tested by counting the probability that UE detects its own sub-group ID in the absent of BS transmissions, i,e, only white noise is transmission.

MDR requirements is tested by counting the probability that UE miss the detection on its own sub-group ID with the BS transmission of LP-WUS carrying its sub-group ID information.

We propose to define 0.1% as metric for FAR requirements and 1% as metric for MDR requirements as starting point.

Proposal 7:

FAR requirements is tested by counting the probability that UE detects its own sub-group ID in the absent of BS transmissions, i,e, only white noise is transmission.

MDR requirements is tested by counting the probability that UE miss the detection on its own sub-group ID with the BS transmission of LP-WUS carrying its sub-group ID information.

Define 0.1% as metric for FAR requirements and 1% as metric for MDR requirements as starting point

Others

Other parameters could be FFS such as M value, number of coded bits, frequency error and time error.

Proposal 8: RAN4 to further discuss parameters such as M value, number of coded bits, frequency error and time error.

- Conclusion

In this contribution we provides our overviews on R19 LP-WUS demodulation requirements. The proposals and observations are:

Observation 1:  Demodulation test framework is similar as RF performance test since the common test framework is MDR and FAR.

Observation 2: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LR, which mode is selected for test should be up to UE declaration.

Proposal 2:  Same as RF test, demod test mode design as well as the details of demod test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

Proposal 3: RAN4 to only define requirements for LP-WUS, not introduce requirements for LP-SS.

Proposal 4: RAN4 to consider following general test setup as start point:

Frequency range: Prioritize FR1, FFS FR2

Duplex mode: FDD and TDD

SCS: 15kHz for FDD and 30kHz for TDD

Number of RB: 11

Antenna configuration: 1T1R

Channel model: TDLC300-100

Proposal 5: RAN4 to define demodulation requirements with single test for both receivers:

LP-WUS receiver based on envelop detection. (OOK based receiver)

LP-WUS receiver based on OFDM sequence correlation. (OFDM based receiver)

Proposal 6: RAN4 to define requirements for LP-WUS with two different coding/decoding algorithms: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding.

Proposal 7:

FAR requirements is tested by counting the probability that UE detects its own sub-group ID in the absent of BS transmissions, i,e, only white noise is transmission.

MDR requirements is tested by counting the probability that UE miss the detection on its own sub-group ID with the BS transmission of LP-WUS carrying its sub-group ID information.

Define 0.1% as metric for FAR requirements and 1% as metric for MDR requirements as starting point

Proposal 8: RAN4 to further discuss parameters such as M value, number of coded bits, frequency error and time error.