3GPP TSG-RAN WG4 Meeting # 115						           R4-2507182

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.26.2

Source: 	OPPO

Title: 	On system parameter

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the system parameter is provided in this paper.

- Discussion

During the RAN4#114 meeting, the agreed WF has been captured as below:

Issue 2-1-1: R2D transmission bandwidth

Agreement:

- For RAN4 requirements, only define R2D transmission bandwidth configuration as 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS.

Issue 2-1-2: R2D channel bandwidth

Agreement:

- Define R2D channel bandwidth with assumption of 90% spectrum utilization.

Issue 2-2-1: D2R bandwidth

Agreement:

- Define D2R channel bandwidth for A-IoT device:

- Based on 2SB transmission,

- Consider SFO impact

- Consider small frequency shift defined in RAN1

- D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

Issue 2-3-1: R2D channel raster

Agreement:

- Consider following options for R2D channel raster

  - Option 1: Define 10KHz as R2D channel raster (i.e. same as enhanced channel raster)

  - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS

  - Option 3: Define 100KHz as R2D channel raster

  - Other options are not precluded.

2.1 R2D Channel bandwidth

Firstly, for the R2D channel bandwidth, it has been agreed that 1 to 4 PRB with 15kHz SCS to be defined as the R2D transmission bandwidth configuration. Furthermore, the 90% spectrum utilization has been also agreed. In such case, the channel bandwidth definition is quite stable. However, as mentioned in our paper in the last meeting, besides the channel bandwidth configuration as 1 to 4 PRBs, the exact number of SCSs to be used for R2D transmission is with 15kHz SCS granularity. The agreement in the RAN1#120 meeting has been captured as below.

Agreement

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

With the RAN1 agreement as captured above that: “Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.” This means for the exact transmission bandwidth, 15kHz SCS can be further applied to the transmission channel bandwidth, this can be similar method as NR that configured RBs are defined but the exact RBs used for transmission can be configured. And for A-IOT case, the configured RBs are defined but the exact RBs + the Sub-carriers used for R2D transmission can be further configured, in such case, the below channel bandwidth definition is proposed. Below figure 1 is the NB-IoT channel bandwidth definition and we believe similar definition can be reused for A-IoT R2D channel bandwidth.

Figure 1 NB-IoT channel bandwidth definition

Below figure 2 illustrates the 1PRB with 12 Sub-carriers configuration. The A-IOT R2D channel bandwidth is the transmission bandwidth configuration plus the guard band. The transmission bandwidth can be any number of sub-carriers, i.e. 10 sub-carriers as shown in figure 2.

Figure 2 A-IOT R2D channel bandwidth

As the SU has been agreed to be defined as 90% for A-IoT R2D channel bandwidth, hence the guard band should be 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration.

For the upper limit of the channel bandwidth, we see no limitation in current stage. From NR bands perspective, it is suggested that for current channel bandwidth for each NR band, the largest channel bandwidth for the exact band can be the upper limit of the R2D channel bandwidth.

Proposal 1: The guard band should be 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration.

Proposal 2: The largest channel bandwidth for the each NR band can be the upper limit of the R2D channel bandwidth.

Proposal 3: Define the A-IoT R2D channel bandwidth as figure below:

2.2 D2R Channel bandwidth

For the D2R channel bandwidth, in the last RAN1 meeting, 2SB has been agreed. For the small frequency shift, it has been discussed in RAN1 and the conclusion is captured as below:

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
|  | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Background

For D2R transmission, the following parameters and relationships are considered:

- Tb: Time duration corresponding to a bit, where the bit is that after FEC if applied.

- Tc: Chip duration, where Tb = Tc×2R.

- R: SFS factor, where the small frequency shift location is determined by ± (R/Tb = 1/2/Tc) Hz.

- Btx,D2R: D2R transmission bandwidth, where Btx,D2R = 4/Tb = 2/(R*Tc).

The background also captures the relationship between Tb, Tc, R and BTX,D2R. The corresponding numbers are also shown in above table. Also, from the background, the frequency shift can be calculated as frequency shift to CW=1/(2Tchip) and the Btx,D2R = 4/Tb. In such case, the FDM operation and corresponding frequency shift as well as the BTX,D2R can be illustrated below as figure 3.

We need to define two parameters, one is the frequency shift as Fs and the other one is the transmission bandwidth of the device as Btx,D2R. The three critical equation has been marked as yellow above and captured again below:

-Tb = Tc×2R

-Btx,D2R = 4/Tb

-frequency shift to CW Fs=1/(2Tchip)

Figure 3 D2R channel bandwidth

From our understanding, the transmission bandwidth is directly connected to the Tb as the bit duration. And with further repetition factor R, the frequency shift Fs can be defined and hence with the transmission bandwidth and frequency shift, the frequency domain FDM operation is determined. The exact frequency shift when considering the TChip will be below table 1

Table 1 frequency shift compared to Tchip

| Freq. shift to CW =1/(2Tchip) | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 45 kHz | 60 kHz | 90 kHz | 120 kHz | 180 kHz | 240 kHz | 360 kHz | 480 kHz | 720 kHz | 960 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tchip (μs) | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |

And the transmission bandwidth of D2R when compared to Tb can be calculated as below table 2.

Table 2 transmission bandwidth of D2R compared to Tb

| Btx,D2R | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 180 kHz | 240 kHz | 360 kHz | 480 kHz | 720 kHz | 960 kHz | 1440 kHz | 1920 kHz | 2880 kHz | 3840 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) | 266.67 | 133.33 | 66.67 | 33.33 | 22.22 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 |

Observation 1: With the frequency shift and the transmission bandwidth, the frequency domain FDM operation is determined.

Observation 2: the frequency shift and transmission bandwidth of D2R is related to Tb and Tchip and Tb=Tchip * 2R.

Once RAN1 has determined and down selected the bit duration as well as the repetition factor, the frequency shift and the transmission bandwidth of the device will be determined. From RAN4 perspective, we need to define the core requirement to guarantee such transmission bandwidth and the frequency shift because RAN1 has not considered the 10% SFO impact.

In such case, for one device, the two parameter as frequency shift and transmission bandwidth should be defined as below with the frequency shift Fs with 10% accuracy and the transmission bandwidth of D2R BWTX,D2R with 10% accuracy.

Proposal 4: Define the A-IoT D2R frequency shift Fs based on table 1 and with 10% accuracy as illustrated in figure 4.

Proposal 5: Define the A-IoT D2R transmission bandwidth based on table 2 and with 10% accuracy as illustrated in figure 4.

For the D2R channel bandwidth, with the above defined frequency shift and D2R transmission bandwidth, the channel bandwidth can be calculated as D2RCBW= 2*Fs+BWTX,D2R.

Proposal 6: The D2R channel bandwidth can be calculated as D2RCBW= 2*Fs+BWTX,D2R.

Figure 4 D2R frequency shift and transmission bandwidth requirement

2.3 Channel raster

For the channel raster of A-IoT, it has been proposed to use 10kHz with the NR enhanced channel raster to A-IoT system. The enhanced channel raster of TS 38.101-1 has been captured as below. Since currently only band n8 has been used for example band, we did not captured all the NR bands. From the table below we can see, the 10kHz channel raster are mandatory support in some of the NR bands while for the rest they are optional supported, especially for band n8, it is not mandatory support.

Observation 3: The 10kHz channel raster are optional support for band n8.

From our understanding, directly use 10kHz channel raster for A-IoT system regardless of the band will have problem. For stand-alone operation, there is no limitation of the A-IoT system channel raster point. However, if the band is with 15kHz/30kHz channel raster, then use 10kHz as channel raster for such band is for sure not appropriate. If we have to use 10kHz as channel raster, then it only apply to NR bands with 100kHz channel raster and in stand-alone operation.

Observation 4: If we have to use 10kHz as channel raster, then it only applies to NR bands with 100kHz channel raster and in stand-alone operation.

Furthermore, when considering the in-band operation, since the 10kHz is not mandatory supported for all NR bands. Take band n8 as example, if the NR BS use 100kHz channel raster on band n8 (since 10kHz channel raster is not mandatory) and the A-IoT system use 10kHz channel raster, then the orthogonality between the A-IoT carrier and NR carrier cannot be guaranteed. Even if both 10kHz channel raster, to guarantee the orthogonality, the offset of 7.5kHz is still needed for the odd number of RBs of NR channel bandwidth. This can be illustrated as below figure 5.

Figure 5 Channel raster point of in-band A-IoT with 10kHz channel raster

This will be similar when considering the 15kHz channel raster as figure 6:

Figure 6 Channel raster point of in-band A-IoT with 15kHz channel raster

From above figure 5 and 6, it can be observed that ±7.5kHz shift for channel raster is needed for both 10kHz and 15kHz channel raster granularity for odd number of NRB of a channel bandwidth for NR in-band opeartion.

Observation 5: ±7.5kHz shift for channel raster is needed for both 10kHz and 15kHz channel raster granularity for odd number of NRB of a channel bandwidth for NR in-band operation.

For even number of NRB of a NR channel bandwidth, the ±7.5kHz shift is not needed.

In such case, below channel raster for A-IoT is proposed:

Proposal 7：

For stand-alone mode,

-10kHz channel raster can be used for NR bands with 100kHz channel raster.

-15kHz channel raster can be used for NR bands with 15/30kHz channel raster.

For in-band operation,

-10kHz channel raster can be used for NR bands with 100kHz channel raster. For odd number of NRB, ±7.5kHz shift is needed.

-15kHz channel raster can be used for NR bands with 15/30kHz channel raster. For odd number of NRB, ±7.5kHz shift is needed.

3	Conclusions

In this paper, we give discussion on the system parameter, the observations and proposals are as below.

Proposal 1: The guard band should be 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration.

Proposal 2: The largest channel bandwidth for the each NR band can be the upper limit of the R2D channel bandwidth.

Proposal 3: Define the A-IoT R2D channel bandwidth as figure below:

Observation 1: With the frequency shift and the transmission bandwidth, the frequency domain FDM operation is determined.

Observation 2: the frequency shift and transmission bandwidth of D2R is related to Tb and Tchip and Tb=Tchip * 2R.

Proposal 4: Define the A-IoT D2R frequency shift Fs based on table 1 and with 10% accuracy as illustrated in figure 4.

Proposal 5: Define the A-IoT D2R transmission bandwidth based on table 2 and with 10% accuracy as illustrated in figure 4.

Figure 4 D2R frequency shift and transmission bandwidth requirement

Observation 3: The 10kHz channel raster are optional support for band n8.

Observation 4: If we have to use 10kHz as channel raster, then it only applies to NR bands with 100kHz channel raster and in stand-alone operation.

Observation 5: ±7.5kHz shift for channel raster is needed for both 10kHz and 15kHz channel raster granularity for odd number of NRB of a channel bandwidth for NR in-band operation.

Proposal 6: The D2R channel bandwidth can be calculated as D2RCBW= 2*Fs+BWTX,D2R.

Proposal 7：

For stand-alone mode,

-10kHz channel raster can be used for NR bands with 100kHz channel raster.

-15kHz channel raster can be used for NR bands with 15/30kHz channel raster.

For in-band operation,

-10kHz channel raster can be used for NR bands with 100kHz channel raster. For odd number of NRB, ±7.5kHz shift is needed.

-15kHz channel raster can be used for NR bands with 15/30kHz channel raster. For odd number of NRB, ±7.5kHz shift is needed.

4 References

[1] R4-2505230	WF on A-IoT device requirements, CMCC