# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507187

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for downlink coverage enhancement

Agenda item:	7.31.5.2

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved. RAN4 shall define the RRM requirements for downlink coverage enhancement based on the latest revised WID as below:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

Discussion

In past meetings, RAN4 has mainly discussed the RRM requirements on SSB periodicity larger than 20ms. There are still some issues to be discussed in this meeting.

Paging

In current spec, paging interruption has been defined, the interruption time shall not exceed TSI-NR+K*Ttarget_cell_SMTC_period ms. However, RAN4 is discussing whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB period is equal to 160ms. The following way forward could be seen:

Further discussion: - For the paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - No update is needed, and K is 5.

To our understanding, this issue shall consider a trade off between the minimum interruption and measurement accuracy. When the SSB period is extended to 160ms, that is, SMTC periodicity will be extended to 160ms or larger than 160ms. In current spec, K is 5 if target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO. If we use the legacy value as 5, the K*Ttarget_cell_SMTC_period ms will be 900ms, which is nine times that of SSB period=20ms. we deem that this interruption is too large and the key paging message may be missed. However, if we only reduce the K due to the large interruption, the measurement accuracy will also be impacted. Consequently, we think a trade off between interruption and measurement accuracy shall be considered.

Proposal 1: K value shall be reduced if target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO.

- Note: Trade off between interruption and measurement accuracy shall be considered.

SMTC Enhancements

Downlink coverage enhancement is a new feature for R19 UE, and RAN2 sent one LS to RAN4 on SMTC enhancements in the last meeting. The detailed contents could be seen as below:

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

For the first bullet, we think it is feasible for downlink coverage enhancement. In legacy, the scenario is inter-satellite with large time difference, the main task is to capture multiple SSBs within SMTC, where the default SSB periodicity is only 20ms. Consequently, only one SMTC periodicity and different offsets are considered. The case in LS is downlink coverage enhancement from the same satellite, and a new default SSB periodicity was introduced by RAN1 which is 160ms. To our understanding, it is not enough to capture all neighbouring cell SSBs if we still use one SMTC periodicity with different offsets. Thus, we deem that it is reasonable to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbour cell measurements. The SMTC periodicity is changed and there is RRM requirements impact.

Proposal 2: It is reasonable to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbour cell measurements. The RRM requirements may be impacted.

For the second bullet, the following figure could be seen:

- Figure 1 Active neighbouring cells are scattered

- The motivation I understand mainly takes into account that the number of beams that satellites can activate simultaneously is insufficient, and beam hopping is required, resulting in highly dispersed arrival times of neighbouring cell SSBs. Considering the worst scenario, six beams need to be configured. However, UEs still conduct measurement based on previous capabilities which are mandatorily support 2 parallel SMTC measurements and optionally 4 parallel SMTC measurements. NTN cells cover a relatively larger area, UE only need to measure the SMTCs of nearby cells within a certain period. For instance, if a UE is located at one corner of a cell, it may not need to measure the cell at the opposite corner. From our perspective, the UE capability is not changed and it is up to UE implementation to choose the SMTCs to be measured. Consequently, there is no RAN4 requirements impact.

- Proposal 3: To support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs is feasible. There is no RRM requirements impact.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN downlink coverage enhancement.

Proposal 1: K value shall be reduced if target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO.

- Note: Trade off between interruption and measurement accuracy shall be considered.

Proposal 2: It is reasonable to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbour cell measurements. The RRM requirements may be impacted.

- Proposal 3: To support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs is feasible. There is no RRM requirements impact.

- 4References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331