3GPP TSG-RAN WG4 Meeting #115                                        R4-2507252

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:              Discussion on RRM requirements for NES OD-SSB

Agenda item:			7.27.3.1

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some deep discussions on RRM requirements for NES OD-SSB. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update (SSB related)

RAN 1## 120b From Tuesday session Agreement For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T. - Option 3: The SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication. Agreement For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications. From Wednesday session Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}. - N2= [8]. - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE]. Agreement Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication From Thursday session Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions From Friday session Agreement For a cell supporting on-demand SSB SCell operation, - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2. - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB. - od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap. - od-ssb-PositionsInBurst can be absent for Case #2. - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon. Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated. Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met. - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst. - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies.

3 RAN2 agreements update

Agreements on OD-SSB - For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). - A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. - For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. - OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. - RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS. - Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

4 Discussion

4.1 OD-SSB based deactivated SCell measurement – Procedure, Case 1

4.1.1 The start point of T2

First, as we analyzed in our previous paper, the necessity of T2 depends on whether scenario combination case is considered. To be specific,

- CASE 1, scenario combination CASE: Scenario #2+Scenario #3B

A OD-SSB transmission (i.e., OD-SSB with dense periodicity) is initiated in scenario #2  the transmission parameter of the OD-SSB to be updated (e.g., the OD-SSB burst periodicity to be updated to a sparser one) in scenario #3B  the OD-SSB is deactivated in the next scenario #2 to achieve:

- Fast deactivated SCell measurement and fast SCell activation comprehensively for the bursty data traffic.

- Slow synchronization /AGC maintenance once SCell activation is completed for NW energy saving.

Fig. 1 Fast known NES SCell activation based on FMW-based fast deactivated SCell measure’ment (FR1 as example) under Scenario #2+Scenario #3B

In this CASE, T2 is not needed, and the involved procedures, e.g., fast deactivated SCell measurement, NES SCell activation, CSI measurement and reporting are associated with O  D-SSB, and fast NES SCell activation based on FMW-based fast deactivated SCell measurement can be carried out if known condition satisfies.

Observation 1: For scenario combination case, e.g., combination of scenario #2 and scenario #3B case, there is a possibility that T2 is not needed

- CASE2: Pure Scenario #2

Fig. 2 Fast deactivated SCell measurement based on FMW under scenario #2

In this CASE, the time interval can exist, and from our understanding, it is more reasonable to define the start point of T2 as the time point of OD-SSB deactivation. The reasons are: As per RAN1 agreement guidance, two OD-SSB transmission deactivation mechanisms (pre-configured deactivation indication N burst and explicit deactivation MAC-CE signalling) and a time instance B are defined. Furthermore:

- Some companies prefer that N burst duration-based deactivation is more applicable to L3 measuremenent in deactivated SCell

- Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B

Observation 2: The time instance for terminating the OD-SSB transmission is defined in RAN1, and upon the reception of OD-SSB deactivation indication, UE expects that on-demand SSB is NOT transmitted from such time instance

From this, the next story the NW/UE would like to know naturally is when the OD-SSB can be reactivated after it is deactivated in order to obtain the tradeoff between target NW energy saving and performance of OD-SSB based measurement/detection at UE, and this is the primary intention of T2 we think. Desirably, following the rules related to T2 (i.e., the length and the condition) aligned between NW and UE,

- During T2: The NW is not expected to re-activate another OD-SSB transmission, the UE is not expected to perform measurement/detection based on OD-SSB

- After T2: The UE is required to perform fast deactivated SCell measurement based on FMW when UE re-receive the OD-SSB activation command and the FMW is triggered again. The same requirements defined for the first FMW-based measurement can be reused.

Proposal 1: The start point of T2 is the time point of OD-SSB deactivation

- The pros can be to obtain the tradeoff between target NW energy saving and performance of OD-SSB based measurement/detection at UE

Besides, from our understanding, the length of the time interval can be limited as [X]s with the following general conditions during the time span be met:

- The cell remains detectable and measured, and in FR2, the impact by beamforming should be considered

- The available measurement results remain valid/the reported measurement results satisfy measurement accuracy (As per RAN4 agreement guidance: The legacy measurement accuracy is applied for OD-SSB based measurement)

In this sense, based on the inspiration from condition of known cell, suggest to consider the following values as baseline:

- FR2: [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4

- FR1: [max(5*TOD-SSB, 5*DRX cycles)] s

The reason why we consider TOD-SSB instead of measCycleSCell is that based on the agreements we achieved, below

- The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period

from our understanding, the OD-SSB/FMW-based RRM measurement on deactivated SCell is no longer performed every max(measCycleSCell, DRX cycle), so we have a little concern that if we to define the duration as 5*max (measCycleSCell, DRX cycles), 1) whether 5*1.28 s is too long to remain the radio conditions, and 2) the stored OD-SSB based L3 RSRP may be not reliable enough to be used for the subsequent SCell activation.

Proposal 2: For the minimum interval(T2) between multiple FMWs, the time period of known Scell condition but with OD-SSB periodicity for FR1 can be considered

- FR2: [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4

- FR1: [5*max(TOD-SSB, DRX cycles)] s

Speaking of the following option 2a that the start point of T2 is the time point of OD-SSB activation

Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation

we think such option is another understanding on T2. And the option is somewhat like to define a repetition periodicity of FMW, while the repetition periodicity is variable and depends on UE implementation including how to measure, and it is irrelavant to OD-SSB deactivation. In this sense, the intention of option 2a is not clear for us, and from our understanding, if to define the start point of T2 as time point of OD-SSB activation, at least the following issues should be clarified: 1) Whether the measurement window: FMW is periodic; 2) Why NW to re-trigger OD-SSB transmission if N burst timer is not expired or it did not deactivate the OD-SSB transmission; 3) If OD-SSB deactivation indication is involved in the T2, how to define the T2 length containing measurement duration within and after FMW, and the time span between OD-SSB deactivation and OD-SSB re-transmission

4.1.2 UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Based on discussions above, since NW is not expected to re-trigger another OD-SSB transmission within T2 if the T2-related rules satisfy, in theory, no requirement for UE within T2.

Proposal 3: UE’s behaviour when OD-SSB further activation/reconfiguration within T2: No requirement for UE within T2

- NW is not expected to re-trigger another OD-SSB transmission within T2

4.2 Requirement for RAN1 Alt Time-C2

As per RAN1 agreement guidance, for the case when the center frequency of AO-SSB and OD-SSB is the same, the relation in terms of time location between AO-SSB and OD-SSB can be categorized into

- Time-C1：a union of periodic time domain pattern

- Time-C2: the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern

with at least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB. While, whether the half frame index can be the same or not for AO-SSB and OD-SSB is still suspending.

From our understanding, although it is not clear what is the use case of AO-SSB and OD-SSB are distributed with non-equal intervals in the practical deployment, from UE side, even the two sets of SSBs are configured to transmit in the same time occasion, UE does not need to distinguish the SSBs since they are the same. In this sense, to avoid UE implementation complexity, if to define the requirement for Time-C2, it is suggested to define the requirement based on OD-SSB periodicity.

Proposal 4: For the case when the center frequency of AO-SSB and OD-SSB is the same, RAN4 to define the requirement for RAN1 Alt Time-C2 based on the OD-SSB periodicity

4.3 Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

4.3.1 Information obtained from RAN1 and RAN2

Fig. 3 RAN1 and RAN2 discussion on scenario 3

- Different from RedCap configuration, in which, the IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB.

- No consensus: It is up to gNB implementation whether PBCH payload for the same SSB index (other than SFN index, half frame index) is same or different for AO-SSB and OD-SSB

- Spatial domain relation: SS/PBCH blocks with the same SSB indexes for AO-SSB and OD-SSB are QCLed.

- For Case #2, the physical cell ID of OD-SSB is the same as that of AO-SSB. The sub-carrier spacing of on-demand SSB is the same as that of always-on SSB.

Agreement - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

- OD-SSB is just for RRM measurement and transparent to legacy UEs.

4.3.2 Potential issues

Intra-frequency and inter-frequency definition

38.300 Intra-frequency and inter-frequency definition Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. - SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different. NOTE 2: For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells. NOTE 2a: If a RedCap UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above.

Based on the NOTE 2 above,

Observation 3: The legacy UE assumption that one MO corresponds to one SSB and SSBs in different MOs are regarded as different cells does not hold for SSB based measurement in scenario 3 any more

Besides, based on the existing definition, whether the SSB based measurement is deemed as intra-frequency or inter-frequency measurement depends on whether the center frequency of the SSB of the neighbour cell is of the same or different as the one of the serving cell and whether the SCS of the two SSBs are the same or not. In this sense, if AO-SSB and OD-SSB with different frequencies are both configured for the serving SCell measurement:

Proposal 5: For SSB-based measurement in scenario 3, it is necessary to discuss the center frequency of which SSB (i.e., AO-SSB or OD-SSB) can be taken as the baseline to define intra-frequency/inter-frequency measurements

OD-SSB intra-frequency/inter-frequency measurement

Fig. 4 Illustration of inter-frequency OD-SSB measurement

As exemplified in Fig. 4, from our understanding, no matter how to define inter-frequency or intra-frequency for scenario 3, there is one SS block of neighbor cell will be not aligned with serving cell SSB block if two SSB blocks (AO-SSB and OD-SSB) are both within UE operating bandwidth. While, based on the current clarification of the relationship between inter-frequency and MG, for case in Fig.4, gaps are not needed. Meanwhile, for intra-frequency measurement, if seving cell and neighbor cell SS block (AO-SSB or OD-SSB) overlap and are within the UE operating bandwidth, no gap needed. Besides, since there is a note agreed in RAN1 that UE is not required to measure both AO-SSB and OD-SSB, we can have the following baseline assumption: UE can only perform measurements on one frequency location at a time.

Proposal 6: For SSB-based measurement in scenario 3, when the two SS blocks (AO-SSB and OD-SSB) are within the UE operating bandwidth, if to perform OD-SSB inter-frequency/intra-frequency measurement, no gaps needed.

Proposal 7: For SSB-based measurement in scenario 3, based on the agreed Note: UE is not required to measure both AO-SSB and OD-SSB, the baseline assumption of UE can only perform measurements on one frequency location at a time can be considered

First active DL BWP

Although the SCell is configured with AO-SSB and inter-frequency OD-SSB, the OD-SSB and AO-SSB are located in the same BWP, and the two SSBs needs to be placed in the first active DL BWP, which is used upon activation of an SCell.

38.133 8.3.2 The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell. 38.331 firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated or to be used for RLM, BFD and measurements if included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated, upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If the field is absent for the PSCell at SCG deactivation, the UE considers the previously activated DL BWP as the BWP to be used for RLM, BFD and measurements. If the field is absent for the PSCell at SCG activation, the DL BWP to be activated is the DL BWP previously to be used for RLM, BFD and measurements. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon reconfiguration with reconfigurationWithSync, the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value..

Proposal 8: For SSB-based measurement in scenario 3, the AO-SSB and OD-SSB within different frequencies need to be placed in the first active DL BWP

How UE to prioritize OD-SSB based SCell measurements

First, the current TS 38.331 says:

38.331 5.5.3.1 The UE shall: 1> whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows: 2> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO: 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb: 4> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 3> derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

in theory, the UE is mandated to perform serving cell measurement when servingCellMO is configured, however, in our discussions, it should be allowed UE not to perform measurement for the new servingCellMO when OD-SSB transmission is not activated.

Proposal 9: For SSB-based measurement in scenario 3, it is not expected that the UE is allowed to perform measurement for the new servingCellMO for OD-SSB in ServingCellConfig when OD-SSB transmission is not activated if multiple serving cell MOs are configured for a given SCell

Furthermore, based on previous discussions, OD-SSB based L3 measurement is expected to be prioritized over other legacy L3 measurements when OD-SSB transmission is triggered/activated. And to ensure the fast OD-SSB based deactivated SCell L3 measurement can be carried out (factor in CSSF =1 for OD-SSB layer), the SCell’s new servingCellMO for OD-SSB needs to be prioritized even if the UE is configured with multiple serving cell MOs, accordingly. However, except for in Rel-19 RRM enhancment ph5 WI, a new enhancement mechanism of measuring one serving CC per band was introduced, in other previous releases, it is up to UE implementation to determine which MO can to measure first. In this sense, to meet OD-SSB based measurement expectation, from our understanding, there are two solutions:

Proposal 10: To enable the UE to prioritize OD-SSB based SCell measurements (e.g., FMW-based fast deactivated SCell measurement) when OD-SSB transmission is activated.

- Alt.1 UE to measure the serving cell MO for OD-SSB when the UE-specific level OD-SSB signalling is indicated

- Alt.2 UE to measure the serving cell MO for OD-SSB when new network signalling of a specific OD-SSB SCC for measurement is indicated

CSI framework

In RAN1, for L1 measurement based on on-demand SSB, there is no concrete consensus on how CSI framework including CSI report configuration works in Case #2, especially for the following situation: There is already periodic AO-SSB transmission on the SCell, but OD-SSB with different frequency are activated to transmit on the same cell. And even a CSI report configuration can be associated with both of AO-SSB and OD-SSB, when the two kinds of SSB have the same center frequency and the same positions in the same SSB burst, UE behavior for SSBs measurement is not explicit as well. In this sense, suggest to

Proposal 11: To guarantee UE can perform OD-SSB based L1 measurement and reporting accurately when both AO-SSB and OD-SSB within different frequencies are configured in the same serving cell, suggest to wait for RAN1 conclusions on CSI report configuration for OD-SSB for Case #2

4.4 OD-SSB general issues

4.4.1 Additional Processing time for OD-SSB reception – value

In last meeting, the addition processing time was extensively discussed. From our understanding, the value should to cover all the possibilities, while it is also the time used by the NW to prepare for transmitting the OD-SSB and the UE to prepare for receiving the available OD-SSB. In order to avoid 1) the case that NW energy waste when transmitting OD-SSB before the UE can receive; 2) the case UE would not know when to start to receive the on-demand SSB then miss the OD-SSB transmission, we can to consider 3 ms and 5 ms for the additional processing time. Besides, based on our understanding, 2 ms maybe is not sufficient, e.g., 1 ms (searcher re-scheduling mentioned by companies) + 0.5/0.25 ms (∆Delay for FR1/FR2) + 1/0.5 ms (RF preparation especially for case 1 and case 2 when AO-SSB and OD-SSB within different frequencies)~ at least 2.5 ms.

Proposal 12:

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: 3 ms and 5 ms.

4.4.2 Interruption

Based on the current spec.

##### Interruptions during measurements on deactivated SCC Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. - If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1.

From our understanding, since FMW-based fast deactivated measurement is considered in scenario 2, in this sense if we would like to inherit the principle that “interruption is allowed before and after an SMTC due to deactivated SCell measurement but the amount of interruption is restricted with missing ACK/NACK rate”, the limitation in terms of configured measCycleSCell is 640 ms is not applicable any more, since FMW/OD-SSB occasion is irrelevant to the legacy consideration of long measCycleSCell>>SMTC

Proposal 13: If to define interruptions during measurements on deactivated SCC for OD-SSB scenario 2, the condition in terms of “the configured measCycleSCell is 640 ms or longer” should be updated

5 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for NES OD-SSB. The following observations and proposals are obtained:

Observation 1: For scenario combination case, e.g., combination of scenario #2 and scenario #3B case, there is a possibility that T2 is not needed

Observation 2: The time instance for terminating the OD-SSB transmission is defined in RAN1, and upon the reception of OD-SSB deactivation indication, UE expects that on-demand SSB is NOT transmitted from such time instance

Observation 3: The legacy UE assumption that one MO corresponds to one SSB and SSBs in different MOs are regarded as different cells does not hold for SSB based measurement in scenario 3 any more

Proposal 1: The start point of T2 is the time point of OD-SSB deactivation

- The pros can be to obtain the tradeoff between target NW energy saving and performance of OD-SSB based measurement/detection at UE

Proposal 2: For the minimum interval(T2) between multiple FMWs, the time period of known Scell condition but with OD-SSB periodicity for FR1 can be considered

- FR2: [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4

- FR1: [5*max(TOD-SSB, DRX cycles)] s

Proposal 3: UE’s behaviour when OD-SSB further activation/reconfiguration within T2: No requirement for UE within T2

- NW is not expected to re-trigger another OD-SSB transmission within T2

Proposal 4: For the case when the center frequency of AO-SSB and OD-SSB is the same, RAN4 to define the requirement for RAN1 Alt Time-C2 based on the OD-SSB periodicity

Proposal 5: For SSB-based measurement in scenario 3, it is necessary to discuss the center frequency of which SSB (i.e., AO-SSB or OD-SSB) can be taken as the baseline to define intra-frequency/inter-frequency measurements

Proposal 6: For SSB-based measurement in scenario 3, when the two SS blocks (AO-SSB and OD-SSB) are within the UE operating bandwidth, if to perform OD-SSB inter-frequency/intra-frequency measurement, no gaps needed.

Proposal 7: For SSB-based measurement in scenario 3, based on the agreed Note: UE is not required to measure both AO-SSB and OD-SSB, the baseline assumption of UE can only perform measurements on one frequency location at a time can be considered

Proposal 8: For SSB-based measurement in scenario 3, the AO-SSB and OD-SSB within different frequencies need to be placed in the first active DL BWP

Proposal 9: For SSB-based measurement in scenario 3, it is not expected that the UE is allowed to perform measurement for the new servingCellMO for OD-SSB in ServingCellConfig when OD-SSB transmission is not activated if multiple serving cell MOs are configured for a given SCell

Proposal 10: To enable the UE to prioritize OD-SSB based SCell measurements (e.g., FMW-based fast deactivated SCell measurement) when OD-SSB transmission is activated.

- Alt.1 UE to measure the serving cell MO for OD-SSB when the UE-specific level OD-SSB signalling is indicated

- Alt.2 UE to measure the serving cell MO for OD-SSB when new network signalling of a specific OD-SSB SCC for measurement is indicated

Proposal 11: To guarantee UE can perform OD-SSB based L1 measurement and reporting accurately when both AO-SSB and OD-SSB within different frequencies are configured in the same serving cell, suggest to wait for RAN1 conclusions on CSI report configuration for OD-SSB for Case #2

Proposal 12:

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: 3 ms and 5 ms.

Proposal 13: If to define interruptions during measurements on deactivated SCC for OD-SSB scenario 2, the condition in terms of “the configured measCycleSCell is 640 ms or longer” should be updated

6 References

[1] R4-2502598, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson