3GPP TSG-RAN WG4 Meeting #115                                        R4-2507255

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:              General discussion on AI mobility regarding testability and interoperability

Agenda item:			7.23.4

Document for:	Discussion

1 Introduction

Some AI mobility regarding testability and interoperability were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Ground Truth Definition for RSRP accuracy for FR1

How to define the ground truth is still suspending. Alt 1: Be based on the transmitted or reception power; Alt 2: Be based on the reported measurement value under certain conditions

As one of the imperative considerations, the following information should be known at the TE: Prediction related time including absolute and relative prediction time. Irrelevant to when UE to report the prediction result, the TE should know the prediction related time, the time can be the absolute time UE performs the prediction and the relative prediction time (prediction window), otherwise the TE does not know the accurate timestamp the maximum available Tx power to transmit, and consequently, the predicted RSRP accuracy requirements cannot be verified under fast fading channel accurately. And such prediction related timing needed to be aligned can be as an assistance information.

Fig. 1 Illustration of conducted test

- Alt. 1. If to comply with the legacy handling method, seems that Alt 1 is more suitable for FR1, and FR1 RRM testing is in conducted test environment and the input of useful signals to the UE baseband can be known. While, it should be acknowledged that situations are more complex in AI mobility, at the root of it, the fast fading channel condition consideration.

  - Unlike AWGN, the rapid variation in channel condition contains too much randomness. And the effects brought by various channel models are very different. If the multi-path faded DL channel model is considered, it is rather challenging to establish huge enough dataset to cover all possible states generated by the random variables/parameters. In this sense, worst case, the un-testability caused by different kinds of randomness and channel characteristics in various complex real-world environment will be occurred.

  - As above mentioned, the timing instance/timestamp between prediction and GT should be aligned, however, realistically, the alignment can not be guaranteed. For example, the following issue may exist

    - Time gap between sliding and non-sliding based predictions. Basic reason: SlidingThe granularity: every sampling period. It can predict L3-RSRPs on a small time scale and capture short term variation in channel conditions in the temporal domain; No-slidingThe granularity: the measurement period. The interval between two consecutive samples is larger, and greater channel variation/decreased channel correlation between the two samples is considered.

Observation 1: Realistically, the mis-alignment of timing instance/timestamp between predicted RSRP and ground truth RSRP cannot be ignored.

  - Besides, L1 filtering and L3 filtering are involved, different L1 filtering options/the value of L3 filtering parameters (i.e., FilterCoefficient) may influence the derivation of the ideal SS-RSRP conducted at TE.

However, from our understanding, the information regarding how UE to measure, including how many samples for L1 filtering to be used is not necessary to be known at TE, otherwise, “deterministic testing” will inevitably occur if too many parameter values become known at TE, the generalization capability of AI model may be poorly verified and the test becomes meaningless.

In this sense, if to consider Alt.1, impacts from all the uncertainties can not be mitigated, a test tolerance range needs to be defined especially when the level of mis-alignment is large. However, how to define the tolerance range should be carefully in case “too wide test tolerance range of accuracy difference, so easy for UE to pass the testing; Shrinked test tolerance range of accuracy difference, UE can not to pass the testing” happens.

Observation 2: For Alt.1, if to consider all the uncertainties including filtering, channel fading condition, measurement samples, etc., an unnecessarily wide predicted accuracy gain range shall be introduced, which may make it easier for the UE to pass RSRP prediction accuracy test.

Proposal 1: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

- A suitable test tolerance range of predicted accuracy gain should to be defined to consider the uncertainties, including e.g., filtering, channel fading condition, measurement samples, etc., as much as possible and to ensure UE will not pass the testing easily in parallel

  - FFS: How to define the test tolerance range

- Alt 2. Furthermore, from our understanding, from handling channel variation perspective, Alt 2 seems more preferable, but if to consider Alt 2, cheating issue should be also solved. The ideal SS-RSRP will be measured by UE and reported to TE, since there is no limitation on UE implementation, it can report a fake value, e.g., the best previous predicted value based on AI prediction or even non-AI approach, since the later can also achieve comparable performance under some scenarios/sub-use-cases. In this sense, it might be so easy for UE to pass the testing, but predicted RSRP accuracy requirement can not be verified. Besides, recalling that for measurement accuracy requirement, AWGN is typically specified as a side condition, and SNR side condition, number of samples, and measurement accuracy are linked together as a packet. If Ground truth for FR1 will be settled by measurement report, the SNR side condition under the fast fading channel environment should be defined.

Proposal 2: From handling channel variation perspective, Alt 2 seems more preferable, but if to define the ground truth by using the reported measurement value under certain conditions, at least the following issues should to be solved

- UE cheating issue

- The SNR side condition requirement under the fast fading channel environment

Other new testability aspects should to consider: Emulated SNR/Testable SNR range.

From our understanding, emulation of noise-free/very high SNR test conditions are supported. However, due to the fast fading channel condition random fluctuation, time instant A (e.g., the highest peak) and time instant B (the lowest peak) have different understandings on high-SNR. In our opinion, there are two solutions:

- The upper bound of signal power as the maximum available Tx power transmitted from TE, then control the level of artificial noise for target SNRs at different time instances.

- Control the signal and artificial noise dynamically with total power constraint for target SNRs at different time instances. However, in this case, the testable SNR range in the test system and the assessment of the range shall be discussed.

Proposal 3: Regarding the emulated SNR, to align the definition of high-SNR at each time instant under the fast fading channel condition, the following two solutions of controlling the noise/SNR can be done at TE

- The upper bound of signal power as the maximum available Tx power transmitted from TE, then control the level of artificial noise for target SNRs at different time instances

- Control the SNR and artificial noise dynamically with total power constraint for target SNRs at different time instances

  - The testable SNR range in the test system and the assessment of the range shall be discussed

2.2 Principle of FR2 testing in AI/ML mobility

Prior to the start of the FR2 RSRP accuracy prediction test, we’d better to study the methodology of FR2 testing in AI/ML mobility. From our understanding, at least the following aspects

- Emulation of the propagation conditions. In AI mobility L3 cell-level prediction, it is highly possible to emulate the multiple beam environment. In this sense, multi-path propagation conditions, i.e., the multi-path fading conditions between the gNB emulator and test chamber probe may need to be emulated to ensure the verification of MIMO algorithms, where TDL can be modelled. Besides, since RRM requirements are defined with AWGN as side condition, suitable AWGN implementation in an OTA environment is necessary.

- Emulation of channel models. The key parameters can contain: Maximum CBW in one carrier; Maximum CCs (if CA implementation is considered); Total number of faded paths; Number of taps for each faded path; Number of transmitters; Number of UE Rx receivers in different evaluation scenarios (temporal domain, frequency domain, cell-specific approach, cluster approach, intra-frequency cells, inter-frequency cells, different frequencies) for the proposed Sub-use-case (s).

- The number of emulated transmission reception points (TRxP). As a reference, NR cell corresponds to one or multiple TRxPs, which may be co-located or non-collocated.

- Emulation of noise and EPRE. How the emulated noise is generated/transmitted. And the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point.

- AoA setup. For OTA testing, one of the primarily part is AoA setup. The baseline RRM setup assumes that measurement equipment shall be able to emulate the signals from 2 AoAs. However, considering the aims of different tests, we have: verification of TX/RX beamforming is needed (2 AoAs/3 AoAs, 3AoAs is for multi-Rx simultaneous reception), verification of TX/RX beamforming is not needed (1AoA). While, if we focus on intermediate KPI, i.e., RSRP difference and/or measurement overhead reduction test, the assumptions of AoA setup should be further discussed.

- Rx beam direction assumption. RRM tests will consist of tests with signals transmitted from different directions that have to be identified and reported by UE. And Rx antenna gain is an important factor that to enable the search of signals from different directions by improving the link budget. Actually, the Rx antenna gain experienced by the UE will depend on the Rx beamforming codebook selected, and the exact orientation of the DUT is controlled by the TE turntable. That is to say, on the proposed AoA, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage. While, antenna gains corresponding to different directions are different, can be found in Annex B in TS 38.133. Besides, Noc level for signals coming from the two directions (beam peak and non-beam peak) are different also.

  - Considering the test uncertainties and complexities the non-beam peak direction may bring, we have concern that if our RRM tests at current stage are mainly to verify the RRM RSRP prediction performance without involving beamforming performance, whether test in non-beam peak direction is not necessary. And another reason is that since the cells to predict and train are co-located cells so far, UE has the peak beam direction for the predicted/inferred neighbor cell (s) can be testable

Proposal 4: For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

- Emulation of the propagation conditions

    - AWGN and TDL

- Emulation of channel models

- The number of emulated transmission reception points (TRxP)

- Emulation of noise and EPRE

- AoA setup

- Rx beam direction assumption

2.3 Testing of Generalization aspects

AI Model generalization is a very important and challenging issue in deep learning, which refers to the ability of the trained model to perform well on new inputs unseen during the training. From our understanding, compared to the PHY prediction, the generalization/adaptability of the AI models for mobility is more worth studying. The reasons are:

- For AI mobility, various deployment scenarios (e.g., UMa, UMi, etc.), channel environments, UE movement patterns and speeds, and frequencies exist. Meanwhile, different RRC configurations and conditions for measurement (events) need to be considered in practice. All of them will impact generalization performance of an AI/ML model.

- Multiple cells would be considered in L3 AI mobility WI. While, across different cells may capture variations in NW conditions and local radio conditions. In this sense, the risk of model overfitting to specific cell characteristics may arise, which may decrease the adaptability to different cells

- Since the prediction measurement results are L3 filtered. Different L3 filtering factors may incur different prediction accuracy. While, when training datasets collected with fixed L3 filtering factor, the corresponding AI models may have worse generalization capability in different cells, especially.

- The measurement event prediction may benefit of cell-, site- or area-specific models and the study on AI model generalization for measurement event prediction is needed

During the Rel-18 studies about AI/ML for PHY, the generalization evaluation performance of AI/ML models is verified by testing its performance with different training and test datasets (evaluation methodology in TR38.843 for generalization study), we can reuse such principle/methodologies.

Proposal 5: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Reuse the principle/methodologies/signallings on generalization achieved in AI/ML for NR air interface as much as possible

3 Conclusion

In this contribution, we provided our viewpoints on AI RRM requirements for testability and interoperability. The following observations and proposals are obtained:

Observation 1: Realistically, the mis-alignment of timing instance/timestamp between predicted RSRP and ground truth RSRP cannot be ignored.

Observation 2: For Alt.1, if to consider all the uncertainties including filtering, channel fading condition, measurement samples, etc., an unnecessarily wide predicted accuracy gain range shall be introduced, which may make it easier for the UE to pass RSRP prediction accuracy test.

Proposal 1: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

- A suitable test tolerance range of predicted accuracy gain should to be defined to consider the uncertainties, including e.g., filtering, channel fading condition, measurement samples, etc., as much as possible and to ensure UE will not pass the testing easily in parallel

  - FFS: How to define the test tolerance range

Proposal 2: From handling channel variation perspective, Alt 2 seems more preferable, but if to define the ground truth by using the reported measurement value under certain conditions, at least the following issues should to be solved

- UE cheating issue

- The SNR side condition requirement under the fast fading channel environment

Proposal 3: Regarding the emulated SNR, to align the definition of high-SNR at each time instant under the fast fading channel condition, the following two solutions of controlling the noise/SNR can be done at TE

- The upper bound of signal power as the maximum available Tx power transmitted from TE, then control the level of artificial noise for target SNRs at different time instances

- Control the SNR and artificial noise dynamically with total power constraint for target SNRs at different time instances

  - The testable SNR range in the test system and the assessment of the range shall be discussed

Proposal 4: For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

- Emulation of the propagation conditions

    - AWGN and TDL

- Emulation of channel models

- The number of emulated transmission reception points (TRxP)

- Emulation of noise and EPRE

- AoA setup

- Rx beam direction assumption

Proposal 5: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Reuse the principle/methodologies/signallings on generalization achieved in AI/ML for NR air interface as much as possible

4 References

[1] R4-2504901 “WF on RRM requirements for FS_NR_AIML_Mob”, Nokia