# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507495

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			TP to TS 38.914  Inband blocking OOB and Spurious

Agenda item:		7.26.3.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT BS RF requirements based on WF [1]. TP is proposed in section 5.

# Discussion

## Transmitter requirements

### Issue 1-3-1: Transmitter OFF power

- Agreement:

  - Transmitter OFF power spectral density shall be less than [-85] dBm/MHz per antenna connector.

For the transmitter off power, this value is taken from the TDD operation. This power level -85 dBm/MHz (or -95 dBm/100kHz) is in par with the protection of the own receiver for FDD band, the requirement is -91 dBm/100kHz. It can be relaxed to -91 dBm/100kHz to align with FDD operation.

The OFF power can be the same with protection of own receiver in FDD band operation.

### Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

    - Option 1: ACLR1 is 40dB and ACLR2 is 50dB for A-IOT BS 1PRB transmission BW with 100kHz offset

    - Option 2: ACLR 1 is 30dB and ACLR2 is 40dB for A-IoT BS 1PRB transmission BW with Foffset=100kHz.

  - FFS on whether frequency offset is needed or not for other PRB numbers

In 36.104, the Foffset is used in the NB-IoT standalone is specified with 200kHz and this also apply when NB-IoT carrier is placed at lowest or highest carrier, meaning there will be 100kHz guard band to the RF bandwidth edge. In this sense, when there is more RB configured for A-IoT, 100kHz offset to the RF bandwidth edge should be kept. If the same Foffset concept is used, the center distance between a A-IoT carrier to the center of the first adjacent carrier for ACLR requirement will be A-IoT channel bandwidth + Foffset.

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 5.6-3A.

Table 5.6-3A: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

Apply the same frequency offset of 100kHz between the edge channel .

Apply the same Foffset definition as NB-IoT standalone in TS 36.104.

### Issue 1-4-2: Modulation quality

- Agreement:

  - FFS whether to define requirements such as modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

The RF envelop rise/fall time is discussed in Tx transient and for modulation depth, Ripple defined in EPC specification. Figure 1 to Figure 2 that these are relating to the baseband synthesized signal. For example, the DFT approach is compared with ideal brick waveform with different subcarrier number and M segments in one OFDM symbol. It can be observed that the modulation depth is relating to the algorithm used to synthesize the OOK and also the ripple performance is relating to the configuration of the number of the subcarrier and M-Chip per OFDM symbol for baseband signal. For the ripple performance, the step response of channel filter can also introduce the ripple in time domain as illustrated in Figure 3. With a baseband signal input to the transmitter, the output will be convolution of both baseband signal and the impulse response of the transmitter’s transfer function. Channel filter is designed to attenuate the unwanted emission in frequency domain but not the ripple in time domain. It is illustrated in Figure 3 for the ripple performance of two LPF filter of different order filter and attenuation and can be observed that higher order filter with high attenuation may introduce higher ripple in step response compared to a lower order filter. From this analysis, we suggest not considering the ripple performance at A-IoT transmitter.

Modulation depth and ripple defined in EPC specification relating to the algorithm used to synthesize the OOK waveform.

Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

Not to define the modulation depth but depends on the RAN1 defined signal progressing for OOK signal generation.

Not to define the ripple performance.

Figure 1: Ripple and modulation depth with FFT size N=1024 and different size of K

Figure 2: modulation depth and ripple with FFT size N=1024 and K=24

Figure 3: Step response of the two LPF filter

As it is not relevant to define the time domain requirement as specified in EPC specification in [2]. We think there is a need to align the OOK signal quality requirement with LP-WUS. Some of the analysis is illustrated below for what has been done based on the LP-WUS signal design. As the NR signal quality requirement are tested in frequency domain referring to the reference point definition below, so the analysis also done in frequency domain to evaluate the signal quality distortion for OOK-M signal.

Figure B.1-1 [3]: Reference point for EVM measurement

Figure 4:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 5: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 6: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

- The tolerance of the distortion of the frequency domain data of OOK-4 is tested by remapping the frequency constellation of overlaid ZC sequence to one of existing modulation scheme.  Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered to reduce the impact on gNB. As one example, for OOK4 waveform, the higher modulation (e.g. 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 4) to 64QAM constellation points (right constellation in  Figure 4) for CP-OFDM symbols as illustrated in Figure 4.  Figure 5 and Figure 6 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for OOK signal detection at UE receiver due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

From the constellation re-mapping, it can be observed that the time domain signal quality of OOK-4 is quite robust and can be maintained without big degradation of SNR when the frequency domain OOK-4 data is distorted by the amount of EVM equivalent to the 64QAM. The frequency domain data of OOK-4 is degraded if it is distorted by the amount of the EVM equivalent to the 16QAM.  As the A-IoT BS may not transmit the NR signal, therefore, a 64QAM test vector is needed to test 64QAM EVM instead.

The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QAM before iFFT.

Specify the OOK-M signal quality requirement with equivalent EVM requirement better than 64QAM.

For Tx intermodulation requirement, it can be discussed whether the A-IoT BS can be co-located with another A-IoT BS transmitting in another sector. From the LLS simulation, it is observed that the R2D receiving will not be impacted when interfering signal is below -22 dB while the test of the Tx inter-modulation is -30 dB assuming the antenna isolation between antenna of wanted signal and antenna of the interfering signal as illustrated in Figure 7. So it is possible to deploy a NR BS with adjacent channel with lower output power. If the co-located NR BS only is deployed with another band than the A-IoT band, the need of such requirement may be further discussed according to the note in legacy requirement:

NOTE 1:	Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements.

Tx intermodulation should be defined if the NR BS is deployed within the same A-IoT band.

Figure 7: BLER performance with 49 RB NR signal

## Receiver requirement

For REFSENS requirement, the CW received power should be considered when testing the REFSENS. In Figure 8, the received CW power and D2R signal power is illustrated with 20 m ISD between CWT and BS. The CW received power is higher (e.g -15.5 dBm for LOS) with CW Tx of 33 dBm then the received CW power of -25.5 dBm with CW Tx power of 23 dBm for ISD of 20 meter.

To study the impact of CW signal on the REFSENS, the receiver IIP3 is firstly calculated using the receiver intermodulation requirements. REFSENS for 3MHz is – 103.6 dBm

Ptone= -47 dBm

IIP3= -47 + (-47 +103.6)/2=-18.7 dBm

Assuming the zero-IF architecture and the component lineup of duplexer + LNA+ Mixer, a rough cacluaiton of the total IIP3 can be done as below:

| Stage | Gain (dB) | IIP (dBm) |
| --- | --- | --- |
| Duplexer | G1=-1 | + 50 dBm |
| LNA | G2=20 | + 0 dBm |
| Mixer | G3=-6 | + 5dBm |

Using the formular of

1/IIP3_total= 1/IIP3_duplexer + G1/IIP_LNA + (G1*G2)/IIP_Mixer = 0.79/1+100*0.79/3.16=25

and IIP3_total = -14 dBm is calculated for the receiver linearity.

Therefore, the 0 dBm IIP3 LNA meet the min linearity requirement. In Figure 9, the receiver de-sensitization is illustrated with different LNA IIP3 the CW input power. It can be observed that for a 0 dBm IIP3 LNA, the needed de-sensitization could be 3 dB given enough margin for the approximation of the de-sensitization for -15.5 dBm CW power. If the CW power would be -25.5 dBm, 1 dB de-sensitization is good enough.

Secondly, except the IMD degradation, the filter leakage power could be another source of the noise. This relate the attenuation of the filter. Assume the noise figure of 5 dB, the noise floor for 15kHz filter bandwidth is -127 dBm ( -174 + 5+ 10log10(15000)). The attenuation from – 15.5 dBm to -127 dBm  is 111 dB.  As the frequency distance between wanted signal and CW is very narrow, the transit bandwidth is small. For example, for R= 2, Tc=66.67 us and Tb/2= 133, the frequency offset between DC to center of signal is 15 kHz – BW/2 = 7.5kHz.  If the sampling frequency is 3.84Msps, the transition band is 7.5kHz /1,92Msps=0.39% and according to rule of thumb equation, which is Δf = (A-8)/(2.25*N) where A=111 dB and Δf=0.39% , the filter order needed will be 11738.  This is to the de-sens to 3 dB. This is quite demanding on implementation and processing time.  If the receiver is tuned to the CW frequency, the CW signal can be removed by DC cancellation network (either analog or digital), this perhaps easier and gives about 40 dB cancellation. Drawback is that the receiver can handle one CW signal only. RAN4 can discuss if the receiver architecture should be aligned to further evaluate the CW signal impact.

Lastly, the phase noise from CW signal also raise the noise floor, this is illustrated in Figure 10. The noise power especially higher for small Tb (high data rates). For a 15kHz signal, the noise power is around -105 dBm (for CWT of 23 dBm). The de-sense will be 22 dB ( -105 – (-127)). The de-sense is worse for higher data rates.

From above analysis, the dominant de-sense factor is phase noise. Then potentially the leakage power from digital filter to remove CW signal. As the phase noise relates to the CW node RF discussion. The de-sensitization factor can be decided when RAN4 reach more concrete agreements on phase noise modelling and phase noise mask/profile.

Decide de-sensitization after phase noise impact conclusion in CW node.

Figure 8: BS received D2R power and CW power with different CW TX power

Figure 9: LNA IIP3 and desensitization by CW signal

Figure 10: noise power within ½ signal BW (1/Tb) with frequency shift = 10kHz and 100kHz

Based on the simulation assumption, BLER is illustrated inFigure 11. The residue SFO is used to D2R simulation and it can be observed that that the SNR performance depends on the residue SFO. Therefore, the SFO estimation and compensation should be needed and as the D2R amble is still under RAN1 design, RAN4 can wait RAN1 progress or make reasonable residue SFO.

SNR is 10.2 dB with SFO residue of 10^2 to 10^3 ppm.

RAN4 can wait RAN1 progress or make reasonable residue SFO.

Figure 11:BLER plot for coherent OOK signal detection with SFO=0 and message size = 20bits

Table 1 Required SNR to achieve a target BLER=0.1 (message size=20bits)

| Receiver BW | SFO=0 | SFO = 10^4-10^5 | SFO = 10^3-10^4 | SFO = 10^2-10^3 | SFO = 10^1-10^2 |
| --- | --- | --- | --- | --- | --- |
| Option 1: 15kHz | 9.6 dB | NA | NA | 10dB | 9.8dB |
| Option 2: 18kHz | 10.5 dB | NA | 23.5dB | 10.6dB | 10.5dB |
| Option 3: 30kHz | 13 dB | NA | 17.2 dB | 13dB | 13.2 dB |

Table 2 Required SNR to achieve a target BLER=0.1 (message size=96bits)

| Receiver BW | SFO=0 | SFO = 10^4-10^5 | SFO = 10^3-10^4 | SFO = 10^2-10^3 | SFO = 10^1-10^2 |
| --- | --- | --- | --- | --- | --- |
| Option 1: 15kHz | 10.3 dB | NA | NA | 10.4 dB | 10.4 dB |
| Option 2: 18kHz | 11.2 dB | NA | NA | 11.2 dB | 11.2 dB |
| Option 3: 30kHz | 13.4 dB | NA | NA | 13.4 dB | 13.4 dB |

##### 6.7.2.2.3 In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase.

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

Last meeting it is agreed only UE outdoor is considered. However, in our previous analysis on co-location requirement, a NR UE operating in adjacent channel can roam into the indoor A-IoT coverage before being hand overed to another frequency. Therefore, we think ACS and IBB legacy requirement should apply.

NR UE operating in adjacent channel can roam into the A-IoT coverage before handover occurs.

Reuse the ACS and IBB requirement in legacy NR BS.

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.  As discussed in ACS and IBB, a NR UE operating in adjacent channel can roam into the A-IoT coverage. Another case is multiple reader activation at the same time (e.g. 2 active readers is assumed in coexisting study). In such case, the BS may receive two different D2R signal in two channels at the same time, which is similar with the legacy NR BS. Therefore, the intermodulation requirement could be kept for receiver linearity requirement. Above all, there is no other requirement for receiver linearity but when receiver receiving a CW signal together with a weak backscattered signal, the linearity requirement will be needed anyway. From this perspective, it will be good to retain the same IM requirement.

Reuse the RX intermodulation requirement.

For ICS requirement, RAN1 has agreed FDMA for multiple devices, so different device can be multiplexed in the frequency domain at the same time. All the D2R signals from different devices are backscattered signals using the same CW signal but with different frequency offset. As there is no UL power control, the received power from different device may vary but still the highest received power should be CW signal. As the legacy ICS requirement allows the desensitization of the wanted signal in presence of the higher PSD in-band interfering signal, REFSENS tested with CW presence may be deemed as in-channel selectivity test.

In legacy ICS test requirement, there is only one channel filter applied in the configured channel bandwidth. So the ICS test the received power imbalance tolerance between different UE. In A-IoT BS receiver, the different filter should be applied to different UE as the backscatter signal is time domain signal (OOK). In this sense, receiver can tolerate much high power imbalance between different received UEs. As such, we think there is no necessary to specify the ICS requirement.

REFSENS tested with CW presence can be regarded as one ICS test case.

A-IoT reader Receiver can tolerate high power imbalance between different UE receiving.

No ICS requirement is needed for A-IoT BS.

TP proposal is in section 5.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 Modulation depth and ripple defined in EPC specification relating to the algorithm used to synthesize the OOK waveform.

Observation 2 Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

Proposal-1: The OFF power can be the same with protection of own receiver in FDD band operation.

Proposal-2: Apply the same frequency offset of 100kHz between the edge channel .

Proposal-3: Apply the same Foffset definition as NB-IoT standalone in TS 36.104.

Proposal-4:Not to define the modulation depth but depends on the RAN1 defined signal progressing for OOK signal generation.

Proposal-5: Not to define the ripple performance.

Observation 3 The performance loss for OOK signal detection at UE receiver due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 4 The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QAM before iFFT.

Proposal-6: Specify the OOK-M signal quality requirement with equivalent EVM requirement better than 64QAM.

Proposal-7: Tx intermodulation should be defined if the NR BS is deployed within the same A-IoT band.

Proposal-8:Decide de-sensitization after phase noise impact conclusion in CW node.

Observation 6 NR UE operating in adjacent channel can roam into the A-IoT coverage before handover occurs.

Observation 5 SNR is 10.2 dB with SFO residue of 10^2 to 10^3 ppm.

Proposal-9: RAN4 can wait RAN1 progress or make reasonable residue SFO.

Proposal-10: Reuse the ACS and IBB requirement in legacy NR BS.

Proposal-11: Reuse the RX intermodulation requirement.

Observation 7 REFSENS tested with CW presence can be regarded as one ICS test case.

Observation 8 A-IoT reader Receiver can tolerate high power imbalance between different UE receiving.

Proposal-12: No ICS requirement is needed for A-IoT BS.

# References

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID

# TP proposal

Editor note: The TP is modified based on 38.104 text and using the tacking change so the review is easier to understand the changes comparing to the legacy requirements.

- 7.4	In-band selectivity and blocking

- 7.4.1	Adjacent Channel Selectivity (ACS)

- 7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

- 7.4.1.2	Minimum requirement for BS type 1-C

The MDR performance shall be [1%] of the reference measurement channel.

For BS operating, the wanted and the interfering signal coupled to the BS type 1-C antenna connector or BS type 1-H TAB connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.4.1.2-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.4.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.4.1.2-1: Base station ACS requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| [D2R Channel BW] | PREFSENS + 8 dB | -47 |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables [7.2.2-1]. |  |  |

- Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| [D2R BW1] |  |  |
|  |  | 5 MHz DFT-s-OFDM NR signal |
|  |  | 15 kHz SCS, 25 RBs |

- 7.4.2	In-band blocking

- 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

- 7.4.2.2	Minimum requirement for BS type 1-C

The MDR performance shall be [1%] of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector -using the parameters in tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3 for general blocking and narrowband blocking requirements. The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

The in-band blocking requirement shall apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. The ΔfOOB for BS type 1-C is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.4.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low ≤ 200 MHz | 20 |
| BS type 1-C | 200 MHz < FUL,high – FUL,low ≤ 900 MHz | 60 |

For a BS operating in non-contiguous spectrum within any operating band, the in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the channel bandwidth of the NR interfering signal in Table 7.4.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the narrowband blocking requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in Table 7.4.2.2-3. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

- Table 7.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| [D2R BW] | PREFSENS + x dB | Medium Range BS: -38 |  | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| NOTE 1: , PREFSENS depends also on the BS channel bandwidth as specified in tables [7.2.2-1] NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

- Table 7.4.2.2-2: Base Station narrowband blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| [D2R BW] | PREFSENS + 6 dB | Medium Range BS: -44 |
| NOTE 2: PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |

- Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| [D2R BW] | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
|  | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
|  | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
|  | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| NOTE 1: [Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

- 7.4.2.3	Void

- 7.4.2.4	Void

- 7.5	Out-of-band blocking

- 7.5.1	General

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

- 7.5.2	Minimum requirement for BS type 1-C

the MDR performance shall be [1%] of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in table 7.5.2-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

For a multi-band connector, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to clause 7.4.2.2 shall be excluded from the out-of-band blocking requirement.

- Table 7.5.2-1: Out-of-band blocking performance requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -15 | CW carrier |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in Table 7.2.2-1. |  |  |

- 7.5.3	Co-location minimum requirements for BS type 1-C

This additional blocking requirement may be applied for the protection of NR BS receivers when GSM, CDMA, UTRA, E-UTRA or NR BS operating in a different frequency band are co-located with a NR BS. The requirement is applicable to all BS channel bandwidths supported by the NR BS.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and NR BS receiver and are based on co-location with base stations of the same class.

MDR performance shall be [1%] of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector input using the parameters in table 7.5.3-1. The reference measurement channel for the wanted signal is identified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS channel bandwidth and further specified in annex A.1.

The blocking requirement for co-location with BS in other bands is applied for all operating bands for which co-location protection is provided.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.5.3-1: Blocking performance requirement for NR BS when co-located with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the BS channel bandwidth as specified in Table 7.2.2-1, 7.2.2-2, and 7.2.2-3. NOTE 2: x = -7 dBm for NR BS co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR BS co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR BS co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). |  |  |  |  |  |

- 7.6	Receiver spurious emissions

- 7.6.1	General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the antenna connector (for BS type 1-C). The requirements apply to all BS with separate RX and TX antenna connectors.

NOTE:	In this case for FDD operation the test is performed when both TX and RX are ON, with the TX antenna connectors terminated.

For antenna connectors supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period. For antenna connectors supporting both RX and TX in FDD, the RX spurious emissions requirements are superseded by the TX spurious emissions requirements, as specified in clause 6.6.5.

For RX-only multi-band connectors, the spurious emissions requirements are subject to exclusion zones in each supported operating band. For multi-band connectors that both transmit and receive in operating band supporting TDD, RX spurious emissions requirements are applicable during the TX OFF period, and are subject to exclusion zones in each supported operating band.

-

- 7.6.2	Basic limits

The receiver spurious emissions basic limits are provided in table 7.6.2-1.

- Table 7.6.2-1: General BS receiver spurious emissions limits

| Spurious frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the UL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. For multi-band connectors, the exclusion applies for all supported operating bands. NOTE 5: Applies for Band for which the upper frequency edge of the UL operating band is greater than 5.2 GHz. |  |  |  |

- 7.6.3	Minimum requirement for BS type 1-C

The RX spurious emissions requirements for BS type 1-C are that for each antenna connector, the power of emissions shall not exceed basic limits specified in table 7.6.2-1.

For Band n41 and n90 operation in Japan, the sum of RX spurious emissions requirements over all antenna connectors for BS type 1-C shall not exceed basic limits specified in table 7.6.2-1.

The Rx spurious emissions requirements shall apply to BS that support NR or NR with NB-IoT operation in NR in-band.