3GPP TSG-RAN WG4 Meeting #115	R4-2507641

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.25.3.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#114bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. The requirement with the biggest impact is the RX in-band blocking. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation and emphasizes the importance of setting correctly the RX in-band blocking requirement to enable functionality in real-life deployment.

# Discussion

## In-band blocking

### FR1 WA BS in-band blocking requirement

The following WF was agreed during RAN4#114-bis regarding the in-band blocking requirement for FR1 WA BS:

Issue 2-2: In-band blocking for FR1 WA BS FFS on this issue Option 1: Define the in-band blocking requirement with the same grid-shift [TBD] for both SBFD-TDD adjacent operator scenario and SBFD-SBFD adjacent operator scenario. Additional emission requirement is included in this release Option 2: Define the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19. If a decision is made to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario

Regarding the 2 options above, we agree with the principle proposed by Option 2 since we do not support the introduction of additional emission requirements in this release as proposed by Option 1. Additionally, we don’t see the SBFD-SBFD adjacent operator scenario in the scope Rel-19. Although our preference is Option 2, we still think that the grid-shift assumptions defined by Option 2 might be too optimistic. In our view, the in-band blocking requirements should assume the worst-case scenario. This will avoid potential issues of in SBFD-TDD deployments with small inter-side distance. Note that site-sharing is commonly used when deploying base stations from different operators, and 10% grid shift is what closest resembles the site-sharing scenario. Moreover, a SBFD BS that fulfils the in-band blocking requirement for 10% grid-shift would be automatically compliant for any of the other considered grid-shift values such as 20%, 50% or 100%. However, if the in-band blocking requirement is derived from, e.g., a grid shift of 50%, the SBFD operation for 10% grid-shift would be compromised since the interfering signal power would be higher than the used to determine the in-band blocking requirement. On the other hand, since this discussion has been on-going for several meetings without any progress, we accept 20% grid shift as a compromise.

Support the definition of the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19.

Assume 20% grid shift for the in-band blocking derivation of FR1 WA BS.

### FR1 MR BS in-band blocking requirement

The following WF was agreed during RAN4#114-bis regarding the in-band blocking requirement for FR1 MR BS:

Issue 2-3: In-band blocking for FR1 MR BS FFS on this issue Option 1: The IBB requirements of FR1 micro scenario can reuse existing requirements Option 2: Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm Option 3: propose to define the in-band blocking requirement as -38dBm under the assumption of without any analog filtering design after the LNA of receiver chain which is close to 100% grid shift in the practical deployment Option 4: Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs

Current list of options covers a wide range of in-band blocking requirements ranging from -38 dBm to -1.7 dBm. In our view, one of the reasons for such wide range is the grid-shift assumptions. Note that Option 1 assumes 100% grid shift, while Option 3 and Option 4 use 10% and 5% grid shift, respectively. Therefore, we think that RAN4 should first agree on a fixed grid-shift before making any decision among the listed options. Using the same arguments as for FR1 WA BS, we prefer to focus on the worst-case scenario and therefore 10% grid shift is our preference. Similarly, we are open to compromise to 20% grid shift to unlock the on-going discussion.

RAN4 to agree first on a grid-shift assumption before making any decision on the FR1 MR BS in-band blocking requirements.

Down-select a grid-shift value for FR1 MR BS in-band blocking requirements between 10% and 20%.

### FR2-1 WA BS in-band blocking requirement

The following WF was agreed during RAN4#114-bis regarding the in-band blocking requirement for FR2-1 WA BS:

Issue 2-4: In-band blocking for FR2-1 WA BS FFS on whether the current FR2-1 WA BS in-band blocking requirements can be re-used

Regarding the FFS above, we agree that the current FR2-1 WA BS in-band blocking requirements can be re-used, as supported by the results shown below in Table 3. We rely on the ratio of in-band blocking power versus wanted signal to reach such conclusion. From Table 10.5.2.3-1 in TS 38.104, we note that wanted and interfering signal mean powers are defined relative to the EIS_REFSENS and EIS_REFSENS_50M. Given the relation between EIS_REFSENS and EIS_REFSENS_50M, we can define the in-band blocking requirements as the ratio between both signal components. For instance, the current FR2-1 in-band blocking requirement assumes that the OTA interfering signal power is 27 dB higher than the OTA wanted signal power.

[TS 38.104, Section 10.5.2.3]

Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

#### Analysis of the in-band blocking requirements

Table 3 includes absolute in-band blocking power and wanted signal power as well as the ratio of in-band blocking power versus wanted signal power at 99th percentile for the FR2-1 WA SBFD BS at 10% grid shift.

Table 3. Ratio of in-band blocking power and wanted signal [dBm] for FR2-1 scenarios

|  | Interfering signal power at 99th percentile | Wanted signal power at 99th percentile | Ratio of interfering signal power versus wanted signal at 99th percentile | Current ratio of interfering signal power versus wanted signal in TS 38.104 |
| --- | --- | --- | --- | --- |
| Scenario 6 (Urban macro FR2) | -37.4 dBm | -66.1 dBm | 28.7 dB | FR2-1 WA BS: 27 dB |

Based on the results above the following is observed:

For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

Therefore, there is no need to define new in-band blocking requirements for FR2-1 WA SBFD BS given that the obtained ratio of in-band blocking power versus wanted signal is within the current FR2-1 WA BS requirements.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

### Clarification on the in-band blocking requirement derivation

In the previous RAN4#112 meeting, companies agreed to not account for the SBFD specific co-channel interferences to derive the in-band blocking requirement. In other words, BS self-interference and inter-sector interferences are not considered. However, these interferences also impact the receiver performance since they are also received by the BS LNA. This is an important assumption that should be clearly stated as part of the in-band blocking requirement definition for SBFD-capable BS in order to avoid possible misunderstandings.

RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

### ACLR and ACS effects

One important aspect for the in-band blocking power derivation is whether effects such as ACLR and ACS effects should be included. As described in TR 38.858, Clause E.2.3.1 the DL adjacent channel interference for the in-band blocking requirement should be measured following the definition of IACI, and it is explicitly mentioned that it does not include the “frequency isolation” aspects. i.e., the ACLR and ACS effects.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

## ACS Requirement

The following was agreed in RAN4#114bis:

- FFS on this issue - Option 1: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD. - Option 2: Prioritize discussing the scenarios under non-colocation case first. - The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked. - Option 3: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level. - Option 4: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD. - Derive ACS requirement level based on the simulation results using 5% GS in SI. - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Adjacent channel selectivity is an important measure of the receiver's ability to receive a wanted signal with interfering signal in the adjacent channel. In our view, the definition of ACS can be reused for SBFD. The general in-band blocking interference level defined for SBFD with interference level of 9dB offset should be used to define the interfering signal power level for the ACS requirement.

RAN4 to agree that definition of ACS is reused for SBFD and the interfering signal power level keeps 9dB offset to the in-band blocking interference level defined for SBFD.

## RX intermodulation

### Necessity of New RX intermodulation requirement with 1 interfering signal

The following was agreed in RAN4#114bis:

- FFS on this issue - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement - Option 2: RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal, and to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies

The proposed new RX intermodulation scenario with 1 interfering signal might be a possible scenario, and it should be discussed further. Caution needs to be applied when defining RF requirements and especially reusing existing ones. The differences with legacy TDD and SBFD systems are imminent and might end up causing disastrous effects in the field in the worst case.

Careful consideration is needed when defining requirements for SBFD.

As the power of the own TX sub-band signal (self-interference) could be significantly higher than what is currently assumed in the general RX IMD requirement (-52dBm for WA BS), this could lead to situation where the current RX IMD requirement is inadequate and doesn’t guarantee the linearity of RX is enough. If one would take the values companies calculated in the study item phase for the self-interference signal in gNB TX subband, measured at the input of LNA, the range varied from -23dBm to -56dBm. Additional robustness to RX would be highly welcomed to ensure functionality in this situation. The SBFD RX blocking requirement might not capture this implicitly, unless the own TX is on during the test. Therefore, we think additional RX IMD requirement with self-interference (TX on) and one interfering signal might be needed. The frequencies of the interfering signals need to be adjusted to cause IMD in the UL subband. If self-interference can be cancelled to reasonable level, this should not be an issue for SBFD-capable BS.

The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal.

It also must be noted that in the current FR1 WA BS RX IMD requirement, the mean power of interfering signals is defined as the RX in-band blocking requirement (-43dBm) -9dB offset = -52dBm. If, for SBFD, the RX in-band blocking interfering signal mean power is agreed and more stringent than the expected self-interference, there might not be need for the new RX IMD requirement with self-interference and 1 interfering signal. But in this case, there is then a need to update the SBFD RX IMD requirement to reflect RX in-band blocking interfering signal mean power with the 9dB offset. Frequencies of the interfering signals to cause IMD to UL subband need to be adjusted if needed.

RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

### Impact on existing RX intermodulation requirement with 2 interfering signals

The following was agreed in RAN4#114bis:

- FFS on this issue - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed - Option 3: RAN4 to consider new RX intermodulation requirement with 2 interfering signals. RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB

The new RX IMD requirement with 2 interfering signals could also be a possible scenario and should be considered further. The mean power of the interfering signals should reflect the RX in-band blocking with the 9dB offset.

RAN4 to consider new RX intermodulation requirement with 2 interfering signals.

After RAN4 has considered new possible RX IMD requirements with 1 or 2 interfering signals and the adjustment of current RX IMD requirement, the most challenging one should be chosen to cover the others and save effort on the conformance testing.

RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

# Conclusion

In this contribution, our views on the existing BS RF RX requirements for SBFD operation are presented. There were observations and proposals as listed below.

- For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

- Careful consideration is needed when defining requirements for SBFD.

- The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

- Support the definition of the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19.

- Assume 20% grid shift for the in-band blocking derivation of FR1 WA BS.

- RAN4 to agree first on a grid-shift assumption before making any decision on the FR1 MR BS in-band blocking requirements.

- Down-select a grid-shift value for FR1 MR BS in-band blocking requirements between 10% and 20%.

Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

RAN4 to agree that definition of ACS is reused for SBFD and the interfering signal power level keeps 9dB offset to the in-band blocking interference level defined for SBFD.

RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal.

RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

RAN4 to consider new RX intermodulation requirement with 2 interfering signals.

RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

# References

- R4-2504752 “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025

- R4-2418021 “SBFD RX in-band blocking simulation results”, Nokia, RAN4#113