3GPP TSG-RAN WG4 Meeting #115	R4-2507705

Malta, MT, May 19 – 23, 2025

# Title:	DC_28_n71 MSD

# Agenda Item:	6.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution proposes a cross-band isolation MSD test point for DC_28_n71 for the agreed-upon configuration of way forward (WF) [1].

- Discussion

At RAN4 meeting #114-bis, it was agreed in [1] to evaluate the Band 28 (B28) MSD requirements for the test point captured in Table 1. As a reminder, this band combination is restricted to “single-switched uplink” operation only.

Table 1: DC_28_n71 MSD Test Point from WF [1].

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | FFS | >ACLR2 |

# We note, however, that RAN4 has already agreed to a band n105 MSD 5 MHz CBW requirement for DC_28_n105. Considering that the Band n71 5 MHz CBW REFSENS requirement is identical to that of Band n105 for the upper channels, we propose to reuse the same MSD requirements for Band n71.

# Observation: The Band n71 5 MHz MSD due to cross-band isolation for DC_28_n71 may reuse the requirements from DC_28_n105 as shown below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| 28 | n71 | 713 | 20 | 15 | 25 (RBstart=0) | 649.5 | 5 | 5.7 | >ACLR2 |

  - Band 28 MSD Evaluation

For Power Amplifier (PA) transmitter (TX) noise emission measurements in the receiver (RX) band, we assume:

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, and 100 RB at lower channel edge with 1 dB maximum power reduction (MPR)

- Power Class 3 (PC3) operation and 4 dB post-PA loss

- Local Oscillator (LO) leakage: –28 dBc; IQ image rejection: –28 dB

- C-IM3: –60 dBc, C-IM5: –70 dBc.

For REFSENS/MSD estimations, we assume based on dual triplexer architecture:

- Transmitter noise floor PSD in RX band: –125 dBm/Hz (PA output referred).

- 38 dB TX to RX and 34 dB TX to antenna transmitter noise rejection in RX band [2]

- 44 dB/36 dB TX carrier rejection for main/diversity receivers and 54 dBm receiver IIP2

- 10 dB primary antenna to diversity antenna isolation

- 4 dB post-PA loss and 4 dB antenna-to-LNA front-end losses

- 3 dB antenna diversity gain and –1 dB SNR.

Figure 1 shows the Band n71 PA output spectrum using an ideal signal source (i.e., with an input noise PSD close to thermal noise).

Figure 1: Measured PC3 Band n71 35MHz CBW 20RB(RBstart=0) PA Spectrum Emissions Affecting DL Band B28.

In Table 2, the B28 5 MHz MSD is evaluated at 8.3 dB based on the following interference levels measured at B28 
5 MHz CC at Fc = 760.5 MHz:

- PA noise contribution: –52 dBm/4.5 MHz

- PA + TX composite noise contribution: –51.1 dBm/4.5 MHz.

Table 2: B28 MSD Evaluation Based on Measurement Results.

Proposal: Consider adopting the following PC3 cross-band isolation MSD requirements for DC_28_n71.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | 8.3 | >ACLR2 |
| 28 | n71 | 713 | 20 | 15 | 25 (RBstart=0) | 649.5 | 5 | 5.7 | >ACLR2 |

- Conclusion

This paper presents PA measurements to evaluate the DC_28_n71 MSD requirements.

Proposal: Consider adopting the following PC3 cross-band isolation MSD requirements for DC_28_n71.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | 8.3 | >ACLR2 |
| 28 | n71 | 713 | 20 | 15 | 25 (RBstart=0) | 649.5 | 5 | 5.7 | >ACLR2 |

# References

- R4-2505216 WF for UL CA_n28-n71, 3GPP TSG-RAN WG4 Meeting #114-bis, Wuhan, China, Huawei, HiSilicon.

- R4-2311748 CA_n28-n105 cross band MSD based on measurements, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.