3GPP TSG-RAN WG4 Meeting #115									                        R4-2507784

St Julian’s, Malta, May 19 – 23, 2025

Agenda item:			7.10.3.2.1

Source:	Ericsson

Title:	On the UE RF Tx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issue on the ACLR requirement for PC1 for NR-NTN HPUE from the approved WF in RAN4#114-bis [2].

# Discussion

One of the open issues from the approved WF is on the ACLR requirement for PC1:

Issue 1-7-2: PC1 ACLR

Tentative agreement:

- PC1 ACLR is [34] dBc

  - Companies to further evaluate this, and to be confirmed in next meeting.

For TN, the NR ACLR requirements have been specified as follows:

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

These requirements have been derived having in mind the co-existence between the different UEs, ensuring that not too much power is leaked to adjacent channels, with the goal to primarily protect the UL reception of the gNB at the adjacent frequencies. No isolation distance has been assumed between the UEs when these requirements have been derived.

If these requirements are not met, the minimum performance in the network cannot be guaranteed.

In this WI, it was agreed to specify requirements assuming 0 km isolation distance as well. Since the definitions of power classes are the same between NTN and TN, the same ACLR requirements should be specified for NTN and TN, which was also confirmed by our co-existence simulation results [3]. Relaxing ACLR requirements should not be used as a “tool” to potentially decrease the MPR requirements, as their purpose is to ensure the co-existence both between NTN UEs and between NTN and TN UEs. The consequences of relaxing the ACLR requirements would be detrimental for the TN performance (and unpredictable).

However, in the last two meetings some companies have expressed their concerns on the implementation challenges for PC1 (from the complexity and cost perspectives) if the ACLR is specified as 37 dBc for NR-NTN. To accelerate the roll-out of PC1 UEs for NR-NTN, they have proposed to use the same ACLR as for PC2 and PC1.5: 31 dBc. They also see it as a tool to tighten the MPR requirements and thus to improve the UL performance, which we do not agree on for the reason stated above.

In the end, [34] dBc was proposed as a compromise and it was left to companies to further evaluate the consequences of it.

In a separate contribution for this meeting [4], we have presented our conclusions of the co-existence system-level simulations on the throughput degradation for different ACLR requirement values. We have concluded that the 5%-ile throughput degradation is slightly higher than 5% (~7-8%) towards the TN network with 0 km isolation distance assumption i.e. when the NTN UEs are at the edge of the TN clusters.

Having in mind the UE implementation concerns for PC1, we are ready to compromise for 34 dBc ACLR requirement for PC1 under the condition that all our findings are correctly captured in TR38.863 (see Proposal 2 in [4]).

Proposal: For PC1, specify the ACLR requirement as 34 dBc as a compromise, where the conclusions of the co-existence system-level simulations results should be captured in TR38.863.

# Conclusion

Based on the discussion in the previous sections we have made the following proposal:

Proposal: For PC1, specify the ACLR requirement as 34 dBc as a compromise, where the conclusions of the co-existence system-level simulations results should be captured in TR38.863.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2505113, “WF on HPUE RF requirements for NR-NTN”, Samsung, RAN4#114-bis.

- R4-2501401, “On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE”, Ericsson, RAN4#114.

- R4-2507671, “On NR NTN and IoT NTN HPUE co-existence”, Ericsson, RAN4#115.