3GPP TSG-RAN WG4 Meeting #115	R4-2507979

St Julian, Malta, 19 – 23 May 2025

Source:	Ericsson, Verizon

Title:	TP for TR 38.719-03-01 to add UL CA_n5A-n77C and CA_n5A-n48B for CA_n5-n48-n77

Agenda item:	6.3.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.719-03-01 to add UL CA_n5A-n77C and CA_n5A-n48B for CA_n5B-n48A-n77C, CA_n5B-n48B-n77A, CA_n5A-n48B-n77C, CA_n5A-n48B-n77A and CA_n5B-n48B-n77C.

# 2. Text Proposal

# ---Start of changes---

## 5.73	CA_n5-n48-n77

### 5.73.1	Common for 1 band UL and 2 bands UL CA

#### 5.73.1.1	Operating bands for CA

Table 5.73.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

#### 5.73.1.2	Channel bandwidths per operating band for CA

Table 5.73.1.2-1: Supported bandwidths per CA band combination of band n5+n48+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n5A-n48A-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5A-n48B-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48B_BCS2 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n48B |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n48B-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | CA_n77C BCS1 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | CA_n77C BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | CA_n77C BCS1 |  |
|  | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n5A-n77C CA_n48B CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5A-n48(2A)-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n48(2A)-n77C | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n48A-n77C | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n48B-n77A | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n5B CA_n48B |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5B-n48B-n77C | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n48B CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

### 5.73.2	Specific for 2 bands UL CA

#### 5.73.2.1	UE co-existence studies

##### 5.73.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

##### 5.73.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.73.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n5A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.73.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis.

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n5, n77) | 824 – 849 |  | 3300 – 4200 |  |
| fDL (n48) | 3550 – 3700 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 624 – 824 |  | 849 – 1049 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.73.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n5A-n48B with n48B transmitting with a 40 MHz maximum instantaneous bandwidth.

Table 5.73.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis.

| Band / CA1 | Single Carrier |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n5, n48) | 824 – 849 |  | 3550 – 3700 |  |
| fDL (n77) | 3300 – 4200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 150 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 784 – 824 |  | 849 – 889 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### 5.73.2.2	REFSENS requirements

Based on the triple beat analysis of the added UL CA_n5A-n77C and CA_n5A-n48B, there are no triple beat issues and no need to define new MSD values.

---End of changes---

# Reference

[1] 		RP-250214, Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells; Ericsson, ZTE, Huawei