3GPP TSG-RAN WG4 Meeting #115	R4-2508019

St Julian’s, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS38.108 Introduction of NR-NTN combined L-band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_combinedLband-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NR-NTN combined L-band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SAN channel bandwidth for NR-NTN combined L-band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SAN channel bandwidth for NR-NTN combined L-band is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

5.3	Satellite Access Node channel bandwidth

5.3.1	General

The SAN channel bandwidth supports a single RF carrier in the uplink or downlink at the SAN. Different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the SAN. The placement of the UE channel bandwidth is flexible but can only be completely within the SAN channel bandwidth. The SAN shall be able to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the number of carrier resource blocks on the RF carrier, in any part of the carrier resource blocks.

The relationship between the channel bandwidth, the guard band and the transmission bandwidth configuration is shown in figure 5.3.1-1.

.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one channel

5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each SAN channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 35 MHz | 40 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 160 | 188 | 216 |
| 30 | N/A | 11 | 24 | 38 | 51 | 78 | 92 | 106 |
| 60 | N/A | N/A | 11 | 18 | 24 | 38 | 44 | 51 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-NTN

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

NOTE:	All Tx and Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN.

The transmission bandwidth configuration for NB-IoT is specified in TS 36.108 [17] clause 5.3B.

5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guard band for each SAN channel bandwidth and SCS is specified in table 5.3.3-1 for FR1-NTN and in table 5.3.3-2 for FR2-NTN.

Table 5.3.3-1: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 35 MHz | 40 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 | 572.5 | 552.5 |
| 30 | N/A | 505 | 665 | 645 | 805 | 945 | 925 | 905 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1290 | 1630 | 1610 |

Table 5.3.3-2: Minimum guardband (kHz) (FR2-NTN)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

The number of RBs configured in any SAN channel bandwidth shall ensure that the minimum guard band specified in this clause is met.

Figure 5.3.3-1: SAN PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol, the minimum guard band on each side of the carrier is the guard band applied at the configured SAN channel bandwidth for the numerology that is transmitted/received immediately adjacent to the guard band.

Figure 5.3.3-2: Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-2 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

5.3.4	RB alignment

For each SAN channel bandwidth and each numerology, SAN transmission bandwidth configuration must fulfil the minimum guard band requirement specified in clause 5.3.3.

For each numerology, its common resource blocks are specified in clause 4.4.4.3 in TS 38.211 [5], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.

For each numerology, all UE transmission bandwidth configurations indicated to UEs served by the SAN by higher layer parameter carrierBandwidth defined in TS 38.331 [6] shall fall within the SAN transmission bandwidth configuration.

5.3.5	SAN channel bandwidth per operating band

The requirements in this specification apply to the combination of SAN channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1-NTN and table 5.3.5-2 for FR2-NTN. The transmission bandwidth configuration in table 5.3.2-1 and table 5.3.2-2 shall be supported for each of the SAN channel bandwidths within the SAN capability. The SAN channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band |  | SCS (kHz) |  |  | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 3 | 5 |  | 10 |  | 15 |  | 20 | 30 (NOTE) |  |  | 35 (NOTE) |  | 40 (NOTE) |
|  |  | 15 |  |  | 3 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |
| n256 |  | 30 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |
|  |  | 15 |  |  | 3 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |
| n255 |  | 30 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |
|  |  | 15 |  |  | 3 | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |
| n254 |  | 30 |  |  |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |
|  |  | 15 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n253 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  |  | 5 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |
| n251 |  | 30 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |
|  |  | 15 |  |  | 5 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |
| n250 |  | 30 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |
| NOTE: Deployment of 30, 35, 40 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

# << End change >>