3GPP TSG-RAN WG4 Meeting #115	                                                	         R4-2508262

Malta, MT, 19 – 23 May, 2025

Agenda item:			7.11.7

Source:	Apple

Title:	WF on RRM requirements for NR_NTN_Ku_bands

Document for:	approval

# Topic #1: RRM impact of NR NTN on Ku bands

Issue 1-1: applicability update (section 3.6.12).

Agreement:

- Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

Issue 1-2: Assumption for defining RRM requirements in Ku-band NTN

Candidate options:

- Option 1: (LGE)

  - The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies;

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

- Option 2: other options are not precluded.

Issue 1-3: satellite switch requirements

Agreement:

- Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies

  - Define requirement for hard satellite switch,

    - Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms

      - Further discuss the details on the definition of each component.

  - FFS whether to define requirements for soft satellite switch

    - FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement

    - Option for the delay requirement:

      - Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Issue 1-4: Measurements of inter-frequency NR cells with NTN carrier (section 4.2.2.12).

Agreement:

- Do not extend the applicability of clause 4.2.2.12 to VSAT.

Issue 1-5: inter-satellite cell reselection requirements (section 4.2C).

Candidate options:

- Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19 (Nokia)

- Option 2: Do not define inter-satellite cell reselection requirements in this WI. (HW, ZTE, E///)

- Option 2a: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20). (LGE)

Issue 1-6: timing accuracy requirements for case 4 (section 7.1C)

Agreement:

- For timing requirement in Case 4, capture it as a NOTE in core requirement: The requirement applies when the interval between adjacent epoch times does not exceed 5.12 sec.

Issue 1-7: UE measurement capability (section 4.2C.2.1).

Agreement:

- Re-use existing measurement capability requirements for VSAT in Ku bands.

Issue 1-8: RLM (section 8.1C)

Candidate options:

- Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology.

- Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

Issue 1-9: terminology update

Agreement:

- For VSAT UE in Ku-band with FR1-NTN numerology, use the terminology of ‘VSAT UE in FR1-NTN’.