3GPP TSG-RAN WG4 Meeting #115	R4-2508266

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	WF on RRM requirements for NR_RRM_Ph5_Part2

Agenda Item:	7.18.1

Source: 	Moderator (CATT)

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [115][213] RRM requirements for NR_RRM_Ph5_Part2.

Topic #1: RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Sub-topic 1-1 RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement:

For the quitting condition of FBS:

- Introduce a new UAI indicating “overheating” issue:

  - RAN4 will not discuss the UE behaviour after reporting UAI

- Introduce a timer-based solution:

  - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition

    - Further discuss the candidate numbers for T1.

Way forward:

For re-evaluation mechanism for scenario 1/2/3/4:

- Check whether the following mechanism is agreeable:

  - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

Issue 1-1-2: Status report of fast beam sweeping

Agreement:

  - No need to define UE status report of fast beam sweeping.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

For FBS capability design:

  - UE capability is per-band and optional

  - TDD only

  - FR2-1 only

  - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

Issue 1-1-4: Applicability of L3 measurement delay reduction by optimizing Rx BSF

Proposals resolved in the related CRs. No agreement needed.

Topic #2: RRM requirements of Fast SCell activation for UE supporting Rel-18 EMR

Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-1-1: UE capability for EMR-based fast SCell activation

FFS:

- Option 1: Introduce a UE capability for R19 SCell activation based on EMR

  - Prerequisite feature groups: 39-8 or 39-9 in R18 RAN4 feature list

  - Need for the gNB to know if the feature is supported: No

  - Granularity: Per-UE

  - FDD/TDD differentiation: No

  - FR1/FR2 differentiation: Yes

- Option 2: Not to introduce additional UE capability

Issue 2-1-2: Updates to the EMR-based PUCCH SCell activation requirement in the CR

Proposals were treated in CR review phase. No agreement needed.

Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Test case list

Agreement:

  - Scenario:

    - Normal SCell activation for FR1 and FR2

    - Direct SCell activation at SCell addition for FR1

    - PUCCH SCell activation for FR1 and FR2

    - FFS:

      - Direct SCell activation at SCell addition for FR2

  - UE capabilities and network configurations:

    - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

    - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

    - FFS:

      - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Issue 2-2-2: Details of test case

Further discuss this issue.

Issue 2-2-3: Applicability rules of test case

Further discuss this issue.

References

- R4-2505562, Topic summary for [115][213] NR_RRM_Ph5_Part2, CATT, RAN4#115.

- R4-2508265, Ad-hoc minutes for NR_RRM_Ph5, Apple, RAN4#115.