3GPP TSG-RAN WG4 Meeting # 115											R4-2508271

Malta, MT, 19th – 23rd May, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part1

Agenda Item:	7.24.1

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM core requirements for UE-initiated/event-driven beam management >

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

- RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement”

- To check with RAN1 whether the parameter can be configured if RAN1 identify any issue

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams.

  - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air.

  - UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0.

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

Issue 1-4: CSI-RS based L1-RSRP for inter-cell BM?

Agreement:

- Reuse the same requirements structure as that in Rel-17 clause 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Issue 1-5: Whether to specify new unified TCI switching delay requirements?

Agreement:

- The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management

  - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion.

Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Nokia, Ericsson, MTK)

  - No need to capture this scenario in RAN4 requirements

- Option 2: (Xiaomi, Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Issue 1-7: Requirements for event-1 and event-7?

Agreement:

- For Event-1 and Even-7, reuse the same design as the requirements defined Event-2

Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

- Option 1: (Samsung, Apple, Huawei, Ericsson)

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2: (Nokia, MTK)

  - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

Issue 1-9: Compatibility of capabilities with LTM

<Way forward >

- If such capabilities are defined in Issue 1-8, then discuss on:

- Option 1:

  - The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO.

- Option 2:

  - The capability for supported number of events triggered L1 measurement reporting is defined as common for Rel-19 mobility and Rel-19 MIMO.

< Topic #2: RRM performance requirements for Rel-19 MIMO >

Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management

Issue 2-1-1: Measurement accuracy

Agreement:

- Confirm previous agreement in RAN4#114. No further discuss is needed.

Issue 2-1-2: Test scope

<Way forward >

- Further discuss based on the table below:

|  | QC | Nokia | Samsung | Apple | CMCC | Huawei | Ericsson |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Y |  | Event-2 1 TC for Event-2 No for Event-7 |  | Y | Event-2 FFS on others | Event-2&7 |
| Whether to cover Mode A and Mode B? |  | Mode A FFS on Mode B | Y |  |  |  | Mode A at least |
| Event-trigger for Intra-cell BM for SSB | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y |  |  | FFS | ? |
| Event-trigger for Inter-cell BM for CSI-RS |  | FFS |  |  |  |  |  |
| Whether to cover no time window and eventInstanceCount-r19 > 1? |  |  | Y |  | Y |  | Y |
| FR2? |  |  |  | Y |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI |  |  |  |  |  | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |

Issue 2-1-3: Whether to add requirement to verify the performance of CMI?

<Way forward >

- Option 1: (Xiaomi)

  - RAN4 to define requirement to verify the performance of condition met indicator (CMI) in the report.

  - RAN4 to discuss how to define performance metric for event triggered reporting

- Option 2:

  - Others are not precluded

Sub-topic 2-2: Performance requirements for CJTC reporting

Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

<Way forward >

- Option 1: (Samsung, Apple, CMCC)

  - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

<Way forward >

- Option 1: (Samsung)

  - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

- Option 2: (Apple)

  - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

- Option 3: (Ericsson)

  - Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for:

    - Different number of samples

    - High and low SNR

    - Normal and extreme conditions

Issue 2-2-3: Test scope

<Way forward >

- Further discuss based on the table below:

| High level: | QC | Nokia | Samsung | CMCC | Huawei | Ericsson |
| --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y | Y | Y | Y | FFS | Y |
| Whether to define TCs to verify CJTC-f，measurement period | Y | Y | Y | Y | FFS | Y |
| Whether to define TCs to verify CJTC-P， (To be verify?) | Y |  |  |  | FFS | Y |
| Whether to define TCs to verify scheduling restriction |  |  |  | Y | FFS |  |

Sub-topic 2-3: Performance requirements for enhancement for Asymmetric DL sTRP / UL mTRP

Issue 2-3-1: Test scope

<Way forward >

- Further discuss based on the table below:

| High level: | QC | Nokia | Apple | Samsung | CMCC | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y |  | Y |
| Whether to define TCs to verify TCI state switching requirements? | Y |  |  | N |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | both | both | both |  | both | Scenarios#1 |