3GPP TSG-RAN WG4 Meeting #115	R4-2508377

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR38.133 CG-SDT requirements for RedCap in NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Rel-19 work item NR_NTN_Ph3 specifies in objective 5 the support of (e)RedCap UEs in FR1-NTN. Thus, RRM core requirements need to be introduced for CG-SDT in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of RRM core requirements for CG-SDT in RRC_INACTIVE for (e)RedCap UEs in FR1-NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements for CG-SDT in RRC_INACTIVE for (e)RedCap UEs in FR1-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR …CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2507356 |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

## 5.5X	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN

### 5.5X.1	Introduction

The requirements in this clause are applicable for 1 Rx (e)RedCap and 2 Rx (e)RedCap UEs with FR1-NTN when the UE is configured with timing alignment (TA) validation for transmitting in uplink using CG-SDT as specified in TS 38.331 [2].

The 1 Rx (e)RedCap UE for determining whether to perform SDT procedure defined in clause 5.27 [7] applies:

-	sdt-RSRP-Threshold-r17 as the signaled value of sdt-RSRP-Threshold-r17 [2] + 1 dB.

### 5.5X.2	Requirements on UE synchronization for small data transmissions

The requirements in this clause are applicable for the UE performing small data transmissions using configured resources as TS 38.331 [2].

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA, NTA,adjcommon, and NTA,adjUE values as specified in subclause 7.1X.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell, then the UE shall drop the small data transmission. The UE determines the small data transmission occasion according to the received CG-SDT configuration [2].

### 5.5X.3	TA validation requirements

The UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA, NTA,adjcommon, and NTA,adjUE values as specified in subclause 7.1C before CG-SDT transmission provided that

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in TS 38.321 [7], and

- 	either cg-SDT-TimeAlignmentTimer is not configured or cg-SDT-TimeAlignmentTimer is running as confirmed by lower layers, as defined in clause 5.27.2 in TS 38.321 [7] with UE expected to not perform any threshold comparison against cg-SDT-RSRP-ChangeThreshold.

Additionally, for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640ms after T2, where T2 is the time when the UE performs TA validation as defined in clause 5.27.2 in (TS 38.321 [7]) for transmission using CG-SDT.

### 5.5X.4	Scheduling restriction

[FFS: The requirements in clause 5.5.4 shall apply for RedCap UEs with FR1-NTN.]

Editor’s Note: The above is under discussion in RAN4.

### 5.5X.5	Applicability conditions for SDT

The UE is not required to meet the following measurement requirements during subsequent SDT transmissions:

-	Measurements of inter-frequency NR cells in clause 5.1X.2.4

-	Measurements of inter-RAT E-UTRAN cells in clause 5.1X.2.5

--- End of change 1 ---