3GPP TSG-RAN WG4 Meeting #116												R4-2509038

Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.10.3.2

Source:	Huawei, HiSilicon

Title:	on NTN OTA tests

Document for:	approval

# Introduction

Last meeting recorded a few open issues [1] as shown below. This contribution discusses them further.

Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2

Agreements:

- For Scenario 2 UEs:

  - The DUT shall be mounted in a suitable hand phantom

  - The elevation tilt of the default position of the DUT can be:

    - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below

    - [±15, ±30, ±45] degrees; Other tilt angles are not precluded

    - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting

  - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration

Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere

Agreements:

- RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°.

  - Option 1:  single range is selected and captured in TR

  - Option 2:  multi-range in TR but based on UE declaration, details are FFS

- Note1: the phi range should be 360 degrees

- Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2.

Issue 2-1-6: Whether Using integration of EIRP/EIS or CDF?

Agreements:

- Integration is baseline.

- If different range is declared in issue 2-1-5, how to ensure the metric comparable should be discussed and resolved.

Issue 2-2-2: Whether Dwell time is considered during NTN OTA testing

Agreements:

- RAN4 further discuss and align understanding of NTN antenna tuning, if any.

- RAN4 further discuss whether Dwell time for NTN OTA test case is needed or not. a decision in RAN4 is needed.

# Discussion

On Issue 2-1-2, to provide more flexibility, the elevation tilt angle should be based on OEM declaration with 0 degree elevation tilt as default if there is no OEM declaration on elevation tilt.

Proposal 1: the elevation tilt angle can be based on OEM declaration with 0 degree elevation tilt as default if there is no OEM declaration on elevation tilt.

On Issue 2-1-5, ±90° range for a partial sphere would be good choice given that OEM can declare the elevation tilt angle. In addition, the ±90° range also covers cases where satellites have a large elevation angle relative to a device.

Proposal 2: use ±90° range for partial sphere as elevation tilt angle can be declared by OEM.

On Issue 2-1-6, it is difficult, if not impossible, to compare performance metrics with different ranges because radiation patterns from devices are irregular and not the same across devices. Therefore, use of different ranges should be avoided.

Proposal 3: different ranges in performance metric integration should be avoided.

On Issue 2-2-2, the need for dwell time comes from the possibility that some devices may have beam forming or satellite tracking ability, resulting changes in their radiation patterns in real time as highlighted in [2]. Typically beam forming or satellite tracking by NTN devices uses received signal as a reference. Satellites, well above devices in sky, provide such reference. However, in OTA test labs, the link antenna plays this role and could be located below test platform on which device is mounted or above test platform. Clearly, the situation where the link antenna is below the test platform is not conducive for beam forming or satellite tracking and device may behave erroneously. Even the cases where the link antenna is above test platform could result in inconsistencies among test labs because the link antenna location above test platform is not specified, giving rise to different link antenna locations across labs.

Possible solutions could be (a) specify a fixed location for link antenna above the test platform (b) switch off beamforming or satellite tracking functions. Neither solution is ideal because solution (a) would incur lab adaptation cost while solution (b) is not future proof as more and more devices could provide beamforming or satellite tracking functions in the coming years.

Proposal 4: discuss solutions for different link antenna locations in OTA test labs on measurement results for devices with beam forming or satellite tracking capabilities, for example (a) specify a fixed location for link antenna above the test platform (b) switch off beamforming or satellite tracking functions.

# Conclusion

This contribution makes the following proposals.

Proposal 1: the elevation tilt angle can be based on OEM declaration with 0 degree elevation tilt as default if there is no OEM declaration on elevation tilt.

Proposal 2: use ±90° range for partial sphere as elevation tilt angle can be declared by OEM.

Proposal 3: difference ranges in performance metric integration should be avoided.

Proposal 4: discuss solutions for different link antenna locations in OTA test labs on measurement results for devices with beam forming or satellite tracking capabilities, for example (a) specify a fixed location for link antenna above the test platform (b) switch off beamforming or satellite tracking functions.

# Reference

- R4-2508782 Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3

- R4-2505849 On Partial Radiated Surfaces