3GPP TSG RAN WG4 Meeting #116                  	           R4-2509144

Bengaluru, IN, 25th - 29th Aug, 2025

Source:	China Telecom

Title:	Discussion and simulation results for UE demodulation requirements for 7MHz

Agenda Item:	7.4.6

Document for:	discussion

Introduction

RAN4#115 is the first meeting for the UE demodulation performance part for demodulation and CSI requirements for 7MHz. Till now there are still some open issues in the approved WF [1].

In this contribution, our views on the remaining open issues as well as the simulation results for PDSCH CA are given.

Discussion

Release independence for 7MHz bandwidth requirements

- Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

According to the RF Core part agreement [2], the 7MHz CHBW requirements are release independent from Rel-15:

- 2-2-1 Release independence Agreement: - Support of 7 MHz channel BW could be Release independent from Rel-15 - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz. - Support of 7MHz is optional in Rel-19 and in the previous releases

Therefore, the 7MHz demodulation requirements should also be release independent from Rel-15, to follow the RF Core part.

Proposal 1: The 7MHz demodulation requirements should also be release independent from Rel-15, to follow the RF Core part.

Simulation Results

Based on the following agreed simulation assumption for PDSCH CA test for 7MHz CC in [1], our Ideal and impairment simulation result is provided.

Table. Simulation result for PDSCH CA test for 3MHz CC (FDD 15kHz SCS)

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Simulation results |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | Ideal SNR (dB) | Impairment SNR (dB) |
| 7 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.6 | 13.6 |
|  |  |  | 2x4, ULA Low | 70 | 5.5 | 7.5 |

Conclusion

Proposal 1: The 7MHz demodulation requirements should also be release independent from Rel-15, to follow the RF Core part.

Reference

R4-2508757, WF on demodulation and CSI requirements for 7MHz, Huawei, HiSilicon, RAN4#115, May 2025

R4-2502870, WF on requirements for 7MHz channel bandwidth	, Ericsson, RAN4#114, Feb 2025