3GPP TSG-RAN WG4 Meeting #116		                                                        R4-2509280

# Bengaluru , IN, 25th – 29th Aug, 2025

Title:	Discussion on RRM requirements for on-demand SSB SCell operation

Source:	CATT

Agenda Item:	7.23.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for OD-SSB SCell operation were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for on-demand SSB SCell operation and give our views.

- Discussion

Sub-topic 1-1: OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 2: OD-SSB in FMW collided with MG

In the last meeting, the issue for OD-SSB measurement if FMW collides with MG occasion haven’t been discussed, and the two options are copied as below [1]:

- Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In legacy, while measurement gaps are considered, the measurements in gaps are with higher priority than the measurement outside gaps. For OD-SSB based deactivated SCell measurement requirement in FMW, RAN4 has agreed that the length of FMW is Tuncertain + min(Treport , Tidentify), and Tidentify equals the minimum RAN4 OD-SSB measurement period requirement. In our view, UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion and the definition of scaling factor ‘Kp’ defined in Tidentify will reflect this UE behavior.

Proposal 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

In the last meeting, RAN4 has agreed the starting point of T2 and the definition of the minimum interval(T2) between two FMWs. For UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs, the two options are copied as below [1]:

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion. - Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2. - Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2. - Other option is not precluded.

In our view, T2 was introduced to avoid power consumption caused by frequently fast measurement of UE, to follow the intension, when OD-SSB further activation/reconfiguration occurs within T2, it is acceptable for UE to follow legacy deactivated SCell measurement after T2.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

In the last meeting, the two options for requirement for RAN1 Alt Time-C2 are copied as follows [1]:

Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion.

In Alt Time-C2 scenario, the combination AO-SSB and OD-SSB are aperiodic patterns, now that the network has sent OD-SSB, it will expect UE to measure OD-SSB, so we think that RAN4 to define the delay requirement only based on the OD-SSB periodicity regardless whether the periodicities of AO-SSB and OD-SSB are same, and there is no scheduling restriction is expected for AO-SSB.

Proposal 3: RAN4 to define the delay requirement only based on the OD-SSB periodicity in Alt Time-C2.

Proposal 4: No scheduling restriction is expected for AO-SSB in Alt Time-C2.

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies)

In RAN1 #120 meeting, RAN1 agreed to introduce a new scenario in which OD-SSB and AO-SSB have different frequency location. RAN2 has introduced a new servingCellMO to indicate OD-SSB and RAN4 confirmed in the last meeting. However, how to define the related requirements has not been discussed, and the agreement and diagram are copied as below [1]:

Sub issue 1: RAN2 LS for scenario 3 Agreement: - RAN4 to confirm RAN2’s WS to introduce a new servingCellMO. scenario 3：

In our view, scenario 3 is different with the legacy scenario since there are two servingCellMOs for the same serving cell. For measurement requirements, RAN4 need to discuss how to define intra-frequency and inter-frequency. RAN2’s agreement on the UE behavior of measuring serving cell is copied as follows:

10. When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

In our view, in scenario 3, the UE only measure one servingCellMO at the same time, and the specific UE behavior depends on OD-SSB activation/deactivation. Therefore, the intra-frequency is indicated by OD-SSB activation/deactivation, Otherwise, the intra-frequency shall be always CD-SSB(AO-SSB) frequency. When OD-SSB is activated, UE shall perform intra-frequency measurement on f1. After OD-SSB deactivation, UE will fall back to f0 as intra-frequency measurement. UE shall perform inter-frequency measurement on f1 before OD-SSB activation and after OD-SSB deactivation.

Proposal 5: If scenario 3 is supported, the intra-frequency is indicated by OD-SSB activation/deactivation, Otherwise, the intra-frequency shall be always CD-SSB(AO-SSB) frequency.

Proposal 6: If scenario 3 is supported,

- When OD-SSB is activated, UE shall perform intra-frequency measurement on f1. After OD-SSB deactivation, UE will fall back to f0 as intra-frequency measurement.

- UE shall perform inter-frequency measurement on f1 before OD-SSB activation and after OD-SSB deactivation.

Sub-topic 1-3: OD-SSB based SCell activation.

Issue 1-3-1: The known cell condition in Case 2

In the last meeting, RAN4 agreed the current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with combination. But there are still some FFS for other cases, and the agreement is copied as follows [1]:

Agreement - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination. - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB. - FFS whether apply to Alt Time-C2, if the requirement is defined. - FFS whether apply to scenario 3, if the requirement is defined. - FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

In our view, we don’t see any reasons that the current known SCell activation delay methodology can’t be reused for Alt Time-C2, so the agreement will also apply to Alt Time-C2 if the requirement is defined.

Proposal 7: The known cell condition agreement for Case 2 Alt Time-C1 can also be applied to Alt Time-C2.

Sub-topic 1-4: General issues

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

In last meeting, there were two options for the additional processing time for OD-SSB parameter update, which is copied as below [1]:

Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed

We prefer to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates, so the OD-SSB activation capability can be updated to include OD-SSB transmission parameter update.

Proposal 8: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

- The additional processing time is 5ms or 2ms based on UE capability reporting.

- Note 1: The additional processing time is only applied to L1 and L3 measurement transition period after the SCell is activated.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

In the last meeting, RAN4 confirmed RAN1’s working assumption about OD-SSB deactivation processing time. For the UE additional processing time for OD-SSB deactivation, RAN4 need to further discuss and there were still two options as below [1]:

Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

For OD-SSB deactivation, from the network perspective, it does not care about the subsequent behavior of UE after OD-SSB deactivation, as long as the UE knows that there is no need to measure the activated OD-SSB after decoding MAC CE, it seems that it doesn't matter if the UE processing time is longer or shorter, unless the OD-SSB is activated again very soon, which is a very corner case.

In addition, if there is no such UE additional processing time for OD-SSB deactivation, some companies pointed that when OD-SSB has already been deactivated, but UE has not finished processing, then the UE will assume that there is OD-SSB transmission and will continue to measure. In this case, if the network transmits data, the UE will miss the data reception, so scheduling restrictions need to be defined. However, in our view, OD-SSB is sent and measured periodically, so the time between two SSB bursts can be used to receive data. The above problems do not exist in most cases, unless the next transmission of OD-SSB burst happens to be after the MAC CE of OD-SSB deactivation, which is a very corner case.

Therefore, we prefer not to introduce additional processing time for OD-SSB deactivation.

Proposal 9: RAN4 not to introduce additional processing time for OD-SSB deactivation.

- Conclusion

In this paper, we have some further discussions on on-demand SSB SCell operation, and the following observations and proposals are given:

Proposal 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

Proposal 3: RAN4 to define the delay requirement only based on the OD-SSB periodicity in Alt Time-C2.

Proposal 4: No scheduling restriction is expected for AO-SSB in Alt Time-C2.

Proposal 5: If scenario 3 is supported, the intra-frequency is indicated by OD-SSB activation/deactivation, Otherwise, the intra-frequency shall be always CD-SSB(AO-SSB) frequency.

Proposal 6: If scenario 3 is supported,

- When OD-SSB is activated, UE shall perform intra-frequency measurement on f1. After OD-SSB deactivation, UE will fall back to f0 as intra-frequency measurement.

- UE shall perform inter-frequency measurement on f1 before OD-SSB activation and after OD-SSB deactivation.

Proposal 7: The known cell condition agreement for Case 2 Alt Time-C1 can also be applied to Alt Time-C2.

Proposal 8: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

- The additional processing time is 5ms or 2ms based on UE capability reporting.

- Note 1: The additional processing time is only applied to L1 and L3 measurement transition period after the SCell is activated.

Proposal 9: RAN4 not to introduce additional processing time for OD-SSB deactivation.

- Reference

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#115

- R4-2505570, Topic summary for [115][221] Netw_Energy_NR_enh_Part1, Ericsson, RAN4#115