3GPP TSG-RAN WG4 Meeting #116            	R4-2509409

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.6.5.3

Source:	Samsung

Title:	Discussion on demodulation requirement for NTN NGSO testing

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is NTN_testing_NGSO_channel_model with following

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel mode

In the last meeting, the demodulation requirement for NTN testing was under discussion, the related agreements are captured in the WF [2] as following

- RAN4 requirements - Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Considering the residual frequency error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. - Based on simulation result, further discuss whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Residual frequency offset and time delay - Option 1: Fix value - option 1a: frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s - option 1b: frequency error and timing error as 2Hz/s and 0.05us/s - Option 2: time-varying propagation delay and Doppler shift - FFS - Companies are encouraged to provide the simulation results during RAN4#116 meeting - Test cases for NR NTN with applying time-varying Doppler shift and propagation delay - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - Test cases for NB-IoT with applying the time-varying Doppler shift and propagation delay - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - Test case for eMTC with applying the time-varying Doppler shift and propagation delay model - Only consider test 1 and test 2 for cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - Testing time of demodulation test with NGSO LEO-600 - RAN4 introduce the channel model for both NGSO LEO-600 and LEO-1200. If the LEO-600 model is not enough to cover the test time of NR and IoT NTN test cases defined in RAN4, it is up to RAN5 to decide whether LEO-1200 will be used for testing

In this contribution, the view on the demodulation requirement impact for NR NTN and IoT NTN based on TE-emulated channel model with time-varying Doppler and delay shifts was provided. Meanwhile, the initial results with applying the time-varying Doppler shift and propagation delay model were also provided to check the impact.

# 2	Discussion

Fig. 1 test environment for NGSO NTN UE testing

As agreed in previous meeting, the test environment for NGSO NTN UE testing is illustrated in Fig. 1.  UE should estimate the doppler shift and propagation delay based on the satellite position, velocity and UE position information. The estimation and pre-compensation should be verified and processed before baseband. After pre-compensation, only residual frequency offset error or time offset can be handled by reference signal, such as TRS for NR and CRS for LTE eMTC/NB-IoT. Therefore, to check whether there is a demodulation requirement impact with applying the time-varying doppler shift and propagation delay channel model, we see one possible way is to add some residual frequency offset and time offset with considering the estimation and pre-compensation error at the baseband.

Regarding how to model the residual frequency offset and time offset, practically, TE will calculate and apply the Doppler shift and propagation delay every 1ms, and UE will estimate and pre-compensate the Doppler shift and propagation delay every slot based on the ephemeris information provided by TE per 10.24s.  Therefore, the residual doppler shift and time offset error should be also time-varying.

Observation 1: The residual frequency and time offset error should be time-varying.

For Rel-18 IoT NTN SAN demodulation requirement, the UL pre-compensation is only applied for first transmission with one segment. Due to the moving of satellite, the additional frequency doppler shift offset will be introduced, which may make the dropper shift at the end of segment different with the 1st repetition. In the previous meeting, regarding how to model the frequency shift and time offset, the following was considered

- When using segments size (Nsegmentprecompensation) greater than 1, for NPRACH, NPUSCH and NPUCCH, create additional frequency doppler shift offset for every subsequent repetition after the first in a segment. - Evaluate if an offset in transmit timing needs to be applied to the subsequent repetitions in a segment as well - For additional Doppler shift offset modeling - Option 1: gradual offset accumulation is considered for all Tx duration period in one segment. - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment - For additional Doppler offset applied physical channel - Option 1: only for NPUSCH format 1 and PUSCH - Option 2: for NPUSCH, PUSCH, PUCCH, NPRACH and PRACH - For additional transmit timing offset modeling - Option 1: gradual offset accumulation is considered for all Tx duration period in one segment. - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment - For additional transmit timing offset applied physical channel - Option 1: only for NPRACH and PRACH. - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment

Both gradual offset accumulation and the largest accumulated offset are modeling. Although the gradual offset can reflect the variation of doppler shift or propagation delay due to the mobility of satellite. While from the baseband processing perspective, if the largest offset can be tracked, there should be no issue for tracking the small value of doppler shift and time offset. According to companies result, the performance difference between two modeling methods is minor. As agreed in RAN4, the largest accumulated offset was applied for specifying requirement.

Similarly, we would like to apply the largest residual doppler shift and time offset error to check the impact of demodulation requirement with applying the time-varying doppler shift and propagation delay channel model.

According to the investigation of companies in [2], assuming UE estimate the Doppler shift and propagation delay by interpolation method based on ephemeris information signaled by TE per 10.24s as the worst case, where three different interpolation methods are assumed based ephemeris information. The maximum Doppler shift and time offset error can be observed as

Table 1: maximum Doppler shift and Time offset error based on different interpolation methods

| Method | Maximum Doppler Shift Error | Maximum Time offset Error |
| --- | --- | --- |
| Linear interpolation | 70Hz | 3.5us |
| Quadratic interpolation | 2Hz | 0.05us |
| Cubic spline interpolation | 1Hz | 0.035us |

Observation 2: As the worst case, UE estimate the Doppler shift and propagation delay by interpolation method based on signaled ephemeris information per 10.24s, where the maximum Doppler shift is 70Hz and maximum time offset is 3.5us.

According to core requirement, 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s was considered for UE frequency offset and timing error requirement assuming UE performs the pre-compensation operation as following, respectively. It is obviously that with linear interpolation the timing error cannot meet the core requirement.

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

On the other hand, as agreed in [3], UE will apply the Keplerian model and equation of motion models as baseline for RAN4 requirement, although there is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information. Therefore, UE can estimate the satellite position and velocity based on this model to calculate the Doppler shift and propagation delay. Then, the residual error will be small. We can take value as 2Hz with 0.05us as the residual frequency offset error and time offset error to check the impact of demodulation requirement

Proposal 1: RAN4 can take the residual frequency offset as 2Hz and time offset error as 0.05us to check the demodulation requirement impact with applying time-varying doppler shift and propagation delay channel model.

# 3	Simulation

In this section, we evaluate the performance impact for selected test cases with applying the time-varying doppler shift and propagation delay channel model.

Test cases for NR NTN

Based on agreement in the last meeting, 2 cases were selected to apply the time-varying Doppler shift and propagation delay model as following

Table 2 Minimum performance for Rank 1 for NR NTN PDSCH

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Frequency offset | Time offset | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 0Hz | 0 | 70 | -2.7 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 0Hz | 0- | 70 | 4.84 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 2Hz | 0.05us | 70 | -2.7 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 2Hz | 0.05us | 70 | 4.82 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |  |  |

For NR NTN test cases, based on the fixed residual frequency offset and time offset, the achieved SNR is almost same as that achieved without considering residual frequency offset and time offset. Considering the impairment margin will be included for requirement, we think we can reuse the SNR requirement for NR NTN PDSCH with applying the time varying Doppler shift and propagation delay channel model.

Observation 3: For NR NTN test cases, the achieved SNR under the fixed residual frequency offset and time offset, is almost same as that achieved without considering residual frequency offset and time offset.

Proposal 2： Reuse the same SNR requirement for NR NTN PDSCH with applying the time varying Doppler Shift and propagation delay channel model

Test cases for IoT NTN

Similar as NR NTN, only limited cases were selected for Cat1 M and NB-IoT NTN feature

Table 3 Minimum performance for single antenna port (FRC), cat M1, PDSCH

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Frequency offset | Time offset | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 0Hz | 0Ts | 70 | 5.09 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 0Hz | 0Ts | 70 | -10.57 | M1 |
| 3 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 2Hz | 0.05us | 70 | 5.1 | M1 |
| 4 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 2Hz | 0.05us | 70 | -10.56 | M1 |

Table 4 Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Frequency offset | Time offset | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 0Hz | 0 | 70% | -6.22 | NB1, NB2 |
| 2 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 2Hz | 2Ts | 70% | -6.21 | NB1, NB2 |

Similar as NR NTN test cases, for IoT NTN cases, based on the fixed residual frequency offset and time offset, the achieved SNR is almost same as that achieved without considering residual frequency offset and time offset. Therefore, for IoT NTN cases, we can still reuse the SNR requirement for IoT NTN Cat M and NPDSCH with applying the time varying Doppler shift and propagation delay channel model.

Observation 4:  For IoT NTN test cases, the achieved SNR under the fixed residual frequency offset and time offset, is almost same as that achieved without considering residual frequency offset and time offset

Proposal 3: Reuse the same SNR requirement for IoT NTN Cat M PDSCH and NPDSCH with applying the time varying Doppler shift and propagation delay channel model

# 4	Conclusion

In this contribution, the view on the demodulation requirement impact with time-varying Doppler shift and propagation delay is provided. And the simulation results are provided to check the performance impact with applying the time-varying Doppler shift and propagation delay.

Observation 1: The residual frequency and time offset error should be time-varying.

Observation 2: As the worst case, UE estimate the Doppler shift and propagation delay by interpolation method based on signaled ephemeris information per 10.24s, where the maximum Doppler shift is 70Hz and maximum time offset is 3.5us.

Proposal 1: RAN4 can take the residual frequency offset as 2Hz and time offset error as 0.05us to check the demodulation requirement impact with applying time-varying doppler shift and propagation delay channel model.

Observation 3: For NR NTN test cases, the achieved SNR under the fixed residual frequency offset and time offset, is almost same as that achieved without considering residual frequency offset and time offset.

Proposal 2： Reuse the same SNR requirement for NR NTN PDSCH with applying the time varying Doppler Shift and propagation delay channel model

Observation 4:  For IoT NTN test cases, the achieved SNR under the fixed residual frequency offset and time offset, is almost same as that achieved without considering residual frequency offset and time offset

Proposal 3: Reuse the same SNR requirement for IoT NTN Cat M PDSCH and NPDSCH with applying the time varying Doppler shift and propagation delay channel model

# References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4-2508625, Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod