3GPP RAN WG4 Meeting #116																	R4-2509535

Bangalore, India, 25th – 29th August 2025

Agenda item:	7.29.5

Source:	Apple

Title:	On A-MPR values for the IOT TDD band

WI/SI:	IoT_NTN_TDD-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. After that additional NTN bands were defined including a new spectrum WID [4], which will add the IOT TDD band operating in the same frequency range as the IOT band 254.

During the previous meetings RAN WG4 discussed which regulations should be followed for this IOT TDD band, which NS flags to adopt and, as a result, whether any A-MPR would be needed. In addition to that, during the last RAN4#115 meeting several companies also raised some concerns on potentially high A-MPR values with ETSI regulations and whether it would be possible to avoid high A-MPR.

# 2	A-MPR values for IOT TDD band

## 2.1	Background

Frequency range of 1610-1626.5MHz and its usage for the MSS services has well established international and local regulations, captured in the corresponding WRC provisions and local standards. This was already discussed by 3GPP in Rel-18 during standardisation of the NTN band 254/n254. And since existing regulations and requirements do not differentiate between the FDD and TDD modes, our understanding is that the same regulatory framework is applied to a new IOT TDD band 249 operating in 1616-1626.5MHz frequency range. To be more precise, in case of FCC it will be covered by Title 47 CFR § 25.202 and § 25.216; and in case of ETSI it will be ETSI EN 301441. For countries and regions following neither FCC nor ETSI rules, generic 3GPP requirements would apply which are usually based on the international regulations.

## 2.2	Power back-off simulations

Speaking of the ETSI regulations for this frequency range, in addition to the out-of-band emission requirements there also exist so-called in-band requirements. As discussed extensively during the previous meetings, ETSI in-band emission mask is not the same as the NB-IOT basic mask and thus it may require additional power back-off. To estimate potential power back-off needed in this case we ran a set of power back-off simulations for both FCC and ETSI regulations, summary for which is presented below in Table 2.2-1.  Power back-off simulations for the FCC regulations can be viewed as a logical calibration baseline because they should not cause any power back-off.  Annex A also presents more detailed spectrum plots for the corresponding configurations with different number of tones.

Table 2.2-1: Summary of power back-off values for PC3 (23dBm)

| Number of tones | FCC 1610-1626.5 MHz | ETSI |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | 1610-1618.25 MHz | 1618.25-1626.5 MHz |  |
| 12 | 0 | 0 | 0 | Full RB 12-tone allocation |
| 6 | 0 | 0 | 1 | 0-5 and 6-11 tones |
| 3 | 0 | 0.5 | 1.5 | 0-2 and 9-11 tones |
| 1 | 0 | 0.5 | 1.5 | 0th and 11th tone for the PSD limit of -3dBW/4kHz (33.3dBm/15kHz) |
|  | 0 | 2 | 2 | 0-11 tones for the PSD limit of -15dBW/4kHz (21.3dBm/15kHz) |

According to our simulations, we might need some power back-off for cases when only a few tones are scheduled, and the tones are at the edge of the 12-tone RB. In this case all energy is concentrated in the edge tones, which in conjunction with small NB-IOT guard bands may indeed require additional power back-off. It is also worth noting that since ETSI regulations have two PSD limits for this frequency range then we will have no option, but to apply at least 2dB power back-off when a regulatory enforces a more stringent -15dBW/4kHz (21.3dBm/15kHz) restriction. Nevertheless, according to our results the anticipated power back-off values are relatively low.

On the contrary to the presented results, there were contributions during the previous RAN4#115 meeting indicating that much larger power back-off values are needed for the edge allocations. While reasons for high power back-off values are not entirely understood, we believe that 3GPP can work further to agree on the acceptable values that would account for a wide range of devices with different HW capabilities.

Proposal 1:	Define, if needed, A-MPR values for the IOT TDD band 249 subject for further discussions on how large A-MPR values should be.

Proposal 2:	Other solutions, such as extended/nominated channel bandwidth, can be considered as a more generic topic.

# 3	Conclusions

In this discussion paper we have presented our considerations on the IOT TDD band regulations and anticipated power back-off values. Our understanding is that since this band falls into the same frequency range as band 254 UL, the same international and local regulations would apply, for which we conducted additional power back-off simulations. And while some allocations may indeed require additional A-MPR, our understanding is that it is like other satellites bands where A-MPR is also defined. In addition to that, 3GPP can of course consider other techniques to mitigate A-MPR, such as nominated channel bandwidth, which is however much wider discussion that the spectrum IOT TDD WI [5].

Proposal 1:	Define, if needed, A-MPR values for the IOT TDD band 249 subject for further discussions on how large A-MPR values should be.

Proposal 2:	Other solutions, such as extended/nominated channel bandwidth, can be considered as a more generic topic.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-230792, "New WID: FDD LTE band (L+S band) for IoT NTN operation", MediaTek

RP-242415, "New WID for introduction of IoT-NTN TDD mode", Iridium Satellite LLC

R4-2511499, "On nominated channel bandwidth concept in the 3GPP specifications", Apple

Annex A: Power back-off results for 12, 6, 3 and 1-tone allocations

Figure A-1: 12-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure A-2: 6-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure A-3: 3-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure A-4: 1-tone spectrum plots (FCC, ETSI sub-range 1 and 2)