3GPP TSG-RAN WG4 Meeting #116                	                R4-2509593

Bengaluru, IN, 25th - 29th Aug 2025

Title:	RRM Core requirements on CSI-RS based L1 measurement for LTM

Source:	China Telecom

Agenda item:			7.25.3.2

Document for:	Discussion

- Introduction

In the RAN4 #115 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on CSI-RS based L1 measurement for LTM.

- Discussion

Issue 3-2: whether step 2 is skipped for serving cell

Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

In the previous release, L1-RSRP measurements for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP have been defined.

So, we think for workload, it is better to keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

Proposal 1: Keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

Issue 3-2: early CSI acquisition

Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

In previous RAN1 meeting, they agreed to support early CSI acquisition. But it is different with CSI-RS based L1-RSRP measurement before cell switching, So, a lot of issues on early CSI acquisition need to be discussed. Considering limited time and workload, we prefer not define RAN4 requirements for early CSI acquisition.

Proposal 2: Not define RAN4 requirements for early CSI acquisition.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

Proposal 2: Not define RAN4 requirements for early CSI acquisition.

- References

[1] R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.