3GPP TSG-RAN WG4 Meeting #116	R4-2509628

# Bengarluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM performance requirements for Rel-19 NES

Agenda item:	7.23.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM performance requirements for Rel-19 network energy saving.

Discussion

OD-SSB performance part

Test scope

Sub-issue 1: Test scope Issue 3-1-1: Scenario RAN4 to discuss whether the following test scenarios can be agreed. - RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. - RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a. - For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B. - For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell. - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

In the core requirement for OD-SSB, RAN4 introduced FMW for deactivated SCell measurement, SCell activation and interruption for OD-SSB for Case 1 and Case 2. For OD-SSB test scope, following can be considered,

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Proposal 1: RAN4 to define following test scope for OD-SSB

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Test case list

Sub-issue 2: Test case list Issue 3-1-3: Test case list RAN4 to discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 1 - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

In the core requirement for OD-SSB, RAN4 introduced FMW for deactivated SCell measurement, SCell activation and interruption for OD-SSB for Case 1 and Case 2. Regarding SCell activation test case for Case 2, in order to reduce the dupliacated test cases, we think only SCell activation in Case 2 Alt time C-1 is needed. Thus, for OD-SSB test case, following can be considered:

  - SCell activation in Case 1

  - SCell activation in Case 2 Alt Time C-1

  - OD-SSB based multiple SCell activation

  - OD-SSB based direct SCell Activation at SCell addition in Case 1

  - OD-SSB based PUCCH SCell Activation in Case 1

  - Fast SCell Activation with OD-SSB;

  - OD-SSB based SCell Activation with the L3 reporting during activation

  - Intra-frequency measurement(deactivated SCell) in Case 1

  - Intra-frequency measurement(deactivated SCell) in Case 2

  - Intra-frequency measurement in Case 1

  - Intra-frequency measurement in Case 2

  - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

  - Interruption at OD-SSB activation/deactivation

  - Interruptions during measurements on deactivated NR SCC

Proposal 2: RAN4 to define following test cases for OD-SSB

  - SCell activation in Case 1

  - SCell activation in Case 2 Alt Time C-1

  - OD-SSB based multiple SCell activation

  - OD-SSB based direct SCell Activation at SCell addition in Case 1

  - OD-SSB based PUCCH SCell Activation in Case 1

  - Fast SCell Activation with OD-SSB;

  - OD-SSB based SCell Activation with the L3 reporting during activation

  - Intra-frequency measurement(deactivated SCell) in Case 1

  - Intra-frequency measurement(deactivated SCell) in Case 2

  - Intra-frequency measurement in Case 1

  - Intra-frequency measurement in Case 2

  - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

  - Interruption at OD-SSB activation/deactivation

  - Interruptions during measurements on deactivated NR SCC

Test configuration

Sub-issue 3: Test configuration Issue 3-1-4: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - At least test OD-SSB periodicities with 5ms, 20ms. FFS 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms

For OD-SSB test configuration, following can be considered:

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 5ms, 20ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

Proposal 3: RAN4 to apply following principle for OD-SSB test

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 5ms, 20ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

SSB adaptation performance part

Issue 3-3-1: Test case list RAN4 to discuss whether the following SSB adaptation test case list can be agreed. - L1-RSRP - L1-SINR - CBD - L3 measurement - Deactivated SCell measurement - normal SCell activation - direct SCell activation - PUCCH SCell activation Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection Issue 3-3-3: SMTC and SSB configuration - Proposals - Option 1: - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation. - Option 2: - SSB periodicity: 40 ms to 20 ms - SMTC periodicity: 40ms - Option 3: - SSB periodicity from 80ms to 20ms - SMTC periodicity from 80ms to 20ms

Test case list

In the core requirement for SSB adaptation, RAN4 introduced L1/L3 measurement requirement with updating SSB periodicity. For SSB adpatation test case list, following can be considered,

  - L1-RSRP

  - L1-SINR

  - CBD

  - L3 measurement

  - Deactivated SCell measurement

  - normal SCell activation

  - direct SCell activation

  - PUCCH SCell activation

Proposal 4: RAN4 to consider following test case list for SSB adaptation

  - L1-RSRP

  - L1-SINR

  - CBD

  - L3 measurement

  - Deactivated SCell measurement

  - normal SCell activation

  - direct SCell activation

  - PUCCH SCell activation

OD-SIB1 performance part

Test case

Issue 3-2-1: Test case - Proposals - Option 1: RAN4 not to define OD-SIB1 RRM performance test case. - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

In the core requirement for SSB adaptation, no specification impact was identified. Prefer not to define RRM performance tset case for OD-SIB1 test case.

Proposal 5: RAN4 not to define OD-SIB1 RRM performance test case.

Conclusion

In this contribution, we provide views on RRM performance requirement for Rel-19 NES

Proposal 1: RAN4 to define following test scope for OD-SSB

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Proposal 2: RAN4 to define following test cases for OD-SSB

  - SCell activation in Case 1

  - SCell activation in Case 2 Alt Time C-1

  - OD-SSB based multiple SCell activation

  - OD-SSB based direct SCell Activation at SCell addition in Case 1

  - OD-SSB based PUCCH SCell Activation in Case 1

  - Fast SCell Activation with OD-SSB;

  - OD-SSB based SCell Activation with the L3 reporting during activation

  - Intra-frequency measurement(deactivated SCell) in Case 1

  - Intra-frequency measurement(deactivated SCell) in Case 2

  - Intra-frequency measurement in Case 1

  - Intra-frequency measurement in Case 2

  - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

  - Interruption at OD-SSB activation/deactivation

  - Interruptions during measurements on deactivated NR SCC

Proposal 3: RAN4 to apply following principle for OD-SSB test

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 5ms, 20ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

Proposal 4: RAN4 to consider following test case list for SSB adaptation

  - L1-RSRP

  - L1-SINR

  - CBD

  - L3 measurement

  - Deactivated SCell measurement

  - normal SCell activation

  - direct SCell activation

  - PUCCH SCell activation

Proposal 5: RAN4 not to define OD-SIB1 RRM performance test case.

Reference

[1]