# 3GPP TSG-RAN WG4 Meeting #116	R4-2509711

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.7

Source:	OPPO

Title:	Discussion on test case design for A-IoT

Document for:	Approval

- Introduction

This contribution will provide our opinions on the test case design for A-IoT device 1.

- Discussion

Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

As shown above, last RAN4 meeting discussed and reached the agreement on the D2R transmit timing requirements. Although the timing error is dependent on the 10% SFO, RF session already agreed to not directly test SFO itself if SFO can be indirectly verified via other tests. Then, it is natural to introduce related test cases to verify this timing error requirement. As the starting point, the transmission timing for MSG 1 can be considered.

Proposal 1: Introduce test case to verify D2R transmit timing requirements for MSG 1.

Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s).

X=1 or X=2 TDMA MSG 1 resources can be supported and it will be indicated by reader. From RRM perspective, the same principle is used when defining the timing error requirements. For simplicity, set X=1 in the test case.

Proposal 2: Set X=1 for MSG 1 transmission in the test case.

Working assumption For Toffset1, for CBRA, regardless of the use of FEC, - The reference D2R chip length is the largest D2R chip length among the scheduled FDMed Msg1 for Y>1 or the D2R chip length for Y=1. - If reference D2R chip length is 133.33 or 66,67us, Toffset1 = 20* 66.67=1333.4us - If reference D2R chip length is 33.33, 16.67, or 8.33us, Toffset1 = 20* 33.33=666.6us - If reference D2R chip length is 4.17us, Toffset1 = 4* 33.33=133.32us - If reference D2R chip length is 2.08, 1.04, or 0.69us, - If the R2D chip length is 33.33us, Toffset1 = 4* 33.33=133.32us - If the R2D chip length is 11.11, 5.56 or 2.78us, Toffset1 = 3*11.11=33.33us - The R2D chip length refers to the corresponding R2D transmission triggering the D2R

Toffset1 is used to determine the reference point of MSG 1 transmission timing, and will further impact the length of TSFO. Based on the above RAN1 agreements, the value of Toffset1 is related with the reference D2R chip length and R2D chip length, which is scheduled be reader. We propose to consider all the values of Toffset1, and pick one D2R / R2D chip length for each value as listed below.

Proposal 3: Consider the multiple Toffset1 values as different configuration in the test case:

- Configuration 1: Toffset1 = 1333.4us, reference D2R chip length is 133.33us

- Configuration 2: Toffset1 = 666.6us, reference D2R chip length is 33.33us

- Configuration 3: Toffset1 = 132.32us, reference D2R chip length is 4.17us

- Configuration 4: Toffset1 = 33.33us, reference D2R chip length is 2.08us, R2D chip length is 11.11us

- Conclusion

This contribution gave our considerations on test cases for D2R transmit timing requirements and the following proposals.

Proposal 1: Introduce test case to verify D2R transmit timing requirements for MSG 1.

Proposal 2: Set X=1 for MSG 1 transmission in the test case.

Proposal 3: Consider the multiple Toffset1 values as different configuration in the test case:

- Configuration 1: Toffset1 = 1333.4us, reference D2R chip length is 133.33us

- Configuration 2: Toffset1 = 666.6us, reference D2R chip length is 33.33us

- Configuration 3: Toffset1 = 132.32us, reference D2R chip length is 4.17us

- Configuration 4: Toffset1 = 33.33us, reference D2R chip length is 2.08us, R2D chip length is 11.11us

References

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC, RAN4 #115