3GPP TSG-RAN WG4 Meeting 116	R4-2509763

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility

Agenda Item:	7.19.3.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss RRM impact on AI mobility.

- Relative RSRP prediction accuracy requirement - Ground truth - Intra-frequency temporal domain prediction - Inter-frequency inter-cell prediction - Impacts of measurement error

- Discussion

      - 2.1 Relative RRM prediction accuracy

- Issue 2-1: Relative RSRP accuracy definition - Agreement: - For intra-frequency L3-RSRP: Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2); cell 1 and cell 2 are on the same frequency; the reported RSRP of cell2 can be measured or predicted; FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - For inter-frequency L3-RSRP: Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2); cell 2 is on a different frequency than cell 1 but in the same FR as cell 1; the reported RSRP of cell2 can be measured or predicted; FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

There is no need to consider the time offset. Because in the past, when we defined relative accuracy in non-AI scenarios, especially in inter-frequency scenarios, the measurement times might be different. Even though the times when the results of the two cells are obtained are not the same, we did not take into account the time difference between the two measurement tasks when defining relative accuracy. Following this previous practice, it is unnecessary to consider the time difference here as well.

Proposal 1: Don’t need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

      - 2.2 Ground truth

It’s agreed that absolute L3 predicted RSRP accuracy is the delta between reported L3-RSRP and ground truth of L3-RSRP.

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

The analysis is as below:

| Issues | Impact to Alternative 1 (Ground truth: Transmitted/reception power) | Impact to Alternative 2 (Ground truth: Reported measurement under certain conditions) |
| --- | --- | --- |
| 1. The information at TE side, on the timing UE performs and/or finishes the measurement and/or prediction | High impact. Fading channel introduces time-domain fluctuations; timing mismatch between predicted and ground truth L3-RSRP causes significant differences (e.g., 5.5dB at 200ms mismatch). Alignment depends on RAN2's reporting scheme, with potential need for additional margins if alignment fails. | Low impact. Ground truth is UE-reported, so timing is tied to UE's measurement/prediction completion; TE can rely on UE's reported time instance (similar to AI BM-case 2) to align, reducing timing mismatch issues. |
| 2. Impact of L3 filtering | High impact. L3 filtering method (sliding vs non-sliding) and coefficient (α) affect ideal L3-RSRP derivation. For α≠0 (e.g., k=4, α=0.5), 90% of cases show up to 1.8dB difference between modes. TE must know the filtering method to calculate accurate ground truth, or use k=0 (no filtering) with extra margins. | Low impact. Ground truth is UE-reported, which inherently incorporates UE's L3 filtering; no need for TE to derive it, avoiding mismatches from filtering differences. |
| 3. Number of samples of L1 filtering | High impact. L1 sample count (e.g., 3 vs 5 samples) affects ideal L3-RSRP, with 90% of cases showing ≤3dB difference. TE must align with UE's L1 sample number to calculate accurate ground truth, limiting UE implementation flexibility unless extra margins are added. | Low impact. Ground truth is based on UE's actual L1 filtering (with sample count determined by UE implementation), so TE does not need to enforce or know the sample number, avoiding implementation limitations. |
| 4. Timing window where UE can’t measure the ground truth in Alt2 | Not applicable. Transmitted/reception power exists regardless of UE measurement capability; no "unmeasurable" window for ground truth. | Low impact. UE reports ground truth under high SNR to reduce errors, but if UE can’t measure in a window, ground truth gaps may occur. However, anti-cheating strategies (e.g., randomized channels) can mitigate intentional gaps. |
| 5. Channel condition | High impact. Fading channel causes time-domain fluctuations, directly affecting reception power (ground truth source) and increasing variability in ideal L3-RSRP, requiring stricter timing and filtering alignment. | Moderate impact. Channel conditions influence UE-reported measurements, but high SNR reporting reduces this impact. Anti-cheating strategies (e.g., multi-instance scrambling) further stabilize ground truth. |
| 6. Avoid UE cheat in the test | High robustness. Transmitted/reception power is objective and not manipulable by UE, eliminating cheating risks. | High impact. UE could falsify reported ground truth, requiring anti-cheating strategies (multi-instance channel scrambling, redundant channels, randomized temporal offsets) to ensure reliability. |
| 7. S(I)NR variations in the test at different time instances (for Alt2) | High impact. Fading channel leads to S(I)NR variations, which affect reception power (ground truth) and increase ideal L3-RSRP variability, necessitating alignment and margins. | Moderate impact. S(I)NR variations affect UE-reported measurements, but high SNR reporting and anti-cheating strategies help maintain ground truth consistency. |

This table summarizes the comparative impacts of various issues on Alternative 1 and Alternative 2, highlighting key differences in robustness, applicability, and potential mitigation strategies.

Impact of timing instance mismatch between predicted RSRP and ground truth RSRP

For evaluate RSRP prediction accuracy, time instance for predicted L3 RSRP and ground truth L3 RSRP needs to be aligned. In legacy, AWGN channel is assumed and there is no impact. However, for fading channel, the timing needs to be aligned. We try to analyze the RSRP difference due to timing mismatch.

Suppose that UE speed = 30km/h and RS periodicity is 40ms, according to RAN2 simulation. For simplicity, TDL-C channel is applied.

Here, we plot the two ideal L3 RSRP difference with time mismatch = 40ms/80ms/160ms/200ms. Each L3 RSRP is averaged by 5 samples. As shown below, when time gap increases, RSRP difference increases. When timing mismatch is 40ms, RSRP difference is within 3dB for 90% cases. When timing mismatch increases to 200ms, RSRP difference increases to 5.5dB.

Fig.1 L3-RSRP difference due to timing mismatch

Observation 1: When timing mismatch increases to 200ms, RSRP difference increases to 5.5dB.

How to align timing

Since RSRP difference between different time instance is large, if TE will derive the ground truth of RSRP, the time instance of reported predicted L3-RSRP needs to be clearly known by TE. Currently, it’s FFS How to report predicated L3-RSRP in RAN2, it’s not clear whether TE can exactly know the timing.

if similar as AI BM-case 2, UE also report the time instance for predicted RSRP, then it’s possible for TE to know the predicted timing.

If similar as legacy, UE will only report the latest predicted L3-RSRP in the report, it’s also possible to alignment the timing by reporting configuration. Here, we would like to highlight one thing that the prediction interval for sliding and non-sliding mode is different. For sliding mode, prediction interval is just one L1 RSRP interval. While for non-sliding, prediction interval is just multiple times of L1 RSRP interval. As shown below, the time gaps for non-sliding and sliding mode are T1 and T2 respectively. Therefore, from TE aspect, it can’t decide the ground truth since the time instances corresponding to the predicted L3 RSRP is different.

Fig.2 different prediction interval for sliding and non-sliding

Observation 2: Prediction interval for sliding and non-sliding widow is different, which will have impact on degerming the reported time instance.

By designing reporting configuration, the reporting periodicity shall at least be multiple times of prediction interval for non-sliding mode to guarantee that reported L3-RSRP will correspond to the same time instance. Since the prediction interval for non-sliding mode will be measurement period. Therefore, the reporting periodicity shall at least be multiple times of measurement period.

In summary, how to align timing is highly dependent on reporting scheme designed by RAN2.

- whether NW can explicitly indicate the prediction time instance for report

- whether UE can explicitly indicate the time instance for reported prediction

- if no explicit indication, FFS how to align by reporting configuration

Proposal 2: Timing between predicated L3-RSRP and ground truth L3-RSRP needs to be aligned, how to align timing is highly dependent on reporting scheme designed by RAN2. If timing alignment is infeasible, incorporate additional margins into accuracy thresholds.

In current measurement period requirement in 38.133, 5 samples are defined for L1 filtering as minimum requirement. In fact, some advanced UE can use fewer samples to achieve RSRP accuracy, i.e. 3 samples, and can send filtered L1-RSRP to high layer more quickly. If that is the case, the ideal L3-RSRP will be calculated based on average of 3 samples but not 5 samples. In current AI mobility studied in RAN2, if they consider L3-RSRP to predict L3-RSRP, they don’t have limitation on the L1 filtering sample, it’s up to UE implementation.

If L3-RSRP can be calculated at TE side, it means that TE needs to know how many L1 samples will be used to generate ideal L3-RSRP. It will have highly limitation on UE implementation. It means that UE must average based on 5 samples. Otherwise, there will be mismatch.

Fig.3 L3-RSRP difference due to L1 filtering sample mismatch

Observation 3: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side.

Observation 4: if 3 samples are used to derive L1 filtered RSRP, RSRP delta will be within 3dB for 90% cases when compared with that case that 5 samples are used.

Proposal 3: L1 filtering number needs to be aligned, otherwise, extra margin needs to be considered.

The key difference between non-sliding filtering and sliding filtering is that:

  - For L1/L3 filtering non-sliding window, every 5 L1 samples will generate a new L3 RSRP.

  - For L1/L3 filtering sliding window, every 1 L1 sample will generate a new L3 RSRP.

If TE didn’t know the L1/L3 filtering method, TE can’t know how to calculate the ideal L3-RSRP. According to 3GPP TS 38.331, the L3 filtering formula is as follows:

Fn​=(1−a)⋅Fn−1​+a⋅Mn​

where:

- Fn​ is the updated filtered measurement result used to evaluate reporting conditions or for measurement reporting.

- Fn−1​ is the previous filtered measurement result, with the initial value F0​ set to the first measurement result M1​ received from the physical layer.

- Mn​ is the latest measurement result received from the physical layer.

- α is the filtering coefficient, a = 1/2(k/4), where k is the filterCoefficient

Fig.3 different time duration length to calculate ideal L3-RSRP at time T

Suppose a = 0.5. Many L3 result will have impact on current L3 results. suppose that totally 4 L3 samples will contribute to the L3-RSRP at time instance T. the coefficient of each L3 result is 1/2, 1/4, 1/8, 1/16, 1/32. As the L3 result periodicity of sliding window is smaller than that of non-sliding mode. For sliding window mode, the impact of previous L3 RSRP will reduce more quickly, i.e. T1 is smaller than T2. As shown in the figure, when considering impact of previous RSRP to be 1/16, there are total 8 samples for sliding window and 32 samples for non-sliding window.

Consequently, the ground truth of L3-RSRP at time T differs between sliding and non-sliding filtering methods.

RSRP difference = ideal filtered L3-RSRP of non-sliding method at time T - ideal filtered L3-RSRP of sliding method at time T

Fig.3 L3-RSRP difference due to L3 filtering

α is the filtering coefficient, a = 1/2(k/4). If k = 0, no L3 filtering is applied, as analysed, there is no RSRP difference between sliding and non-sliding mode. In current RAN2 simulation, k=4 is assumed, then α = 0.5

L3 RSRP difference between two modes can be within 1.8 dB in 90% cases.

An alternative approach is to disregard the L3 filtering factor α, meaning L3-RSRP is equivalent to the measurement at point C with α=0, similar to L1-RSRP at point A1. In this scenario, the ideal L3-RSRP is independent of the L1/L3 filtering method. Regardless of the method applied, L3-RSRP equals the average of M instantaneous L1-RSRP samples.

Observation 5: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side.

Observation 6: At the same time T, ideal L3 filtered RSRP difference between non-sliding and sliding mode can be small than 1.8dB for k=4 in 90% cases. If K=0, no L3 filtering is applied and there is no RSRP difference.

Proposal 4: L3 filtering impact can be removed by setting k=0, extra margin needs to be considered.

key factors for option 2

For this case, ground truth of L3-RSRP will be measured by UE and reported to TE, there is no UE implementation limitation. UE will report the ground truth RSRP under high SNR in order to reduce the impact of measurement error.

However, it needs to solve UE cheating issue. For FR2, similar scheme of AI BM may be re-used. For FR1, RAN4 needs to design schemes.

Test Equipment (TE) can employ multiple anti-cheating strategies such as:

- Multi-instance channel scrambling: Randomizing the order of predefined channel sequences.

- Insertion of redundant channels: Introducing randomly generated non-functional channels.

- Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified interval to disrupt UE's pattern recognition.

The following describes a multi-instance channel scrambling implementation to mitigate UE cheating:

- Prediction Phase: The UE reports predicted values for multiple future time instances, as shown in Fig.4(a).

- Validation Phase:

  - The Test Equipment (TE) sends Channel Sequence B (identical to Phase 1) but randomly shuffles the channel occurrence order (e.g., B2 → B1 → B3), as shown in Fig.4(b).

  - The UE is required to report the complete set of measurement values for all time instances: {True_T1, True_T2, True_T3}.

  - The TE matches predicted values with actual measurements at each time instance (T1-T3) based on the shuffled channel sequence, then calculates prediction errors across all instances.

(a) Original channel

(b) randomly shuffles the channel

Proposal 5: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Test Equipment (TE) can employ multiple anti-cheating strategies such as:

- Multi-instance channel scrambling: Randomizing the order of predefined channel sequences.

- Insertion of redundant channels: Introducing randomly generated non-functional channels.

- Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified interval to disrupt UE's pattern recognition.

2.3 Intra-frequency temporal domain prediction

- 2.3.1 RRM impact

RAN2 prioritize 3 scenarios:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Two scenarios are related to intra-frequency intra-cell prediction:

- FR1 to FR1 intra-frequency temporal domain case B

- FR2 to FR2 intra-frequency temporal domain case A

Case A and case B

In TR[1], RAN2 defines observation window and prediction window, which will have impact on RAN4.

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

For legacy L3 RSRP measurement requirement, measurement will be conducted periodically. RAN4 defined measurement period which is required to satisfy the L1 filtered RSRP accuracy requirement. However, when AI is applied, some measurement instance can be skipped. Take case B for example, the target is to reduce measurement in temporal. Therefore, the measured samples will be reduced.

In prediction window, UE can skip measurement. It’s possible that scheduling restriction can be relaxed.

Proposal 6: Observation window and prediction window defined in RAN2 will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

- 2.3.2 Observation window and prediction window

Observation window and prediction length is dependent on different sub-cases, different pattern and different L1/L3 filtering method.

Impact of L1/L3 sliding filtering

RAN2 defines L1/L3 sliding filtering and non-sliding filtering. Therefore, the final observation window length for the two cases is different for L3 RSRP to L3 RSRP prediction case.

Observation 7: Observation window and prediction length is dependent on different L1/L3 filtering method.

Impact of different sub-cases

Currently, RAN2 defined 3 subcases for RSRP prediction.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

- Sub-case 1: RSRP is predicted at L1

For sub-case 1, the prediction happens at L1. In legacy, 5 L1 samples are needed to be measured in one measurement period, which is shown in Fig.4(a). While when AI is applied, for example, only 3 samples are needed to be measured during observation window. Then UE can predict other samples with grey colour, as shown in Fig.4(b). Then measurement period can be reduced. Here observation window is similar as legacy measurement period with reduced samples. Observation window and prediction window length are based on number of L1 samples.

Fig.4(a) measurement period for non-AI L1 filtered RSRP for FR1

Fig.4(b) observation window and prediction window for AI based L1 filtered RSRP

- Sub-case 2: RSRP is predicted at L3

For sub-case 2, we give examples for non-sliding L1/L3 filtering case.

For sub-case 2, filtered L3 RSRP will be predicted by L3 RSRP. Suppose that every 2 L3 filtered RSRP can predict one L3 filtered RSRP. As shown in Fig.5, UE will measure for 10 samples to get 2 L3 results and skip the following L1 5 samples. For this case, the observation window and prediction window length are based on number of L1 measurement period.

Fig.5 observation window and prediction window for AI based L3 filtered RSRP

The observation window will be different for initial stage and later stage. At initial stage, new L1 filtered RSRP will be obtained very L1 sample.

- Sub-case 3: RSRP is predicted at L3

For sub-case 3, RSRP is also predicted at L3. Then observation window and prediction window are similar as sub-case 2.

Observation 8: Observation window and prediction length is dependent on different sub-cases.

Impact of pattern

RAN2 also defines different patterns for prediction window and observation window as below, it will also have impact on the prediction window length and observation window length

Fig.6 Example 1 of intra-frequency temporal domain case B

Fig.7 Example 2 of intra-frequency temporal domain case B

Observation 9: Observation window and prediction length is dependent on different patterns.

Proposal 7: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

      - 2.4 Inter-frequency inter-cell prediction

- 2.4.1 Measurement period reduction

In TR[1], RAN2 agrees that:

For both Intra-frequency prediction inter-cell prediction and FR1 to FR1 inter-frequency inter-cell prediction, no measurement is reduced in both temporal and spatial domain for cell to be measured. For FR1 to FR1 inter-frequency inter-cell prediction, focus on the case where cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighboring site. If inter-frequency correlation model is assumed, section 7.6.5 in [4] is taken as baseline for inter-frequency correlation model. FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub-use cases.

For inter-frequency inter-cell RSRP prediction, UE may skip the measurement for another cell on other frequency. Then it’s possible to reduce CSSF.

Observation 10: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 8: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- 2.4.2 Inter-frequency prediction scenario

Issue 2-1-8: Inter-frequency prediction scenario Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

For the factors that will impact the prediction accuracy for inter-frequency prediction, it can be categories into two parts:

- Common factors for both intra-frequency and inter-frequency predictions

- Factors specific to inter-frequency predictions

Here, we mainly focus on factors specific to inter-frequency predictions.

            - EPRE Impact

EPRE will have impact on inter-cell RSRP prediction, it’s not inter-frequency specific. Since for the current scenarios, UE will only predict RSRP for the same cell for intra-frequency. While for inter-frequency, UE will predict RSRP for another inter-frequency cell. Therefore, we also list the EPRE impact as specific factors for inter-frequency prediction,

- EPRE Impact Under Static Conditions:

  - If Cell 1 and Cell 2 maintain stable EPRE configurations, the UE’s AI/ML model can implicitly learn the fixed RSRP bias (Δ_EPRE = EPRE_Cell2 − EPRE_Cell1) from training data.

  - Example: A constant Δ_EPRE of +3 dB is absorbed into the model’s baseline prediction.

- Dynamic EPRE Adjustment Challenges:

  - When EPRE changes dynamically (e.g., due to load balancing or interference mitigation), the implicit model fails to adapt, causing prediction accuracy degradation:

    - Training data becomes obsolete if new Δ_EPRE values exceed historical ranges.

    - Uncalibrated RSRP measurements violate TS 38.133 §9.1.6 requirements (max 3 dB error).

Observation 11: EPRE may not have impact on prediction accuracy if EPRE is relative static.

            - Channel correlation coefficient between measured and predicted frequency layers

According to 38.901 clause 7.6.5, the factor which may have impact on inter-frequency prediction are as below:

| Step | Issue | Impact on Multi-Frequency Modeling |
| --- | --- | --- |
| Step 1 | Shared parameters except antenna/array/center freq/bandwidth | • Antenna pattern differences cause spatial response mismatch • Bandwidth variation affects PSD |
| Step 2 | Uniform propagation conditions (soft LOS may differ by frequency) | • FR2 typically has lower LOS probability than FR1 |
| Step 4 | LSPs same but DS/AS allow frequency scaling | • DS(f)= 10^(μlgDS(f) + σlgDS(f)x) |
| Steps 5-7 | Identical cluster delays/angles for all frequencies | • Conflicts with Clause 7.5 (noted in standard) |
| Step 6 | Independent per-cluster shadowing, frequency-dependent cluster powers | • Allows realistic power variation but no explicit pathloss linkage |
| Steps 8-11 | Small-scale parameters generated independently per frequency | • Breaks physically existing phase correlations |
| Blockage | Shared blocker positions across frequencies | • Underestimates FR2 blockage probability (wavelength-sensitive) |

Observation 12: The channel correlation coefficient is significantly influenced by factors such as antenna pattern differences, bandwidth variations, LOS probability variations, frequency scaling of large-scale parameters, cluster delays and angles, independent shadowing, and blockage.

      - Cluster approach

Cluster approach will have impact on both intra-frequency and inter-frequency prediction. But the impact may be different.

- For intra-frequency prediction: the focus is on the similarity of multiple cells in the time domain dimension. By analyzing the variation patterns of historical RSRP data in the time series, the commonalities of the signal strength fluctuations of cells over time are utilized for prediction.

- For inter-frequency prediction, more emphasis is placed on the similarity in the frequency domain dimension. The characteristic differences of signals at different frequencies, such as propagation loss and channel characteristics, need to be considered. Through clustering analysis of the signal features of different cells in the frequency domain, the prediction accuracy is improved.

Intra-frequency cluster Focuses on more about temporal relation while Inter-frequency cluster focus more about frequency domain relation. How to choose multiple cells in the same cluster will have impact on the final prediction accuracy.

Observation 13: For cluster based approach, intra-frequency cluster Focuses on more about temporal relation while Inter-frequency cluster focus more about frequency domain relation. How to choose multiple cells in the same cluster will have impact on the final prediction accuracy.

      - Measurement Periodicity & GAP Configuration

Extended measurement gaps, such as the 80 ms intervals in FR1, slow down data collection. As a result, the input signals become outdated, failing to reflect the rapid changes in the channel—a crucial factor for accurate predictions.

Observation 14: Extended measurement intervals degrade inter - frequency prediction performance by causing data obsolescence, which fails to capture rapid channel changes and results in inaccurate RSRP predictions.

      - 2.5 Impacts of measurement error on prediction accuracy

In RAN4114bis, the agreement is as below:

Issue 2-1-7: Impacts of measurement error on prediction accuracy WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

we suggest that the RF channel model follows a Gaussian distribution, which is independent of SNR. However, for baseband processing, it is necessary to consider Gaussian distributions under different SNR conditions—specifically, the variance of the Gaussian distribution varies with SNR. Since we are dealing with a fading channel, this means that the SNR received by the user differs at different times. Therefore, it may be necessary to establish Gaussian-distributed random variables for different time instances and generate Gaussian variables that conform to such random distributions.

Another point to mention is that, due to mobility, there will be "neighbor cells." How to model these neighbor cells in our overall channel modeling will affect the SNR, thereby influencing measurement errors. This, in our view, is an aspect that distinguishes it from beam management in beamforming.

Proposal 9: RAN4 to study the factor and impact of measurement error on RRM prediction.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: Don’t need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

Observation 1: When timing mismatch increases to 200ms, RSRP difference increases to 5.5dB.

Observation 2: Prediction interval for sliding and non-sliding widow is different, which will have impact on degerming the reported time instance.

Proposal 2: Timing between predicated L3-RSRP and ground truth L3-RSRP needs to be aligned, how to align timing is highly dependent on reporting scheme designed by RAN2. If timing alignment is infeasible, incorporate additional margins into accuracy thresholds.

Observation 3: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side.

Observation 4: if 3 samples are used to derive L1 filtered RSRP, RSRP delta will be within 3dB for 90% cases when compared with that case that 5 samples are used.

Proposal 3: L1 filtering number needs to be aligned, otherwise, extra margin needs to be considered.

Observation 5: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side.

Observation 6: At the same time T, ideal L3 filtered RSRP difference between non-sliding and sliding mode can be small than 1.8dB for k=4 in 90% cases. If K=0, no L3 filtering is applied and there is no RSRP difference.

Proposal 4: L3 filtering impact can be removed by setting k=0, extra margin needs to be considered.

Proposal 5: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Test Equipment (TE) can employ multiple anti-cheating strategies such as:

- Multi-instance channel scrambling: Randomizing the order of predefined channel sequences.

- Insertion of redundant channels: Introducing randomly generated non-functional channels.

- Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified

Proposal 6: Observation window and prediction window defined in RAN2 will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

interval to disrupt UE's pattern recognition.

Observation 7: Observation window and prediction length is dependent on different L1/L3 filtering method.

Observation 8: Observation window and prediction length is dependent on different sub-cases.

Observation 9: Observation window and prediction length is dependent on different patterns.

Proposal 7: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

Observation 10: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 8: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

Observation 11: EPRE may not have impact on prediction accuracy if EPRE is relative static.

Observation 12: The channel correlation coefficient is significantly influenced by factors such as antenna pattern differences, bandwidth variations, LOS probability variations, frequency scaling of large-scale parameters, cluster delays and angles, independent shadowing, and blockage.

Observation 13: For cluster based approach, intra-frequency cluster Focuses on more about temporal relation while Inter-frequency cluster focus more about frequency domain relation. How to choose multiple cells in the same cluster will have impact on the final prediction accuracy.

Observation 14: Extended measurement intervals degrade inter - frequency prediction performance by causing data obsolescence, which fails to capture rapid channel changes and results in inaccurate RSRP predictions.

Proposal 9: RAN4 to study the factor and impact of measurement error on RRM prediction.

- Reference

[1] RP-242393 Revised SID: Study on Artificial Intelligence (AI)/ Machine Learning (ML) for mobility in NR, OPPO

[2] R2-2406309, text proposal on TR 38.744 v0.03

[3] R2-2407849, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions.