3GPP TSG-RAN WG4 Meeting #116				                                         R4-2509801

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.3.4

Title: 	Discussion on LB-CA RRM requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP#105, a new WI was agreed to enable LB-LB CA with swicthing in [1]. And the detailed WID is self-contained here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In the last RAN4 meeting, the discussion on RRM impacts due to LBCA operation was initiated and some agreements and WF was achieved [2]. In this contribution, we provided further analyze on the open issues below.

- RRM impacts:

  - Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC

  - SDL SCell related requirement.

- Discussion

  - Interruption requirement for carrier switching between FDD CC and SDL CC

In the last meeting, the following agreements/WF were achieved.

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered [Wayforward]: FFS: options in topic summary R4-2505556.

According to RF session agreements below, in our view the scope of LB-CA switching is limited to two carriers (one is FDD, the other SDL). That is when the switching between these two carriers, the impacts on the other carrier can be ignored.

Figure 1. LB-CA scenarios

Proposal 1: RAN4 does not define interruption requirements to other bands/carriers in Rel-19.

  - SDL SCell related requirement

    - Activated SDL SCell L3 measurement

In the last meeting there are extensive online discussion on whether RAN4 needs to define the requirements for the activated SDL SCell L3 measurement but without consensus.

Activated SDL SCell L3 measurement Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement Online Agreement on Tuesday: - RAN4 to define the requirement for activated SDL SCell L3 measurement. Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

In our view, the requirements can be based on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. The most fundamental factor to define the requirements is the SSB occasion to be measured. But some specific issues shall be considered.

According to the current RAN4 requirements below, in order to achieve the reliable measurement results, usually the more than one SSBs samples need to be measurement.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; TSSB_measurement_period_intra. No DRX; max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

However, for the activated SDL measurements it shall be occurred within the switching pattern inside. It is possible that one successful measurement reporting can be distributed over the multiple SDL switching periodicities as illustrated in Figure 2.

Figure 2. Deactivated SDL SCell measurement procedure

Observation 1: If there are not sufficient measurement samples available within a single duration when UE switching to SDL (e.g. there are 1 SSB sample but 5samples at least required), the total measurement delay need count the additional time because SSBs in SDL are not available.

Therefore, we can propose that:

Proposal 2：The measurement reporting delay requirement for the activated SDL in LBCA can be defined as “Tssb + Tssb_not”. Where in

- Tssb is [5]SSB samples which are within the SDL switching pattern.

- Tssb_not is the time in which SDL SSBs are not available (e.g. NW switching to FDD).

Additionally, if there is limited SSB samples which are overlapped within the switching pattern, the upper bound of the measurement delay shall be defined in order to avoid the meanness measurement happened. Thus, in order to simplify these requirements, we can also the upper bound of the measurement delay shall be defined to avoid the meanness measurement happened.

Proposal 3: In order to avoid too long waiting time of UE switching back to SDL in a measurement reporting period, the upper bound of activated SDL SCell measurement delay can be specified. And if the waiting time is out of the upper bound, UE can discard the previous measurement samples and restart a new measurement procedure.

    - Deactivated SDL SCell L3 measurement

In the last meeting, the requirements on the interruption because of the deactivated SDL SCell measurement was agreed as:

Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

For the open issues on the detailed interruption allowed, if the total interruption length allowed are fixed, the allowed ratio shall be differentiated depending the measurement cycle. Thus, we can propose that:

Proposal 4：For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. X can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

- Conclusion

In this contribution, serval issues related to RRM in Rel19 LB-CA are discussed. The observations and proposals can be summarized as:

Interruption requirements

Proposal 1: RAN4 does not define interruption requirements to other bands/carriers in Rel-19.

SDL SCell related requirements

Observation 1: If there are not sufficient measurement samples available within a single duration when UE switching to SDL (e.g. there are 1 SSB sample but 5samples at least required), the total measurement delay need count the additional time because SSBs in SDL are not available.

Proposal 2：The measurement reporting delay requirement for the activated SDL in LBCA can be defined as “Tssb + Tssb_not”. Where in

- Tssb is [5]SSB samples which are within the SDL switching pattern.

- Tssb_not is the time in which SDL SSBs are not available (e.g. NW switching to FDD).

Proposal 3: In order to avoid too long waiting time of UE switching back to SDL in a measurement reporting period, the upper bound of activated SDL SCell measurement delay can be specified. And if the waiting time is out of the upper bound, UE can discard the previous measurement samples and restart a new measurement procedure.

Proposal 4：For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. X can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

- References

- RP-240830

- R4-2508259

- R4-2502588

- TS38.133 v19.1.0

- TS38.331 v18.1.0