3GPP TSG-RAN WG4 Meeting # 116		R4-2510039

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.28.5

Source: 	CMCC

Title: 			 Discussion on test cases for IOT-NTN Phase 3

Document for:	Discussion

- Introduction

In last meeting, the discussion of performance part was started, and the WF has been agreed at [1]. In this contribution, we will provide our views on the test list and test configuration for NB-IOT over NTN.

- Discussion

Issue 2-1: Test cases for CB-Msg3 - General

Agreement:

- Define test cases for random access for CB-msg3-EDT, and the correct behaviour and transmit timing can be verified in the same test case.

- Further discuss whether to down-select the test cases:

| Random Access (A.13.3.2) | A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT |
| --- | --- |
|  | A.13.3.2.X2 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage for CB-msg3-EDT |
|  | A.13.3.2.X3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage for CB-msg3-EDT |

- We support all three tests above since both normal coverage and enhanced coverage, both anchor carrier and non-anchor CB-msg3 EDT procedure are supported by RAN2.

Proposal 1: Define CB-msg3 EDT test cases for all there cases above.

- For the specific test setup, the CB-Msg3 EDT configuration as defined in CB-Msg3-ConfigSIB-NB-r19 shall be added, the configuration divided into cell level configuration and CE level configuration. For CE level configuration, the maximum 2 CE level shall be set up.

- Cell level configuration including:

- cb-Msg3-MinRSRP-Threshold-NB-r19

- cb-Msg3-RSRP-CE-Level-NB-r19: 2 RSRP shall be configured corresponding to 2 CE level

- CE level configuration including:

- cb-Msg3-TBS-NB-r19

- cb-Msg3-NumOfReplicas-NB

- cb-Msg3-TimeResource-NB-r19

- cb-Msg3-PhysicalConfig

- cb-Msg3-TxWindow-NB-r19

- cb-Msg3-ResponseWindow-NB

- cb-Msg3-MaxAttemptNum-NB

Proposal 2: Set up 2 CE level in the test, add CB-Msg3 EDT configuration as defined in CB-Msg3-ConfigSIB-NB-r19

- During the test, in order to trigger the CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN defined in TS 36.331 5.3.3.1x shall be met first.

Proposal 3: In order to trigger CB-Msg3, Conditions for initiating CB-Msg3 EDT in NTN shall be met at first as defined in TS 36.331 5.3.3.1x.

- Correct UE behavior of replica transmission and re-attempt a CB-Msg3 transmission shall be tested. For this purpose, the System Simulator shall transmit a CB-Msg4 over CB-RNTI after 2 CB-Msg3 transmission attempts (the replicas may be transmitted in each attempt) have been received by the System Simulator. In response to the first CB-Msg3 transmission attempts, the System Simulator shall transmit a CB-Msg4 not corresponding to the transmitted CB-RNTI.

Proposal 4: In order to test correct UE behavior of replica transmission and re-attempt a CB-Msg3 transmission, the System Simulator shall transmit a CB-Msg4 over CB-RNTI after 2 CB-Msg3 transmission attempts

- Regarding how to test the correct behavior of CB-Msg3-EDT failure, it can be discussed after finishing the core requirement.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Define CB-msg3 EDT test cases for all there cases above.

Proposal 2: Set up 2 CE level in the test, add CB-Msg3 EDT configuration as defined in CB-Msg3-ConfigSIB-NB-r19

Proposal 3: In order to trigger CB-Msg3, Conditions for initiating CB-Msg3 EDT in NTN shall be met at first as defined in TS 36.331 5.3.3.1x.

Proposal 4: In order to test correct UE behavior of replica transmission and re-attempt a CB-Msg3 transmission, the System Simulator shall transmit a CB-Msg4 over CB-RNTI after 2 CB-Msg3 transmission attempts

- Reference

[1] R4-2508289, WF on RRM requirements for IoT_NTN_Ph3, MediaTek