3GPP TSG-RAN WG4 Meeting #116	R4-2510189

Bengaluru, India, 25 August – 29 August, 2025

Source: 	vivo

Title: 	Discussion on remaining RRM impacts for SBFD operation

Agenda Item:			7.21.4.2

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD.

- Discussion

At previous meeting, based on the WF [2] and topic summary [3], the following agreements are available:

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Agreement

  - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex.

  - Further discuss the upper bound of the measurement period due to processing time and its applicability.

Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

- Agreements

  - For L1-RSRP and L1-SINR

    - Measurement period is extended by using R16 NR-U approach.

  - For RLM/BFD/CBD

    - Confirm the following for defining the requirements

      - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type.

      - Existing measurement period apply.

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

- Agreements

  - For DUD case

    - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS.

- Agreements (Monday online session)

  - For collision between CSI-RS resource and dynamic UL transmission

    - existing scheduling restriction apply, i.e. CSI-RS based L3 measurement is prioritized

Issue 2-1-3: Requirements for scheduling restriction

- Agreements

  - Do not define new scheduling restriction requirements for transition between UL and DL.

  - RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point, i.e. no new restriction is defined.

The following issues are still open for discussion:

Issue 2-1-4: Requirements for RACH requirements

- Proposals

  - Proposal 1 (Nokia, LGE):

    - The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2.

- Recommended WF

  - Discuss whether the proposed clarification in P1 is needed in 38133.

To our understanding the appropriate UE behaviour is clearly captured by other specs and it is not necessary to have further clarification on this issue in RAN4 specs.

Proposal 1: It is not necessary to have further clarification on this issue in RAN4 specs.

Issue 2-1-5 (new): Lmax when using NR-U approach for L1 CSI-RS measurement

For issue 2-1-5, an example for maximum value defined in NR-U when measurement period is extended is illustrated below. We understand the maximum value is introduced with the intention that the measurement can be restarted when too many measurement occasions are missed.

Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR1

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(10 ms, P * TSSB) if no DRX is used, Once per Max(10 ms, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320 ms, Once per P * TDRX if DRX cycle > 320 ms. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

For the SBFD scenario, the rationale to introduce the maximum value for NR-U may not exist since all reference resources are NW configuration. For this reason, the Lmax when using NR-U approach for L1 CSI-RS measurement can either be introduced or not.

Proposal 2: The Lmax when using NR-U approach for L1 CSI-RS measurement can either be introduced or not.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: It is not necessary to have further clarification on this issue in RAN4 specs.

Proposal 2: The Lmax when using NR-U approach for L1 CSI-RS measurement can either be introduced or not.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4 115

[3] R4-2505568,	Topic summary for [115][219] NR_duplex_evo, Huawei, RAN4 115