3GPP TSG-RAN4 Meeting #116 R4-2510296 Bangalore, India, 25 Aug - 29 Aug, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.101-1; CR; draft; rev; -; Current version:; 19.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; Draft CR to 38.101-1 on LP-WUS REFSENS ACS and ASCS. Source to WG:; vivo. Source to TSG:; R4. Work item code:; NR_LPWUS-Core; Date:; 2025-08-05. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This CR provides content to accommodate LP-WUS/WUR RF requirements. This draft CR use the endorsed content in R4-2506882 as a basis.. Summary of change:; The following changes: - REFSENS - ACS - ASCS. Consequences if not approved:; The spec is not appicable for LP-WUS feature.. Clauses affected:; 7.1M, 7.3M, 7.5M. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

## <<< START OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## 7.1M	General for LP-WUR

The minimum requirements are specified assuming only one receiver for the wake-up signal. The criterion for verifying all receiver core RF requirements shall be MDR of the LP-WUS, which shall not exceed 1% with the LP-WUS parameters given in Annex A.3M. Whether type-1 or type-2 requirement applies is based on UE declaration, only one type a UE can support.

<<< Skip Unchanged Sections >>>

## 7.3M	Reference sensitivity for LP-WUR

7.3M.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna port, at which the MDR criterion with the side conditions specified in 7.1M shall be met.

7.3M.2	Reference sensitivity power level for LP-WUR

The REFSENS for the two types of LR shall be as specified in Table 7.3M.2-1a. The requirement shall apply to all bands for which LP-WUS is specified in clause 5.2M and supported by the UE. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.3M.2-1a: One antenna port reference sensitivity for FDD and TDD bands

| RX parameter | SCS kHz | Value |
| --- | --- | --- |
|  |  | (dBm) |
| REFSENS for UEs supporting [Type1 LR] | 15 | -97.5 |
|  | 30 | -94.5 |
| REFSENS for UEs supporting [Type2 LR] | 15 | -102.0 |
|  | 30 | -99 |

<<< Skip Unchanged Sections >>>

7.5M	Adjacent channel and subcarrier selectivity for LP-WUR

7.5M.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the configured channel frequency to the receive filter attenuation on the adjacent channel(s).

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR.

7.5M.2	Adjacent channel selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.2-1 and Table 7.5M.2-2 for LP-WUR type 1 UE and the minimum requirements specified in Table 7.5M.2-3 and Table 7.5M.2-4 for LP-WUR type 2 UE. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.5M.2-1: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + 11 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 39.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-2: Test parameters for LP-WUR type 1 UE, case 2

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | -53.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.5M.2-3: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + 12.5 dB |
| Pinterferer | dBm | REFSENSLP-WUS + 44 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-4: Test parameters for LP-WUR type 2 UE, case 2

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS bandwidth | dBm | -56.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

7.5M.3	Adjacent subcarrier selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 and 7.5M.3-2 for LP-WUR. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.5M.3-1: ASCS parameters for LP-WUR type 1 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS+0.5 |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 1 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration as interferer, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. 0 power boost means equal PSD between NR RBs and LP-WUS RBs. |  |  |

Table 7.5M.3-1: ASCS parameters for LP-WUR type 2 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 0 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration as interferer, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. 0 power boost means equal PSD between NR RBs and LP-WUS RBs. |  |  |

## <<< END OF CHANGES >>>