3GPP TSG-RAN WG4 Meeting #116	R4-2510303

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	TP for TR 38.774 on LP-WUS

Agenda item:	7.24.2

Document for:	Discussion

Introduction

In this text proposal for TR 38.774, we provide our inputs on section 7.1. These inputs are based on the agreements achieved in the previous meetings.

- References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, China, 7th – 11th April, 2025.

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo, RAN4#113, Orlando, US, 18th – 22nd Nov, 2024.

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei, China, 14th – 18th Oct, 2024.

- R4-2414309, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112, Maastricht, Netherlands, 19th – 23rd August, 2024.

****************************************** Start of TP ******************************************

### 7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR.

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively. Further, there are two set of values for NFLR + IMLR, i.e. 18.0dB and 13.5dB for Type 1 and Type 2 UEs, respectively. There is only single receiver so there is no diversity gain included in the REFSENS.

In the REFSENS test, only the RBs allocated for LP-WUS are populated with the LP-WUS and the rest of the RBs for a given channel bandwidth are left empty. This configuration, however, does not correspond to the real-life case where NR signals in other RBs are generally present and thus can create a coverage shortfall. Separately, the ASCS requirement allows a maximum degradation of 0.5 dB and the side conditions incorporate populated adjacent RBs.

Given the small value of the ASCS degradation factor (0.5 dB), it was agreed not to include this factor in the REFSENS.

For FR1, a SNR value of -4.5 dB for both Type 1 and Type 2 UEs was selected by RAN4 to be used for the REFSENS equation.

Regarding bands with FDL_low ≥ 3300 MHz, the REFSENS is further relaxed by [0.5] dB based on the fact that the difference in the worst REFSENS for bands with FDL_high < 2700 MHz and FDL_low ≥ 3300 MHz is 0.5 dB.

****************************************** End of TP ******************************************