# 3GPP TSG-RAN WG4 Meeting #116	R4-2510464

# Bengaluru, India, 25th – 29th August, 2025

Title: 	PC2 A-MPR requirements for 7MHz channel bandwidth

Agenda item:	7.4.3

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In the RAN4 114-bis meeting the WF was agreed, where the following configurations were considered for A-MPR evaluations:

- Evaluate A-MPR for: - NS_12 considering 7 MHz deployed within 814-824 MHz - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz). - NS_14 considering 7 MHz deployed within 824-849 MHz - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

In this document we are presenting our PC2 A-MPR evaluations using these configurations

# Discussion

The PA was calibrated based on the following assumptions:

- PC2

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

- ASE, SEM and ACLR are considered as gating factors (EVM was excluded).

In our plots the back off values below MPR level are in gray, as no A-MPR needed for them.

## NS_12

The following requirements should be considered by the UE when BS issues NS_12:

As an example, the A-MPR evaluations for QPSK with 7 MHz at 817.5 MHz (closest to the band edge) is shown in Figure 1, the remaining evaluations can be found in the Annex.

Figure 1: NS_12 PC2 power back off for7MHz CBW with QPSK and Fc= 817.5 MHz

Based on our evaluation we propose the following A-MPR regions and values.

Proposal 1: Consider the following PC2 A-MPR regions and values for NS_12 with 7MHz CBW.

Table 1: A-MPR regions for NS_12 for 7MHz CBW (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.34 | >0 | A3 |

Table 2: A-MPR for NS_12 (Power Class 2)

| Modulation/Waveform | A3 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

## NS_13

The following requirements should be considered by the UE when BS issues NS_13:

As an example, the PC2 A-MPR evaluations for QPSK with 7 MHz at 820.5 MHz is shown in Figure 2, the remaining evaluations can be found in the Annex.

Figure 2 NS_13 PC2 power back off for7MHz CBW with QPSK and Fc= 820.5 MHz

Proposal 2: Consider the following PC2 A-MPR regions and values for NS_13 with 7MHz CBW.

Table 3: PC2 A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 820.5 MHz | ≤1.98 | >0 | A4 |

Table 4: PC2 A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
| DFT-s-OFDM QPSK | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

## NS_14

The following requirements should be considered by the UE when BS issues NS_14:

Our evaluations, in the Annex, show that no A-MPR is needed due to ASE.

Proposal 3: No A-MPR for NS_14 with 7MHz CBW.

## NS_15

The following requirements should be considered by the UE when BS issues NS_15:

As an example, the PC2 A-MPR evaluations for QPSK with 7 MHz at 845.5 MHz is shown in Figure 3, the remaining evaluations can be found in the Annex.

Figure 3 NS_15 PC2  power back off for7MHz CBW with QPSK and Fc= 845.5 MHz

Proposal 4: Consider the following A-MPR regions and values for NS_15 with 7MHz CBW.

Table 6: A-MPR regions for NS_15 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 845 < Fc ≤ 845.5 | ≥4.86 | ≥2.7 (Zone1) | A1 |
|  |  | ≥2.7, ≤4.86 | ≥1.44(Zone2) | A2 |
|  |  | ≥4.86 | <2.7(Zone3) | A3 |
|  |  | ≤1.26 | ≤0.36(Zone4) | A4 |
|  | 838 < Fc ≤ 841.5 | ≥4.86 | ≥2.7 (Zone1) | A5 |

Table 7: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 14.5 | ≤11 | ≤ 9 | ≤ 11 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 14.5 | ≤ 11 | ≤ 9 | ≤ 11 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 14.5 | ≤ 11 | ≤ 9 | ≤ 11 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 14.5 | ≤ 11 | ≤ 9 | ≤ 11 | ≤ 6.5 |

Table 7 is a modified version of Table 6.2.3.24-4 in TS 38.101-1

# Conclusion

Proposal 1: Consider the following PC2 A-MPR regions and values for NS_12 with 7MHz CBW.

Table 1: A-MPR regions for NS_12 for 7MHz CBW (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.34 | >0 | A3 |

Table 2: A-MPR for NS_12 (Power Class 2)

| Modulation/Waveform | A3 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

Proposal 2: Consider the following PC2 A-MPR regions and values for NS_13 with 7MHz CBW.

Table 3: PC2 A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 820.5 MHz | ≤1.98 | >0 | A4 |

Table 4: PC2 A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
| DFT-s-OFDM QPSK | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

Proposal 3: No A-MPR for NS_14 with 7MHz CBW.

Proposal 4: Consider the following A-MPR regions and values for NS_15 with 7MHz CBW.

Table 6: A-MPR regions for NS_15 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 845 < Fc ≤ 845.5 | ≥4.86 | ≥2.7 (Zone1) | A1 |
|  |  | ≥2.7, ≤4.86 | ≥1.44(Zone2) | A2 |
|  |  | ≥4.86 | <2.7(Zone3) | A3 |
|  |  | ≤1.26 | ≤0.36(Zone4) | A4 |
|  | 838 < Fc ≤ 841.5 | ≥4.86 | ≥2.7 (Zone1) | A5 |

Table 7: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 14.5 | ≤11 | ≤ 9 | ≤ 11 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 14.5 | ≤ 11 | ≤ 9 | ≤ 11 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 14.5 | ≤ 11 | ≤ 9 | ≤ 11 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 14.5 | ≤ 11 | ≤ 9 | ≤ 11 | ≤ 6.5 |

Table 7 is a modified version of Table 6.2.3.24-4 in TS 38.101-1

# Annex