3GPP TSG-RAN4 Meeting #106	R4-2510526

Bengaluru, India, 25 Feb – 29 Feb, 2025

Title:							Discussion on beam correspondence for RRC_IDLE and RRC_INACTIVE

Source:			Huawei, HiSilicon

Agenda Item:			5.28.1

Document for:			Discussion

# 1.	Background

A LS R4-2509022 from RAN5 requests clarification from RAN4 on the usage of beam lock function for testing of beam correspondence for RRC_IDLE and RRC_INACTIVE. This contribution provides our review on this issue.

RAN5 has discussed the test procedure and feasibility of 6.6.4 Beam Correspondence in RRC_INACTIVE requirements. RAN5 seeks the clarification from TS38.101-2 sub clause 6.6.4 Beam correspondence for power class 3 to support test procedure with UE Beamlock test function in the IDLE mode and without UE Beamlock test function in the IDLE mode.

As stated in 6.6.4.1, UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4. RAN5 seeks the following clarifications:

- RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either  procedure be used to verify RAN4 core requirements?

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

  - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

# 2	Discussion

# 2.1	RAN4 discussion in Rel-18

In Rel-18, the UE’s behaviour for PRACH preamble transmission in IDLE or INACTIVE was intensively discussed. It was identified that several issues will cause the transmit power fluctuation in conformance testing procedure.

- To achieve required accumulate measurement period, RAR will not be transmit to the UE.

- The UE might change beam type (rough beam vs fine beam) across multiple PRACH preamble transmission

- The UE might choose sub-optimal Tx beam at the beginning as long as the DL RSRP meets RRC configured threshold

- The UE might switch to different Tx beams when RAR is not received for a long time.

It should be noted that the power fluctuation issue only happens in conformant testing environment due to RAR holding. In real network deployment, the UE will be able to access the network when RAR is received.

To derive the core requirement for PRACH spherical coverage, the transmit power fluctuation needs to be considered and two options could apply:

- Option 1: Introduce additional tolerance related to the power fluctuation

- Option 2: Define beam lock function to avoid additional tolerance

RAN4 sent LS R4-2220825 to RAN5 to check the feasibility of defining a beam lock function and got positive feedback in reply LS R4-2304021. Following that RAN4 discussed the core requirement assuming the beam lock function would be in place. After the requirements are stable, RAN4 sent another LS R4-2317764 informing RAN5 the need to develop the beam lock function.

Observation 1: Beam lock function is important to maintain existing core requirement of PRACH spherical coverage.

Observation 2: RAN4 has informed RAN5 to develop the beam lock function.

# 2.2	Discussion in RAN5

When RAN5 starts working on the test procedure of PRACH spherical coverage, the procedure with beam lock function was suggested as below.

…

4)	SS activates the UE Beamlock_IDLE Function (UBF_IDLE) by performing the procedure as specified in TS 38.509 [10] clause 5.4a.2.

5)	SS transmits an RRCRelease message.

6)	SS transmits a Paging message. The UE’s Random access procedure is initiated.

7)	Through its beam correspondence procedure, DUT refines its TX beam by autonomously choosing the corresponding TX beam for PRACH preamble transmission using downlink reference signals to transmit in the direction of the incoming DL signal. DUT doesn’t transmit PRACH preamble in this step.

8)	The DUT locks the TX beam and starts transmiting the PRACH preambles.

9)	SS detects UE’s transmit power on the PRACH occasions. If on PRACH occasion n the transmit power exceeds the Pmin specified in 6.3.1.5, SS measures the mean power Pmeas (PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator) starting from PRACH occasion n until accumulated measurement period achieves 1ms.

…

According to step 7 and step 8, the Tx beam measured is selected by the UE completely based on the DL SSB measurement without any assistance information. The test procedures intentionally verify the UE’s beam correspondence ability. If the UE could pass the test case, it could be expected the UE is able to access to the network precisely and quickly.

Observation 3: The test procedure with beam lock function could fully verify the UE’s beam correspondence ability.

Unfortunately, the procedure with beam lock function was repeatedly blocked in RAN5. Another test procedure without beam lock function was proposed. In last RAN5 meeting, the procedure without beam lock function was suggested as below:

…

5.	SS transmits a Paging message (including PagingRecord with ue-Identity) for the UE to trigger PRACH

6.	The UE starts sending PRACH to the SS. In response, SS shall transmit RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

7.	SS shall measure simultaneously the PRACH transmission power of both measurement polarization.

8.	Among the collected PRACH transmission from the UE, SS picks the set of X (X = 24 for PRACH format C2) consecutive PRACH providing the highest average PRACH power.

9. SS determines the EIRP for PolLink = theta by adding up the EIRP of the 2 measurement polarization.

…

In the proposed test procedures, SS always picks the best transmit power a UE could achieve as the measurement result. If we consider a UE with bad beam correspondence capability, the correct Tx beam selection can’t be assured barely based on DL SSB measurement. The randomly switch across several candidate beams might happen to help the random access. Using the produce without beam lock function, the UE will always pass the test case since TE help it to pick the best transmission samples.

Observation 4: The test procedure without beam lock function will pass the UEs with bad beam correspondence capability.

There was another proposal to define both kind of test procedures in RAN5 and UE could freely pick either one in conformance testing. Comparing the test procedures with/without beam lock we could see that the test result will be different for the same UE. This hurts the important values of conformance testing, i.e. stable, comparable, consistency.

Observation 5: Having both test procedures in RAN5 will cause different testing result for the same UE, which damages the value of conformance testing.

# 2.3	Proposal of LS reply

Based on above analysis, we propose to clearly reply RAN5 that the beam lock function is necessary from RAN4 point of view.

1. Overall Description:

RAN4 thanks RAN5 for the LS on testing procedure and feasibility of 6.6.4 Beam Correspondence in RRC_IDLE and RRC_INACTIVE requirements. Please find the answers from RAN4 below:

Question 1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer: RAN4 derived the core requirements based on the assumption of using beamlock test function, which is capture in TR 38.891 and the LS R4-2317764 sent to RAN5. RAN4 confirms the beam correspondence in RRC_IDLE and RRC_INACTIVE shall be tested with beamlock test function.

Question 2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer: The target of having a beamlock test function is to keep the beam unchanged during the measurement period. There is no intention of preventing UE from autonomously choosing Uplink beams. Therefore, the beamlock test function should be activated after the UE have done the Uplink beam refinement autonomously.

Question 3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer: RAN4 derived the core requirements based on the assumption of using beamlock test function. Missing of the beamlock test function is not aligned with the RAN4 requirements. If beamlock test function is not feasible, RAN4 is obliged to revisit the core requirements and introduce additional tolerance.

2. Actions:

To RAN5 group:

ACTION:

RAN4 respectfully requests RAN5 to take above information into consideration in conformance testing works.

# 3.	Conclusion

In summary, following observations and proposal are for RAN5 discussion:

Observation 1: Beam lock function is important to maintain existing core requirement of PRACH spherical coverage.

Observation 2: RAN4 has informed RAN5 to develop the beam lock function.

Observation 3: The test procedure with beam lock function could fully verify the UE’s beam correspondence ability.

Observation 4: The test procedure without beam lock function will pass the UEs with bad beam correspondence capability.

Observation 5: Having both test procedures in RAN5 will cause different testing result for the same UE, which damages the value of conformance testing.

Proposal 1: RAN4 approves the LS contents in section 2.3.

# 4.	Reference

[1]	R4-2220825, LS on testability for beam correspondence in initial access, RAN4, RAN4#105

[2]	R4-2304021, Response LS on testability for beam correspondence in initial access, RAN5, RAN4#106bis

[3] R4-2317764, LS on testability issues of beam correspondence requirement for initial access, RAN4, RAN4#108bis

[4] R4-2509022, LS on Beam Correspondence Initial Access, RAN5, RAN4#116