3GPP TSG-RAN WG4 Meeting # 116	R4-2510581

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.3.4

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides further analysis on low band carrier aggregation via switching from RRM perspective.

Discussion

General Aspects

Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching [Way forward]: FFS: options in topic summary R4-2505556.

The key point of the issue is whether the L3 measurement delay shall be scaled with a factor (called Kp-LB herein) when a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} are not available based on the LB-CA switching pattern. As we know, the lower bound of L3 measurement periodicity is 200ms and RAN1 agreed that the periodicity of switching pattern is 40ms.

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Figure 1 provides a worst case where the available SMTC on SDL is the least (where the available SMTC for SDL is not colliding with gap, otherwise the measurement will be performed within gap). We try to find whether Kp-LB (scaling factor in accordance to the fraction of available occasions) is needed or not to be multiplied by the legacy L3 measurement period in this case.

- if SDL SMTC is 5ms, seven-eighths ( 78) of the SMTC locations are colliding with FDD pattern. So Kp=1, Kp-LB=8, then, max(200 ms, ceil( 5 x Kp) x Kp-LB x SMTC period)=max(200 ms, 200ms)=200ms

- if SDL SMTC is 10ms, three-forths ( 34) of the SMTC locations are colliding with FDD pattern. So Kp=1, Kp-LB=4, then, max(200 ms, ceil( 5 x Kp) x Kp-LB x SMTC period)=max(200 ms, 200ms)=200ms

- if SDL SMTC is 20ms, half SMTC locations are colliding with FDD pattern. So Kp=1, Kp-LB=2, then: max(200 ms, ceil( 5 x Kp) x Kp-LB x SMTC period)=max(200 ms, 200ms)=200ms

- if SDL SMTC is 40ms, to ensure measurement on SDL carrier, the SMTC shall be allocated in SDL pattern, and no colliding with switching pattern happens. Kp=1, Kp-LB=1, then: max(200 ms, ceil( 5 x Kp) x Kp-LB x SMTC period)=max(200 ms, 200ms)=200ms

Figure 1. LB CA switching pattern where the available SMTC on SDL is the least

Figure 2 takes another example to seek that on top of colliding with switching pattern, whether further colliding with gap could make the L3 measurement delay exceed the lower bound 200ms.

- if SDL SMTC is 10ms, two SMTC locations are colliding with FDD pattern, and 1 SDL SMTC is colliding with gap, So updated Kp’=4 (considering both meas gap and LB-CA switching pattern) then: max(200 ms, ceil( 5 x Kp’) x  x SMTC period)=max(200 ms, 200ms)=200ms

Figure 2.  Another LB CA switching pattern

In summary, for the case that parts of the SDL SMTC are colliding with switching pattern, even when multiplied by scaling factor Kp-LB, the L3 measurement delay will not exceed the low boundary 200ms. For the case SMTC ≥ P(switching pattern periodicity, i.e. 40ms), SMTC would not be collided with switching pattern any more. Therefore Kp-LB =1.

Observation 1: When a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} are not available based on the LB-CA switching pattern, even when multiplied by scaling factor Kp-LB, the L3 measurement delay will not exceed 200ms (low boundary of measurement period).

Observation 2: When SDL SCell SMTC ≥ switching pattern periodicity P (i.e., 40ms), the SMTC would not be impacted by switching pattern.

In legacy, the low bound of measurement period 200ms is based on the assumption that one measurement sample can be acquired every 40ms. The essential reason for not considering the scaling factor Kp-LB is that in low band CA switching gap-less case, there is at least one available SMTC on SDL carrier every 40ms, as the periodicity of switching pattern is 40ms. The case well coincides with the logic of defining the low bound of legacy L3 measurement period.

Proposal 1: For L3 measurement requirement, it is unnecessary to consider a scaling factor Kp-LB, when a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} are not available due to LB-CA switching pattern.

Regarding L1 measurement requirement, the low bound of measurement period is Treport which has multiple candidate values from 4 slots to 320 slots. This current low bound is different as that of L3 measurement period. It is NOT able to ensure UE can acquire at least one available SSB/CSI-RS occasion for L1-RSRP measurement on SDL during Treport.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise, K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

So when a fraction of {SSB occasions or CSI-RS for L1 measurement} are not available due to the LB-CA switching pattern, the L1-RSRP delay will be correspondingly impacted.

As we know both measurement gap and LB-CA switching pattern would disrupt available SSB/CSI-RS occasions. In legacy L1-RSRP measurement requirements, scaling factor P is introduced. In LB-CA via switching, P shall be updated to involve the impact of switching pattern. We propose the following update:

P value for SSB resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable > 0 - For a window W of duration max(TL1, xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs , periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion: Ntotal is the total number of SSB resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or any SDL duration corresponding to switching pattern for L1-RSRP measurement on PCell, or any FDD duration corresponding to switching pattern for L1-RSRP measurement on SDL SCell within the window W, and - Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

Proposal 2: When a fraction of {SSB occasions or CSI-RS for L1 measurement} are not available due to the LB-CA switching pattern, the L1-RSRP delay shall be updated with updating the scaling factor P where fraction of available occasions due to both measurement gap and LB-CA switching pattern are considered.

P value for SSB resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable > 0 - For a window W of duration max(TL1, xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs , periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or any SDL duration corresponding to switching pattern for L1-RSRP measurement on PCell, or any FDD duration corresponding to switching pattern for L1-RSRP measurement on SDL SCell within the window W, and - Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC Online Agreement on Tuesday: - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. - Note: the above agreement can be revisited if not aligned with RAN1 further agreement.

As per the latest RAN1 agreement@ RAN1#121 meeting, RAN1 will specify all kinds of collision handling.

RAN1 Agreement For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

Proposal 3: Confirm the agreement in last meeting: For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

Switching delay or interruption requirement

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered [Wayforward]: FFS: options in topic summary R4-2505556.

RF session agreed that DL switching happens between two low bands, while the band combination can be more than two bands. Under this scenario, the switching between FDD and SDL would cause interruption to other combined active SCell.

Issue 2-1: Switching delay Agreement: - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

In RF, the switching period for low-low band switching is concluded as 35us, 140us and FFS on other additional value(s) [4].

- Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us Additional value(s) are under discussion. If any additional value(s) are confirmed, RAN4 will provide them in a future LS. RAN4 respectfully requests RAN1 to take this information into account in their work.

As we known, the interruption length in RRM considers not only switching period but also the time misalignment between aggressive band and victim band. Therefore the interruption can be TswitchingSlot-length-victim+1.

Of course, we also observed that companies have various understanding of the RF agreement. Some companies think the combination of two low bands with other band will be specified in future release. If no common understanding on RF agreement, it is fine to us not to specify requirements in R19.

Proposal 4: If RAN4 agree to define requirements for more carriers scenario, switching between FDD and SDL would interrupt other combined active SCell, the interruption length is TswitchingSlot-length-victim+1.  If no common understanding on RF agreement, it is fine not to specify requirements in R19.

SDL SCell related requirements

L3 measurement on activated SDL SCell

Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

This issue depends on the conclusion of Issue 1-1-3. As per proposal 1, for the case that parts of the SDL SMTC are colliding with switching pattern, even when multiplied by scaling factor Kp-LB, the measurement delay will not exceed the low boundary 200ms, so it is unnecessary to introduce new scaling factor.

Proposal 5: L3 measurement requirements on activated SDL follow proposal 1.

L3 measurement on deactivated SDL SCell

Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

As we know, the switching pattern specified in RAN1 is only applicable to activated carriers. Our original thinking is that as the measurement on deactivated SDL would result in interruption on PCell, UE can skip the L3 measurement and network can leverage the measurement result of the co-collocated PCC. However if network configures MO on the deactivated SDL carrier, UE will still follow the network configuration to perform L3 measurement on SDL. There is at least one available SSB within measCycleSCell. So the legacy measurement period on deactivated SDL would not be impacted by switching pattern.

As we know the legacy interruption due to measurement on deactivated SCC requires has two cases:

-when measCycleSCell is smaller than 640ms, no interruption is allowed. It means that UE shall always open the SCC RF chain after SCell addition.

-when measCycleSCell is 640 ms or longer, up to 0.5% interruption is allowed. It means UE can close SCC RF chain after SCell addition, and open the RF chain again when measurement on deactivated SCC is performed.

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

In R19 low band switching scenario, UE is not able to simultaneously receive on both FDD DL and SDL. In other words, UE shall swich to deactivated SDL carrier  to measure and switch back to PCC after measurement, even if measCycleSCell is smaller than 640m. The interruption length on PCell comprises 2x retuning time (switch to and back) and the time duration staying on SDL carrier (i.e., SMTC duration). It seems that the legacy interruption length for measurement on deactivated SCC for intra-band CA can be reused herein.

Table 8.2.2.2.3-1: Interruption duration for measurement on deactivated SCC for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

Regarding the interruption percentage, the loss of ACK/NACK are caused by both the interruption of PDCCH and the interruption of the UL transmission of carrying ACK/NACK. Then missed ACK/NACK percentage shall be represented as X=YmeasCycleSCell  where Y =2 ×interruption length.

Proposal 6: For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL reuse legacy interruption length for measurement on deactivated SCC for intra-band CA (Table 8.2.2.2.3-1 in TS 38.133).

Proposal 7: For R19 LB CA via switching, interruptions due to measurements on deactivated SDL are allowed with up to X=YmeasCycleSCell  probability of missed ACK/NACK, where Y =2 × interruption length.

SDL SCell activation

Issue 1-3-7: SDL SCell activation/deactivation delay requirement - Agreement on Thursday: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple) - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation.(ZTE) - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern. (Nokia)

In last meeting, one issue regarding OOR CSI reporting and lowest valid L1 RSRP reporting during SCell activation procedure was raised. As per legacy SCell activation requirement, UE would report OOR to report CQI for the to-be-activated SCell during the duration from activation MAC CE decoding and until valid CQI is reported.

TS 38.133 clause 8.3.2 Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

The question is that in LB CA via switching, if the CQI OOR reporting occasion (@PCell) is colliding with SMTC occasion (@SDL carrier), what is UE behaviour, e,g., UE shall stay on SDL for SCell activation or switch to PCell for reporting?

We think the SMTC measurement on SDL during SCell activation procedure shall be prioritized over OOR reporting. As we know, SCell activation delay comprises THARQ, Tactivation and TCSI_reporting. During Tactivation UE performs DL synchronization/ AGC based on SMTC, so it is straight forward to guarantee these related SMTC occasions. After Tactivation, TCSI_reporting comprises three components: acquiring the first CSI resource, UE processing time for CSI, and acquiring the first available CSI reporting resources. Herein UE shall measure CSI-RS on SDL SCell for valid CSI reporting, so it shall also be prioritized over UL OOR reporting on PCell.

TS 38.133 clause 8.3.2 TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

One thing also needs to be pointed out, the ending point of valid CSI reporting does not confine to SMTC occasion, even UL reporting resource is colliding with SMTC on SDL.

Proposal 8: During the SDL SCell activation

- Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until Tactivation, SMTC based measurement on SDL is prioritized over transmission/reception on PCell.

- During TCSI_reporting , CSI-RS based CSI measurement on SDL is prioritized over transmission/reception on PCell.

Regarding L1-RSRP reporting during the duration from activation MAC CE decoding until UE has completed a first L1-RSRP measurement, we think the same principle can be applied. That is SSB/CSI-RS L1-RSRP measurement on SDL shall be prioritized over“lowest valid L1 SS-RSRP range” reporting. More precisely, SSB/CSI-RS L1-RSRP measurement on SDL shall be prioritized over transmission/reception on PCell.

It shall be noted that if all L1-RSRP reporting occasions are colliding with L1-RSRP measurement occasions, as per the above principle, UE is not able to report L1-RSRP measurement result to network. Therefore the SCell activation procedure which requires L1-RSRP reporting will fail.

TS 38.133 clause 8.3.2 Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

Proposal 9: Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, SSB/CSI-RS L1-RSRP measurement on SDL is prioritized over transmission/reception on PCell.

Proposal 10: There are no requirements for the SCell activation which requires L1 RSRP reporting, when all L1-RSRP reporting occasions are colliding with L1-RSRP measurement occasions.

Issue 1-3-8: interruption or scheduling restriction during SDL SCell activation/deactivation - Proposal 1 (OPPO, CATT(scheduling restriction), Nokia): - RAN4 to define the scheduling restriction or interruption caused by the measurements during SDL SCell activation procedure. - Proposal 2 (Apple): - the interruption pattern due to CC switching in SDL SCell activation shall follow SSB periodicity or SMTC periodicity used for SCell activation. - the interruption length for each CC switching in SDL SCell activation can be defined as {(switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL)} or reuse same interruption requirement from legacy FR1 intra-band SCell activation. - Proposal 3 (LGE): - RAN4 to specify SCell activation window which ensures SCell measurements of the deactivated SCell. - Proposal 4 (HW): - During SDL SCell activation, if the to-be-used RS resource is SSB: - Option 1: UE stays on SDL carrier until the SCell activation procedure is completed. The whole SCell activation delay would be specified as interruption or scheduling restriction. It shall be noted that UE shall also wait for one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC. - Option 2: UE switches to SDL carrier according to SSB occasions. The interruption or scheduling restriction would happen on SSB symbols and 1 symbol before each consecutive SSB symbols to be measured and 1 symbol after each consecutive SSB symbols to be measured. It shall be noted that UE shall also measure one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC. - FFS if the to-be-used RS is A-TRS or P-TRS. - Proposal 5 (ZTE): - RAN4 needs to consider the switching period/interruption during the SDL SCell activation procedure for the switching between PCell and SDL SCell L3/L1 measurement

During SCell activation procedure, the interruption or scheduling restriction would happen on measured SSB symbols/SMTC and two-way switching time before each consecutive SSB symbols/SMTC to be measured. As network knows the location of SMTC and the switch time, to reduce throughput loss, we prefer to specify scheduling restriction rather than interruption.

Proposal 11: Define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

UE is not expected to receive PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and transmit PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of {(symbol(s) for switching time of FDD-to-SDL) + SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD)}.

Proposal 12: The scheduling restriction in SDL SCell activation can be defined as:

UE is not expected to receive PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and transmit PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of {(symbol(s) for switching time of FDD-to-SDL) + SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD)}.

BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

Issue 1-3-10: BFD/CBD requirements for SDL SCell Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

As we know in FR1, there are multiple transmit beams from network. That is why RAN4 agreed to define requirements for TCI state switch delay requirement and L1 RSRP measurement for SDL SCell. So regarding BFD/CBD on SDL SCell, we don’t observe the strong objection to define corresponding requirements.

Issue 1-3-11: TCI state switch delay requirement for SDL SCell Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Proposal 13: Define requirement for BFD/CBD for SDL SCell in LB CA via switching. The BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

As per legacy evaluation period of BFD and CBD, the low bound of measurement period is 50ms, it implies that one sample shall be acquired at least per 10ms. So it is NOT able to ensure UE can acquire at least one available SSB/CSI-RS occasion for CBD/BFD on SDL during 10ms. So when a fraction of {SSB occasions or CSI-RS for BFD/CBD} are not available due to the LB-CA switching pattern, the BFD/CBD evaluation period shall be correspondingly impacted. This is similar as L1 -RSRP measurement requirements discussed in Proposal 2. No further elaboration is needed here.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle > 320 ms | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. |  |

Proposal 14: When a fraction of {SSB occasions or CSI-RS for BFD/CBD} are not available due to the LB-CA switching pattern, the BFD/CBD evaluation period shall be updated with updating the scaling factor P where fraction of available occasions due to both measurement gap and LB-CA switching pattern are considered.

P value for a BFD-RS resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable> 0 - For a window W of duration max(TL1, xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion: - Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W, and - Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or any SDL duration corresponding to switching pattern for BFD monitoring on PCell, or any FDD duration corresponding to switching pattern for BFD monitoring on SDL SCell within the window W, and

PCC related RRM requirements

For low band switching, UE semi-static switches between PCell and SDL SCell. The SSB/CSI-RS based L3/L1 measurement, TCI state switching, BFD/CBD evaluation on PCC are supposed to be performed on the overlapped occasions between SMTC window and PCC duration in switch pattern. The detailed requirements on SDL carrier are discussed in clause 2.3, the same principle can be applied for PCC in LB CA via switching.

Proposal 15: The SSB/CSI-RS based L3/L1 measurement, TCI state switching, BFD/CBD evaluation on PCC are supposed to be performed on the overlapped occasions between SMTC window and PCC duration in switch pattern.

Proposal 16: The same requirements on SDL carrier discussed in clause 2.3, can be applied to PCC in LB CA via switching.

Conclusions

This contribution provides further analysis on low band carrier aggregation via switching from RRM perspective. The following proposals are provided:

Observation 1: When a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} are not available based on the LB-CA switching pattern, even when multiplied by scaling factor Kp-LB, the L3 measurement delay will not exceed 200ms (low boundary of measurement period).

Observation 2: When SDL SCell SMTC ≥ switching pattern periodicity P (i.e., 40ms), the SMTC would not be impacted by switching pattern.

Proposal 1: For L3 measurement requirement, it is unnecessary to consider a scaling factor Kp-LB, when a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} are not available due to LB-CA switching pattern.

Proposal 2: When a fraction of {SSB occasions or CSI-RS for L1 measurement} are not available due to the LB-CA switching pattern, the L1-RSRP delay shall be updated with updating the scaling factor P where fraction of available occasions due to both measurement gap and LB-CA switching pattern are considered.

P value for SSB resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable > 0 - For a window W of duration max(TL1, xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs , periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or any SDL duration corresponding to switching pattern for L1-RSRP measurement on PCell, or any FDD duration corresponding to switching pattern for L1-RSRP measurement on SDL SCell within the window W, and - Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

Proposal 3: Confirm the agreement in last meeting: For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

Proposal 4: If RAN4 agree to define requirements for more carriers scenario, switching between FDD and SDL would interrupt other combined active SCell, the interruption length is TswitchingSlot-length-victim+1.  If no common understanding on RF agreement, it is fine not to specify requirements in R19.

Proposal 5: L3 measurement requirements on activated SDL follow proposal 1.

Proposal 6: For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL reuse legacy interruption length for measurement on deactivated SCC for intra-band CA (Table 8.2.2.2.3-1 in TS 38.133).

Proposal 7: For R19 LB CA via switching, interruptions due to measurements on deactivated SDL are allowed with up to X=YmeasCycleSCell  probability of missed ACK/NACK, where Y =2 × interruption length.

Proposal 8: During the SDL SCell activation

- Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until Tactivation, SMTC based measurement on SDL is prioritized over transmission/reception on PCell.

- During TCSI_reporting , CSI-RS based CSI measurement on SDL is prioritized over transmission/reception on PCell.

Proposal 9: Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, SSB/CSI-RS L1-RSRP measurement on SDL is prioritized over transmission/reception on PCell.

Proposal 10: There are no requirements for the SCell activation which requires L1 RSRP reporting, when all L1-RSRP reporting occasions are colliding with L1-RSRP measurement occasions.

Proposal 11: Define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

Proposal 12: The scheduling restriction in SDL SCell activation can be defined as:

UE is not expected to receive PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and transmit PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of {(symbol(s) for switching time of FDD-to-SDL) + SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD)}.

Proposal 13: Define requirement for BFD/CBD for SDL SCell in LB CA via switching. The BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Proposal 14: When a fraction of {SSB occasions or CSI-RS for BFD/CBD} are not available due to the LB-CA switching pattern, the BFD/CBD evaluation period shall be updated with updating the scaling factor P where fraction of available occasions due to both measurement gap and LB-CA switching pattern are considered.

P value for a BFD-RS resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable> 0 - For a window W of duration max(TL1, xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion: - Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W, and - Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or any SDL duration corresponding to switching pattern for BFD monitoring on PCell, or any FDD duration corresponding to switching pattern for BFD monitoring on SDL SCell within the window W, and

Proposal 15: The SSB/CSI-RS based L3/L1 measurement, TCI state switching, BFD/CBD evaluation on PCC are supposed to be performed on the overlapped occasions between SMTC window and PCC duration in switch pattern.

Proposal 16: The same requirements on SDL carrier discussed in clause 2.3, can be applied to PCC in LB CA via switching.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19

- R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple