3GPP TSG-RAN WG4 Meeting #116	R4-2510711

# Bangaluru, India, August 25th – 29th 2025

Source:	Ericsson

Title:	Discussion on general aspects of NR SCM for demodulation

Agenda item:	7.12.2

Document for:	Discussion

# Introduction

In RAN4#115 meeting, companies have agreements on common CDL model derivation and antenna configurations for alignment and comparison separately. Regarding many candidate models and test cases are on the table, the TR content should be well structured to be clearer and more readable.

In this contribution, we give some views on TR content and generalization of CDL model.

# Discussion

As we mentioned in the previous meeting, the naming of SCM models and antenna configurations, such as “CDLC A2”, “CDLC A4”, “AAV 1Y” and “AAV 3” should be sorted in the final TR to avoid confusion to readers. The suggestion is to use numbers as index without character in the name. The alignment through whole TR is also necessary.

Proposal 1	Rename SCM model and antenna configuration with sorted numbers in the final TR to avoid confusion to readers.

For simulation alignment, it is suggested to have total throughput values for 8 layers PDSCH case together with per CW throughput results. Currently, per CW throughput results from companies show large gaps, but the total throughput performance could be close on the other hand. One of the reasons could be that the signal power split per CW is different in companies, i.e., different PMI selection sets implementation leads to one CW has much more power than the other but total signal power is same after normalization. To address this potential and help companies to have better alignment, it is better to capture total throughput for 8 layers either.

The total normalized throughput could be similar when per CW normalized throughputs are quite different among different implementations.

Proposal 2	Adding total normalized throughput results for 8 layers PDSCH in alignment chapter.

The CDL model derivation method for MU-MIMO is proposed by some companies but there is no official discussion on it due to the very tight meeting schedule. It was agreed to capture the contribution from companies in the TR. It could be better to capture it in a new Annex as informative reference. But it should be noted that the Rel-20 SID on SCM would continue MU-MIMO study. In that case, the new Annex in Rel-19 TR would be removed in Rel-20 TR, and new sections for MU-MIMO will be added. Regarding the Rel-20 study would start very soon, proponent company could resubmit their methodology for Rel-20 SI. It seems not necessary to capture MU-MIMO part in Rel-19 TR Annex.

Rel-20 SCM study on MU-MIMO would start soon and it seems not necessary to capture MU-MIMO methodology without discussion in Rel-19 TR because the MU-MIMO part in Rel-19 TR will be removed in Rel-20 TR anyway.

Proposal 3	Companies to check if it is necessary to capture MU-MIMO methodology in Rel-19 TR. If companies think it necessary, capture it in a new Annex as informative information.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The total normalized throughput could be similar when per CW normalized throughputs are quite different among different implementations.

Observation 2	Rel-20 SCM study on MU-MIMO would start soon and it seems not necessary to capture MU-MIMO methodology without discussion in Rel-19 TR because the MU-MIMO part in Rel-19 TR will be removed in Rel-20 TR anyway.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Rename SCM model and antenna configuration with sorted numbers in the final TR to avoid confusion to readers.

Proposal 2	Adding total normalized throughput results for 8 layers PDSCH in alignment chapter.

Proposal 3	Companies to check if it is necessary to capture MU-MIMO methodology in Rel-19 TR. If companies think it necessary, capture it in a new Annex as informative information.

# References

[1] 	R4-2508776, Way Forward for [115][321] FS_NR_demod_SCM, Nokia

[2]	TR 38.753 v19.1.0