3GPP TSG-RAN WG4 Meeting #116	   R4-2510791

# Bangalore, India, 25th – 29th , August , 2025

Agenda Item:	7.21.4.1

Source: 	Ericsson

Title:	Discussion on RRM requirements for UE-to-UE CLI handling (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#115 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

# 2.1 UE to UE CLI handling

### Requirement general issue

#### Issue 1-1-1: Requirement applicability Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 : - Note: The assumption for basic UE operation is that UE cannot receive simulatenous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

Based on our understanding of the RAN1 #121 meeting progress, there is no agreement on supporting the DL reception in two DL subbands in SBFD symbols as the basic UE feature. We think RAN4 can wait for RAN 1 conclusion.

- Proposal 1: RAN4 wait for the RAN1 conclusion on whether support DL reception in two DL subbands in SBFD symbols as the basic UE feature.

### L1-SRS-RSRP measurement

#### Issue 1-2-3: Measurement restriction Agreement - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2. Agreement A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol..

Based on existing RAN1 agreements on L1-CLI measurement configuration, the method#1(RSSI measurement on DL subband) and method#3 (RSSI measurement on UL subband) for L1-CLI-RSSI cannot be performed on the same OFDM symbol based on RAN1#119 agreement.

Also, the method# 3 (RSSI measurement on UL subband) and method#2 (SRS-RSRP within UL subband) cannot be performed on the same OFDM symbol.

RAN1#121 A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol. RAN1 #119 For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

The only open question relying on whether the method#2 (SRS-RSRP within UL subband) and method#1 (RSSI measurement on DL subband) can be performed at the same OFDM symbol. Due to the measurement timing of the SRS-RSRP and RSSI is different, we don’t see the technical feasibility to perform the two sets of measurements at the same time.

If RAN4 introduce measurement restrictions between method#1 and method#2, clarification in needed for which method have the priority at what scenario. This only complicates the issue without clear benefit. Also, it is upon the network configuration to decide whether SRS and RSSI measurement resources will be on the same OFDM symbol, we think it is better to clarify this from RAN1 whether the SRS within UL subband and RSSI measurement resources within DL subband can be configured within the same OFDM symbol.

- Proposal 2: RAN4 wait for RAN1 conclusion on whether measurement resource for method#1 and method#2 can be configured at the same OFDM symbol.

### L1-CLI-RSSI measurement

#### Issue 1-3-1: Measurement period - Agreement - Close the issue and use the existing agreement, i.e. measurement period is based on 1 sample. RAN1#120 bis agreements For the following question from the RAN4 LS in R1-2501698: - RAN4 respectfully asks RAN1 to clarify whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1-SRS-RSRP and/or L1-CLI-RSSI measurement. - RAN1 informs RAN4 that the NW configuration of timeRestrictionForChannelMeasurement will apply for L1-SRS-RSRP and L1-CLI-RSSI measurement. - RAN4 kindly asks whether RAN1 will define optional UE capability for FDM-ed DL reception and SRS measurement. - RAN1 informs RAN4 that a new optional UE capability will be defined for FDM-ed DL reception and SRS measurement.

The parameter timeRestrictionForChannelMeasurement is inherited from CSI framework which is used for CSI-RS based L1-RSRP measurement not any RSSI type of measurement.

The RSSI is measurement for total power included wanted signal and unwanted interference. Due to the CLI issue only occurs when aggressor UE is close to the victim UE, adding more samples will leading to longer measurement delay which will introduce uncertainty when UE report the RSSI to the network.

Using 3 samples for RSSI measurement means UE is allow to perform L1 filtering which is totally up to UE implementation. Different UE implementation will have different impact on the RSSI measurement results, This will negatively impact the measurement requirement setting. We don’t see any benefit to revert the existing agreement on 1 sample.

- Proposal 3: 1 The measurement period for L1-RSSI shall be based on 1 sample.

#### Issue 1-1-3: L1 CLI measurement in non-SBFD symbols RAN1# 121 agreements: For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

Based on our interpretation of the RAN1 latest agreement, the L1-CLI measurement will only be applicable when SBFD symbols are configured for the UE. The valid symbol type configured in the CSI-ReportConfig only applicable when SBFD symbols are configured. This indicate there will be no L1-CLI measurement in non-SBFD symbols.

- Proposal 4: RAN4 close this issue due to L1-CLI measurement only applicable when SBFD symbols are configured for the UE.

#### Issue 1-3-3: Measurement restriction Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

Based on the same reference timing for L1-CLI-RSSI and the L1/L3 RRM measurements, we do see there is potential gain to have these two types of measurements performed at the same time. For example, at FR1, there is no RX beam complication, and there is no need to re-tune to another frequency for the measurement gap, we do see the possibility for UE to perform the L1-CLI-RSSI measurement together with other measurements like L1/L3 RRM measurement at the same time. We think there is no need for introducing measurement restrictions when L1-CLI-RSSI and L1/L3 measurement without gaps.

However, when measurement gaps are being configured for L1/L3 measurements, due to the L1-CLI-RSSI can be either within UL or DL subband, it is rather complicated to restrict how UE can perform L1-CLI-RSSI measurement together with the RRM measurement within the gaps, we think the L1/L3 measurement needs to be prioritized.

- Proposal 5: For Method #1& Method#3, measurement requirements do not apply for L1-CLI-RSSI when CLI resources are overlapping with measurement gaps at FR1.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN4 wait for the RAN1 conclusion on whether support DL reception in two DL subbands in SBFD symbols as the basic UE feature.

	Proposal 2: RAN4 wait for RAN1 conclusion on whether measurement resource for method#1 and method#2 can be configured at the same OFDM symbol.

	Proposal 3: 1 The measurement period for L1-RSSI shall be based on 1 sample.

	Proposal 4: RAN4 close this issue due to L1-CLI measurement only applicable when SBFD symbols are configured for the UE.

	Proposal 5: For Method #1& Method#3, measurement requirements do not apply for L1-CLI-RSSI when CLI resources are overlapping with measurement gaps at FR1.

# References

- R4-2508274 “WF on SBFD RRM”, Huawei, May ,2025

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024