3GPP TSG RAN WG4 Meeting #116		R4-2510819

Bengaluru, India, 25th - 29th August, 2025

Agenda item:	7.10.3.1

Source:	Vodafone, Telecom Italia S.p.A., Deutsche Telekom

Title:	Discussion on OTA requirements for XR Headworn Devices

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

At RAN4#115, operators contributed to the discussion on adapting TRP/TRS testing for XR headworn devices by highlighting limitations of full-sphere averaging in capturing directional performance gaps [1]. As a result, RAN4 agreed to further study alternative metrics better suited to XR use cases. Building on that direction, this contribution proposes adopting Near-Horizon Partial Radiated Power (NHPRP) and Near-Horizon Partial Radiated Sensitivity (NHPRS) as minimum OTA performance metrics for XR headworn devices, reflecting real-world usage while reducing test time, and remaining compatible with current TRP/TRS procedures.

# 2	Discussion

XR headworn devices differ from handheld UEs in several key aspects:

- They are worn with fixed orientation relative to the user’s head and body.

- With fewer RX antennas, the likelihood for creation of deep nulls in downlink radiation pattern increases. The more antennas are spatially distributed, the smoother the radiation pattern thereby reducing the likelihood of deep nulls.

- With a reduced number of antennas, there is a potential for coverage gap. Even if the averaged TRP & TRS values appear acceptable, localized regions of poor performance may still exist. In a fixed-use scenario against user’s body like that of XR headworn, these gaps can significantly degrade the user experience, especially in areas where consistent connectivity is essential.

- Their performance is most critical in forward and side directions, aligned with typical base station locations.

- Their battery constraints limit the feasibility of long-duration OTA tests.

Unlike handheld UEs, XR devices are worn with a fixed, forward-facing orientation against the user's face and body. This consistency in use introduces directional sensitivity performance in certain angles directly impacts the user experience. Modern XR headsets typically include fewer RX antennas, often reduced from four to two, due to form factor and power constraints. This reduction increases the likelihood of deep nulls in the spatial radiation pattern. In cases where such nulls fall within directions critical for XR use, the overall averaged TRP/TRS may still pass, while the user experiences degraded connectivity in practice.

Observation 1: The full-sphere averaging of TRP/TRS can mask critical performance nulls in important directions, especially when fewer antennas are used in XR devices.

In real deployments, the majority of useful signal energy is exchanged within a horizontal band around the user, covering front, sides, and rear azimuthal directions. This is especially true for XR glasses, where the user may turn their head frequently while interacting with the environment, and connectivity must be maintained from any horizontal direction. As such, full azimuthal coverage within a limited elevation range becomes critical. In contrast, vertical extremes, particularly the bottom directions are often obstructed by the user’s body.

Observation 2: For XR headworn devices, consistent coverage across the full horizontal azimuth is important, while bottom and top elevation angles are less critical due to frequent body or environmental obstruction.

This mismatch between test metric and deployment reality leads to inefficient testing and potential misalignment with user-perceived performance. Testing the full spherical surface also consumes more time, which is problematic for XR devices with small batteries.

Observation 3: Full-sphere TRP/TRS testing imposes high time costs, which are particularly burdensome for XR headworn devices with limited power budget.

Given these observations, we propose to adopt Near-Horizon Partial Radiated Power (NHPRP) and Near-Horizon Partial Radiated Sensitivity (NHPRS) as minimum OTA metrics for XR headworn devices. The near-horizon region aligns with actual traffic paths in indoor and outdoor deployments and corresponds with the natural interaction angle between XR users and surrounding gNBs.

Observation 4: NHPRP and NHPRS require no change to test equipment or test chamber setup, and can be derived from the same EIRP/EIS per-angle data already collected during TRP/TRS testing.

Based on the above, we propose the following:

- Proposal 1: Adopt NHPRP and NHPRS as minimum OTA performance metrics for XR headworn devices.

- Proposal 2: Define the near-horizon region as an elevation band centered around θ = 90° and 360° azimuth. The exact elevation range (e.g., ±30° or ±45°) can be FFS.

- Proposal 3: Full-sphere TRP/TRS may continue to be reported for informational purposes, but compliance should be assessed using NHPRP/NHPRS.

# 3	Conclusions

In the following, the observations and proposals of the contribution are summarized:

Observation 1: The full-sphere averaging of TRP/TRS can mask critical performance nulls in important directions, especially when fewer antennas are used in XR devices.

Observation 2: For XR headworn devices, consistent coverage across the full horizontal azimuth is important, while bottom and top elevation angles are less critical due to frequent body or environmental obstruction.

Observation 3: Full-sphere TRP/TRS testing imposes high time costs, which are particularly burdensome for XR headworn devices with limited power budget.

Observation 4: NHPRP and NHPRS require no change to test equipment or test chamber setup, and can be derived from the same EIRP/EIS per-angle data already collected during TRP/TRS testing.

Proposal 1: Adopt NHPRP and NHPRS as minimum OTA performance metrics for XR headworn devices.

Proposal 2: Define the near-horizon region as an elevation band centered around θ = 90° and 360° azimuth. The exact elevation range (e.g., ±30° or ±45°) can be FFS.

Proposal 3: Full-sphere TRP/TRS may continue to be reported for informational purposes, but compliance should be assessed using NHPRP/NHPRS.

# 4	References

- R4-2507005, “Discussion on OTA requirements for XR Headworn Devices”, Vodafone, Telecom Italia S.p.A., Deutsche Telekom, May 2025.