3GPP TSG-RAN WG4 Meeting #116	R4-2510836

Bengaluru, India, 25 August – 29 August 2025

Agenda Item:	7.17.7

Source:	Ericsson

Title:	PMI reporting requirements with CSI prediction

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forward on AI/ML for NR air interface [1]. The way forward captured the agreements on CSI prediction as follows:

Issue 1-1: Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII. - The exact test procedure and configurations are FFS - The impact of different timeline between AI and R16 etype II should be also carefully analysed Issue 1-3: Doppler handling Agreement: - De-prioritize 100Hz Doppler in R19 - Consider 100Hz Doppler in the future release if needed Issue 1-5: SNR levels for training data and generalization Agreement: No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS

This contribution discusses the PMI reporting requirements with CSI prediction.

# 2	PMI reporting requirements

## 2.1	SGCS of predicted channel estimates

RAN4 has already agreed that the test metric of AI/ML-based PMI reporting requirements is the throughput ratio of follow PMI with predicted PMI and random PMI. However, RAN4 agreed to try to align the SGCS of the predicted channel estimates before RAN4 evaluates the throughput ratio. The alignment of SGCS consists of two steps [2]:

- Step 1: SGCS between SVD of model output (before PMI generation) and SVD of the ground truth raw channel, average over sub-bands, based on ideal channel estimates (SGCS-1).

- Step 2: SGCS between predicted PMI and SVD of the ground truth raw channel, average over sub-bands, based on ideal channel estimates.

  - SGCS-2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD.

  - SGCS-3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI derived based on Rel-18 eType II-Doppler CB with the ground truth.

After we confirm the alignment of SGCS, RAN4 will check the PDSCH throughput with follow PMI as Step 3.

- Step 3: Check PDSCH throughput with the predicted PMI.

Table 1 summarizes our simulation results of step 1 and step 2 according to [2]. We have evaluated the average of SGCS based on two CSI prediction models. We have trained the first model with a dataset that is the same as the test. For example, when we run the simulation with TDLA30-20, we used the model trained only with TDLA30-20. On the other hand, we have trained the second model with a mixed dataset including TDLA30-20, -50, and -100.

The simulation results do not show significant differences of SGCS-1/2/3 due to the trained dataset option (mixed or dedicated) at least for TDLA30. The summary also compares SGCS-1, SGCS-2, and SGCS-3 according to the simulation assumption. Our simulation results show SGCS-1 is the highest, followed by SGCS-3, and SGCS-2. However, the degradation of SGCS-3 is not large compared with SGCS-1.

Table 1	Summary of average SGCS between the SVD of model output and SVD of ground-truth (Step 1 and Step 2).

| Number of CSI-RS ports | Channel model for test | Channel model for training | SGCS-1 |  | SGCS-2 |  | SGCS-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Layer 0 | Layer 1 | Layer 0 | Layer 1 | Layer 0 | Layer 1 |
| 32 (4x4x2) | 20Hz | 20Hz | 0.99 | 0.99 | 0.87 | 0.77 | 0.98 | 0.97 |
|  |  | Mixed | 0.99 | 0.99 | 0.87 | 0.77 | 0.96 | 0.93 |
|  | 50Hz | 50Hz | 0.94 | 0.87 | 0.83 | 0.68 | 0.88 | 0.77 |
|  |  | Mixed | 0.94 | 0.87 | 0.83 | 0.68 | 0.88 | 0.78 |
| 16 (4x2x2) | 20Hz | 20Hz | 0.99 | 0.99 | 0.92 | 0.86 | 0.97 | 0.95 |
|  |  | Mixed | 0.99 | 0.99 | 0.92 | 0.86 | 0.96 | 0.93 |
|  | 50Hz | 50Hz | 0.93 | 0.86 | 0.85 | 0.75 | 0.89 | 0.81 |
|  |  | Mixed | 0.93 | 0.87 | 0.86 | 0.75 | 0.90 | 0.82 |

Observation 1: Average SGCS does not change whether the AL/ML-based CSI prediction model uses matched training dataset or mixed training dataset, at least for TDLA30-20/50.

Observation 2: Degradation of SGCS after the quantization (SGCS-3) is not significant compared with SGCS-1 (before quantization).

## 2.2	Follow PMI

RAN4 has agreed that the test metric of CSI prediction performance test is the PDSCH throughput ratio between follow Rel-18 Type-II Doppler PMI and random Rel-15 Type-I PMI.

Figure 1 is our simulation results for PDSCH demodulation with follow PMI according to Step 3 in [2], where Figure 1 (a) is with 32Tx and Figure 1 (b) is with 16Tx. The Figures also show the follow PMI results with Rel-18 Type-II Doppler with the same PMI and CSI-RS resource configuration as AI/ML-based CSI prediction, that is, N4=1 and CSI-RS scheduling period of 5 slots.

| (a) 32Tx | (b) 16Tx |
| --- | --- |

Figure 1	PDSCH demodulation with follow PMI.

Although Rel-18 PMI reporting requirements with Type-II Doppler uses the XP medium correlation for the antenna correlation assumption, we evaluated the low correlation and medium correlation scenarios. From the simulation results, we observed the AI/ML-based PMI prediction shows slightly better performance compared with Rel-18 with the same test setup (e.g., CSI-RS resource configuration, CSI report configuration, channel model). Especially 16Tx scenario (~0.4dB@90% of peak) shows larger gain compared with 32Tx (~0.1dB@90% of peak).

Observation 3: AI/ML-based PMI prediction shows slightly better performance compared with Rel-18 with the same test setup.

Observation 4: The performance gain of AI/ML-based PMI prediction over Rel-18 PMI prediction is larger for 16Tx compared with 32Tx.

We propose RAN4 evaluates further the PDSCH demodulation performance with the AI/ML-based PMI to decide the test configuration in the PMI reporting requirement. For example, RAN4 should consider other FRC such as 64QAM 1/2 and channel model such as TDLC300.

Proposal 1: RAN4 evaluates further PDSCH demodulation performance with AI/ML-based PMI.

## 2.3	Scheduling for PMI reporting requirements

Figure 2 and Figure 3 are the possible scheduling of CSI-RS, PMI reporting, and PDSCH, for FDD and TDD scenarios, respectively, used for Rel-19 AI/ML-based CSI prediction, where we have assumed the following:

- N4=1 (Doppler/time-domain basis vector length)

- Delta = 1 (Prediction delay)

- Periodic CSI-RS resources scheduled every 5 slots for FDD and 10 slots for TDD.

- CSI reporting delay is same as ‘typeII-Doppler-r18’.

Figure 2	Test setup for FR1 FDD case of AI/ML-based CSI prediction.

Figure 3	Test setup for FR1 TDD case of AI/ML-based CSI prediction.

RAN1 agreed to introduce additional CSI processing delay t to Z2/Z2’, where RAN1 is still discussing the value(s) and it may depend on the UE reported capability. Since Figure 2 and Figure 3 are derived from the assumption t=0, if RAN1 agreed to introduce t < 0 and/or t > 0, the PMI reporting timing should be revisited.

Observation 5: RAN1 is still discussing the CSI processing time for Rel-19 AI/ML-based PMI prediction.

Proposal 2: RAN4 continues the AI/ML-based PMI prediction performance evaluation based on the CSI processing time assumption with t=0. RAN4 should revisit the scheduling when RAN1 concludes the UE capability of reporting delay t.

# 5	Summary

Observations:

Observation 1: Average SGCS does not change whether the AL/ML-based CSI prediction model uses matched training dataset or mixed training dataset, at least for TDLA30-20/50.

Observation 2: Degradation of SGCS after the quantization (SGCS-3) is not significant compared with SGCS-1 (before quantization).

Observation 3: AI/ML-based PMI prediction shows slightly better performance compared with Rel-18 with the same test setup.

Observation 4: The performance gain of AI/ML-based PMI prediction over Rel-18 PMI prediction is larger for 16Tx compared with 32Tx.

Observation 5: RAN1 is still discussing the CSI processing time for Rel-19 AI/ML-based PMI prediction.

Proposals:

Proposal 1: RAN4 evaluates further PDSCH demodulation performance with AI/ML-based PMI.

Proposal 2: RAN4 continues the AI/ML-based PMI prediction performance evaluation based on the CSI processing time assumption with t=0. RAN4 should revisit the scheduling when RAN1 concludes the UE capability of reporting delay t.

# References

- R4-2508080, “WF on NR_AIML_air_part1”, Qualcomm Incorporated.

- R4-2508102, “WF on updated simulation assumptions for AI based CSI prediction”, OPPO, et al.