3GPP TSG-RAN4 Meeting # 116	R4-2510928

Bangalore, IN. 25th – 29th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Considerations on ModifiedMPR-Behaviour

# Agenda item:	4.2.1

# Document for:	Approval

# 1 Introduction

The issue on ModifiedMPR-Behaviour have been discussing intermittently in recent RAN4 meeting, and it seems it was discussed in separated topics with nonalignment understanding.

Recently, this issue was triggered by the PHS protection, where the existing NS_05/NS_05U A-MPR requirements for band n1 would need to be updated due to the new regulation requirements in Japan. Once the new NS_05/NS_05U A-MPR requirements are introduced in the latest release specification, the consequent issue is how to align the modifiedMPR-Behaviour among releases.

In last meeting, the agreed WF shows some open issues which are going to be discussed in this meeting, which are:

In next meeting, at least the following issues will be further discussed. - Check the difference between bit ‘0’ and absent from BS and UE perspective. - Check if the spec version number can be removed from next release. - Check if the requirements are changed several times, how to handle the legacy bits. <...> - Check if any good approach on how to handle the bits for different form factors, if dedicated bits should be used to differentiate the form factors, FWA and smart phone, for future new cases. - <...>

In this contribution, we give some general considerations on modifiedMPR-Behaviour among releases.

# 2	Discussion

It is well known that the conception of ModifiedMPR-Behaviour aims to meet the new emission requirements as long as some new regulation requirements need to be meet.

We extract some texts related to modifiedMPR-Behaviour from the existing specifications TS38.101-1/2 in the following:

For n41 in TS38.101-1:

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

For n258 in TS38.101-2:

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

From the above extracted texts, we found different approaches were adopted between TS38.101-1 and TS38.101-2, where in TS38.101-1, the MPR/A-MPR requirements with the same NS value(i.e. NS_XX) are updated in the latest specification to meet the new regulations, while in TS38.101-2, the MPR/A-MPR requirements with the new added NS value(i.e. NS_XX) in the latest specification to meet the new regulations.

In TS38.101-2, it seems it may not be a problem to mandatory set (‘shall be’)the bit to 1 for different NS value since NW would know which NS values for the UEs can support in the case of two types of UE existing in a real environment: one is UE working with an original NS and the other is UE working with a new NS.

However, it seems the issue is more serious in TS38.101-1, as an example of n41, there exists ambiguity that NW would not know which version of MPR/A-MPR requirements that the UE complies with since misusing of ‘shall’ and ‘may’ for the bit setting which makes it is possible to set both bit ‘0’ and bit ‘2’ to 1.

Although the issue was mainly triggered by PHS protection discussion, we believe it is not only for band n1 NS_05/NS_05U, but also for other bands who have the same situation. Therefore, we think it should be a generic issue and it would be better that RAN4 can come up with the rule/guidance, and the rule/guidance should be applied to the latest Rel-19 specification, no need touch the earlier specifications, we think that is the business as usual.

Observation 1: Different approaches were adopted between TS38.101-1 and TS38.101-2, where in TS38.101-1, the A-MPR requirements with the same NS value(i.e. NS_XX) are updated in the latest specification to meet the new regulations, while in TS38.101-2, the A-MPR requirements with the new added NS value(i.e. NS_XX) in the latest specification to meet the new regulations.

Observation 2: ModifiedMPR-Behaviour issue is more serious in TS38.101-1 since misusing of ‘shall’ and ‘may’ for the bit setting which makes it is possible to set both bit ‘0’ and bit ‘2’ to 1 for the same NS value.

Proposal 1: RAN4 should come up with the generic rule/guidance on modifiedMPR-Behaviour for both FR1 and FR2, and implement it in the latest Rel-19 specification.

Regarding the open issues in the WF, we give some briefly discussion in the following:

- Check the difference between bit ‘0’ and absent from BS and UE perspective.

In current TS38.331 spec, the modifiedMPR-Behaviour is defined as below:

modifiedMPR-Behaviour    BIT STRING (SIZE (8))          OPTIONAL,

It can be seen that it is 8-bit signalling IE, and we think it is not allowed to set bit to absent and ‘1’ or absent and ‘0’ together. In others words, ‘1’ shall be used together with ‘0’. Absent can only be used when all the bits are set to ‘0’, in this case, absent would be the same meaning as ‘00000000’.

Therefore, in our understanding, when UE report modifiedMPR-Behaviour as absent, it means all of 8 bits are ‘0’, but it does not fit the modifiedMPR-Behaviour descriptions in the Notes, where at least one of the bit shall be set to ‘1’. From this aspect, it seems UE should not report absent for IE modifiedMPR-Behaviour.

- Observation 3: Absent would be the same meaning as ‘00000000’, but it does not fit the modifiedMPR-Behaviour descriptions in the Notes, UE should not report absent for IE modifiedMPR-Behaviour.

- While bit ‘0’ can be understood well, which means the corresponding A-MPR/MPR requirements in a certain version cannot be used, instead updated/new A-MPR/MPR requirements should be used in the case of the other bit is set to ‘1’, and we think NW can also understand well when UE report/set a certain bit to ‘0’.

- Observation 4: Bit ‘0’ means the corresponding A-MPR/MPR requirements in a certain version cannot be met.

- However, NW may feel confusion if more than 1bit are mandatory set to ‘1’ at the same time.

- Check if the spec version number can be removed from next release.

As mentioned above, when the new regulation requirements are introduced, then new emission requirements and the corresponding MPR/A-MPR requirements may need to be introduced and implemented via ModifiedMPR-Behaviour approach.

In general, the new emission requirements and the corresponding MPR/A-MPR requirements will be introduced together in a certain specification version (for example: 19.0.0), and of course, these new requirements are in the next release of this version (for example: 19.1.0.).

Ideally, UE must obey these new requirements from this version onwards unless the requirements are changed again.

Although keeping the spec version number in the release version onwards can provide the information when the requirements are updated/introduced, it seems it may cause confusion or readability problem for the people who may still refer back to the old release version to check the requirement and may raise some questions such as whether there are difference between the old version and latest version. Also if people provide CR to correct the requirements but the CR is not allowed for the old/frozen version(s), then it would cause misalignment between versions and also would cause confusion which ones should be used. In this case, we think it would be workable to refer to the latest version. Thus we slight prefer to remove the spec version number from next release.

Proposal 2: We slight prefer to remove the spec version number from next release.

- Check if the requirements are changed several times, how to handle the legacy bits.

It is not desirable that the requirements are changed overtimes. According to the current approach, once the new MPR/A-MPR requirements are introduced due to the new regulation emission requirements, then the corresponding bit shall be set to ‘1’ in modifiedMPR-Behaviour filed to indicate UE must meet the new regulation emission requirements with the new MPR/A-MPR requirements from the release version onwards. For the backwards release, we think it should allow UEs to optionally meet the new MPR/A-MPR requirements, i.e. ‘shall’ should be correct to ‘may’ in TS38.101-1.

For TS38.101-2, due to new NS value approach was adopted, so in the backwards release, UEs shall meet the new MPR/A-MPR requirements siganlled by new NS_xx, see the following EESS n258 example when bit number #3 and #4 were introduced.

Rel-15~Rel-18:

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 onwards | - This bit may be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.6 in the latest version of Rel-19 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

Rel-19:

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

So it seems the legacy bits can be keep unchanged.

Proposal 3: For 38.101-1, ‘shall’ should be correct to ‘may’ in legacy bits in backwards release to allow UEs to optionally meet the new MPR/A-MPR requirements.

Proposal 4: For 38.101-2, the legacy bits can keep unchanged.

# 3 Conclusion

In this contribution, we give some consideration on modifiedMPR-Behaviour among releases. The proposals and conclusions are:

Observation 1: Different approaches were adopted between TS38.101-1 and TS38.101-2, where in TS38.101-1, the A-MPR requirements with the same NS value(i.e. NS_XX) are updated in the latest specification to meet the new regulations, while in TS38.101-2, the A-MPR requirements with the new added NS value(i.e. NS_XX) in the latest specification to meet the new regulations.

Observation 2: ModifiedMPR-Behaviour issue is more serious in TS38.101-1 since misusing of ‘shall’ and ‘may’ for the bit setting which makes it is possible to set both bit ‘0’ and bit ‘2’ to 1 for the same NS value.

Proposal 1: RAN4 should come up with the generic rule/guidance on modifiedMPR-Behaviour for both FR1 and FR2, and implement it in the latest Rel-19 specification.

- Observation 3: Absent would be the same meaning as ‘00000000’, but it does not fit the modifiedMPR-Behaviour descriptions in the Notes, UE should not report absent for IE modifiedMPR-Behaviour.

- Observation 4: Bit ‘0’ means the corresponding A-MPR/MPR requirements in a certain version cannot be met.

Proposal 2: We slight prefer to remove the spec version number from next release.

Proposal 3: For 38.101-1, ‘shall’ should be correct to ‘may’ in legacy bits in backwards release to allow UEs to optionally meet the new MPR/A-MPR requirements.

- Proposal 4: For 38.101-2, the legacy bits can keep unchanged.

# 4	Reference

[1] R4-2507957, WF on Modified MPR Behavior, Xiaomi

[2] R4-2507540, Topic summary for [115][101] Upto_R16_UERF_maintenance, moderator(OPPO)