3GPP TSG-RAN4 Meeting # 116	R4-2510932

Bangalore, IN. 25th – 29th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Considerations on IMD MSD order for HPUE LUT

# Agenda item:	7.1.2.1.1

# Document for:	Approval

# 1 Introduction

In previous meetings, the HPUE MSD rules were discussed. After long time discussion, the called ‘LUT’ (Look-Up Table) approach was introduced for the HPUE MSD, and the LUT approach details can be found in the WF[1][2].

- It should noted that only the MSD types of 1 band 1 UL CC (i.e. harmonic/harmonic mixing/cross-band isolation) and 2 band 2 UL CC(i.e. IMD) have been completed by LUT approach. For the other twos, i.e. 1 band 2 UL CC (IMD) and 2 band 3 UL CC (i.e. triple beat IMD), are not in the scope so far.

- For the LUT approach of 2 band 2 UL CC IMD, one following issue is how to consider the IMD order for the HPUE band combination, we have proposals in [3] in last meeting, but it was not discussed due to lack of time.

In this contribution, we continue discussing the IMD MSD order for HPUE LUT.

# 2	Discussion

- In TR38.846, the rules for the IMD MSD order can be found in sub-clause 7.4, we copy&paste here for convenience. Note that the rules are applied PC3/HPUE band combination, and there are no further agreements to update this rule from then on.

– For Rel-18 new introduced band combinations, – For type 1 UL configurations (e.g. UL_CA_n41C or CA_n78(2A)) ○ The lowest order IMD is recommended as worst case to represent single band UL transmission with UL configured intra-band CA. ○ If the DL band may be affected by a mix of even and odd order IMD products, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications. ○ ........ – For type 2 UL configurations (e.g. UL_CA_n1A-n3A) ○ If only one IMD order occurs per victim band, the MSD value if any shall be defined in the specifications. ○ If the DL band may be affected by a mix of even and odd order IMD products, then the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications. ○ .....

- The above rules were introduced in RAN4 #106bis meeting in [4], it is not only applied for 1 band 2UL CC IMD MSD, but also applied for 2 band 2UL CC and 3 UL CC IMD MSD. Before that, it seems there were no uniform approach for the companies if there are mix of even and odd order IMD products, which make it flexible for proponent in their TP to TR to define the MSD, some of the proponents defined the MSD requirements for all of the IMD orders, and some of the proponents defined the MSD requirements for part of the IMD order with the NOTE saying ‘This band is subject to IMDx also which MSD is not specified’.

- After the IMD order rules/guidance introduced, it was applied for the new introduced band combination, but leave the legacy band combination unchanged.

- For HPUE band combination, ideally the IMD order should be the same as PC3 band combination. However, due to HPUE and the corresponding PC3 band combination are introduced in different meeting times (even in different release), some HPUE band combinations (EX. CA_n5-n77) would be objected to the above rule while the corresponding legacy PC3 band combination would not, then inconsistency between HPUE and PC3 happened. And some other HPUE band combinations still use the same IMD order as the legacy PC3 band combination to keep alignment.

- In this case mix approach for the IMD MSD order appears in the current specification, not only for PC3 band combinations, but also for HPUE, even between PC3 and HPUE.

- Observation 1: The MSD value of the lowest even and the lowest odd order IMD shall be defined If the DL band may be affected by a mix of even and odd order IMD products

- Observation 2: Before the IMD order rules/guidance introduced, there were no uniform approach for the IMD MSD order, which mainly depends on proponent, where some proponents defined the MSD requirements for all of the IMD orders, and some other proponents defined the MSD requirements for part of the IMD order with the NOTE saying  ‘This band is subject to IMDx also which MSD is not specified’.

- Observation 3: After the IMD order rules/guidance introduced, it was applied for the new introduced band combination, but leave the legacy band combination unchanged.

- Observation 4: Mix approaches for the IMD MSD order appears not only for PC3 band combinations, but also for HPUE, even between PC3 and HPUE

- For LUT approach, it is well-known to use PC3 IMD approach as the footstone. Although the existing tables will be temporary kept, the LUT approach/tables will be used in the specification in order to replace the HPUE MSD tables in the end.

- The question is how to define the HPUE IMD order in the case of all of the even and odd order IMD are defined for its corresponding legacy PC3 band combination. For example, the DL band is affected by IMD2 and IMD4 order, and both IMD2 and IMD4 MSD are defined for the legacy PC3 band combination, then for the HPUE IMD MSD with the LUT approach, only IMD2 MSD is defined (per the above rule) or both IMD2 and IMD4 MSD are defined?

- For the HPUE LUT approach, we do believe that the above rule shall still be valid, which means the lowest even and the lowest odd order IMD shall be defined regardless of all of the even and odd order IMD are defined for its corresponding legacy PC3 band combination. For the above example, only define IMD2 MSD is our preference.

- To accommodate LUT approach, we think we need to clean-up the IMD orders for the legacy band combination to comply with the above rules/guidance in the latest R19 specification. In this way, it would be consistency for PC3 and HPUE LUT approach.

- Proposal 1. For HPUE band combination LUT approach, if the DL band is affected by a mix of even and odd order IMD products with same uplink/downlink allocations as specified for PC3:

- The MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined.

- Proposal 2: To clean-up the IMD orders for the legacy band combination to comply with the IMD order rules/guidance in the latest R19 specification.

# 3 Conclusion

In this contribution, we give some general consideration on HPUE IMD MSD rule. The proposals and conclusions are:

- Observation 1: The MSD value of the lowest even and the lowest odd order IMD shall be defined If the DL band may be affected by a mix of even and odd order IMD products

- Observation 2: Before the IMD order rules/guidance introduced, there were no uniform approach for the IMD MSD order, which mainly depends on proponent, where some proponents defined the MSD requirements for all of the IMD orders, and some other proponents defined the MSD requirements for part of the IMD order with the NOTE saying  ‘This band is subject to IMDx also which MSD is not specified’.

- Observation 3: After the IMD order rules/guidance introduced, it was applied for the new introduced band combination, but leave the legacy band combination unchanged.

- Observation 4: Mix approaches for the IMD MSD order appears not only for PC3 band combinations, but also for HPUE, even between PC3 and HPUE

- Proposal 1. For HPUE band combination LUT approach, if the DL band is affected by a mix of even and odd order IMD products with same uplink/downlink allocations as specified for PC3:

- The MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined.

- Proposal 2: To clean-up the IMD orders for the legacy band combination to comply with the IMD order rules/guidance in the latest R19 specification.

# 4	Reference

[1] R4-2505143, WF on MSD simplifcation for HPUE, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE

[2] R4-2507935, WF on new MSD rules, Skyworks, Murata Manufacturing Co Ltd., Nokia, Samsung, Qualcomm, Apple, Vivo, ZTE, Mediatek, Huawei, Oppo

[3] R4-2506383, Some general considerations on R19 HPUE IMD MSD rule, ZTE Corporation, Sanechips

[4]R4-2306588, TP for TR 38.846 on test burden reduction for multiple MSD in band combinations Skyworks, ZTE