3GPP TSG-RAN WG4 Meeting #116                                        R4-2511226

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:              Discussion on AI mobility regarding RAN4 requirements impact

Agenda item:			7.19.3.1

Document for:	Discussion

1. Introduction

Some AI mobility aspects regarding RAN4 requirements impact were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Issue 2-1: Relative RSRP accuracy definition

Issue 2-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

If we take a look at the current relative accuracy definition no matter for inter-frequency or intra-frequency, below：

<Legacy absolute/relative accuracy specified in TS 38.133> - FR1 intra-frequency intra-cell: The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1 - FR1 inter-frequency inter-cell: - The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell. - (relative accuracy) The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1. - FR2 intra-frequency intra-cell: The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

, the “keywords” we focus on is：RSRP measured from variant cells. While, if to follow the existing definition literally, uncertainty of time difference between the two reported RSRPs actually was implicitly contained in the non-AI case. However, recap some discussions in the very early release, it was confirmed that the relative accuracy requirements are applicable between measurements of a cell (e.g., at time instant T1) and later measurements (e.g., at time instant Tn). On the other hand, from our understanding, UE should report results based on the measurement information, but it is up to the network implementation how to process the reported values. In this sense,

Proposal 1: There is no need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy in AI mobility.

  - Inter-frequency prediction scenario

From our understanding, for inter-frequency prediction, RAN4 can to study the impact on MG reduction in measurement period/scheduling restriction based on the measurement overhead reduction goal. Besides, since measurements of the co-located cell (s) can be reduced, the CSSF requirements can be enhanced.

Furthermore, the prediction can be used to reduce the inter-frequency measurement the UE needs to take, so I. The measurement latency for the certain MO (s) can be reduced, since in the legacy, MG is used to perform RF tuning, and the simplest way to complete measurements of all the measurement objects in in turn; II. The data interruption time can be reduced and scheduling restriction can be relaxed, since in the legacy, during MGs, the UE can not receive/transmit data of the serving cell.

Proposal 2: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

Proposal 3: For the RRM impact on prediction window, at least the following may need to be discussed

- Scheduling restriction

- Measurement latency for MO (s), MO (s) reduction

For scenarios FR1 to FR1 intra-frequency temporal domain case B and FR1 to FR1 inter-frequency inter-cell (frequency domain), cell specific and cluster approaches are considered, thereionto, cluster approach including spatial-cluster and frequency-cluster refers to the prediction method, where the number of cells for measurements and/or predictions is more than one. Based on RAN2 evaluation results, cluster approach outperforms the cell-specific approach not only in temporal domain prediction but also in frequency domain prediction. And based on the simulation analysis, the size of cluster and methods of establishing/selecting cluster input may bring different impact on prediction performance

Besides, if cluster based approach is introduced, two cells based definition considered for L3-RSRP accuracy requirement maybe is not enough. In this sense,

Proposal 4: If to consider RSRP prediction accuracy, the applicability of accuracy definition should be further discussed by taking at least the following into account:

- Cell-specific and/or cluster prediction approaches

  - Measurement error modelling

#114-bis Issue 2-1-7: Impacts of measurement error on prediction accuracy WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

- The similar issue is discussed in AI BM. In such discussions, for RF error, the truncated Gaussian distribution is built, and for BB error, the LLS simulation approach is considered.

- In AI mobility, inter-cell/inter-frequency case is taken into consideration. The received signal power highly depends on the receive antenna pattern, antenna alignment and propagation environment, while the wireless channel propagation has different properties across neighbours, that means distribution standard deviation value can vary widely across cells. In this sense, how to build measurement error model considering the contribution of neighbour cell is another AI mobility-specific challenge

- Fast fading channel instead of static channel is considered in AI mobility. At the same time, based on the observation that as the UE gets closer to the signal source, the RSRP values change more dramatically coming from the signal propagation. In this sense, SNR/SINR side condition maybe is worth discussing when to build the BB error model.

From this, we slightly prefer that

Proposal 5: Regarding measurement error model on prediction accuracy in AI mobility

- For RF error, take the outcome of the error modelling approach in PHY AI/ML as a baseline

- For BB error, LLS approach involving SNR/SINR is considered to determine the distribution

- Different properties of channel propagation across neighbor cells would be captured as a factor

- Conclusion

In this contribution, we provided our viewpoints on general discussion on AI mobility regarding RAN4 requirements impact. The following observations and proposals are obtained:

Proposal 1: There is no need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy in AI mobility

Proposal 2: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

Proposal 3: For the RRM impact on prediction window, at least the following may need to be discussed

- Scheduling restriction

- Measurement latency for MO (s), MO (s) reduction

Proposal 4: If to consider RSRP prediction accuracy, the applicability of accuracy definition should be further discussed by taking at least the following into account:

- Cell-specific and/or cluster prediction approaches

Proposal 5: Regarding measurement error model on prediction accuracy in AI mobility

- For RF error, take the outcome of the error modelling approach in PHY AI/ML as a baseline

- For BB error, LLS approach involving SNR/SINR is considered to determine the distribution

- Different properties of channel propagation across neighbor cells would be captured as a factor

4 References

[1] R4-2508268 WF on AI/ML Mobility Part 1