# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511437

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			A-IoT device requirement overview

Agenda item:		7.22.3.2

Document for:		Discussion

# Introduction

In this paper, we present our view on the A-IoT device RF requirements.

# RF requirement for A-IoT UE

## Tx requirement

In WF [3], the TX requirement relating to backscatter transmission is quoted below:

Issue 3-1-1: Transmit output power

Agreement in RAN4#115:

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

In our companion paper link budget analysis, the maximum of the backscatter loss is around 40 dB assuming there is no reader de-sensitization. Consider the reader de-sensitization 20dB to 30 dB, the needed backscatter loss will be around 10 dB to 20 dB. So we suggest 10 dB should be used in the specification.

With reader de-sensitization of 20 dB to 30 dB, the backscatter loss should be 10 dB to 20 dB range.

10 dB backscatter loss should be specified for backscatter loss.

In another paper[3], the CDF curve is simulated assuming a dipole antenna. Last meeting the majority of the companies view is to declare the device peak antenna gain direction. As the backscatter loss only relate to the antenna pattern when antenna impedance between two modulation states are fixed, it can be observed in Figure 1 that there are a wide range of the directions of UE can declare to meet a good backscatter loss number. A dipole antenna is omnidirectional in the plane perpendicular to the wire axis with a donut shape, the antenna gain falling to zero on the axis. When the declaration method is used, manufacture should describe the refence plane, and the test should be done in the reference plane , e.g perpendicular to the conductor/wire axis. Due to the different implementation, there is no need to specify the detail except the reference plane.

The manufacturer declare the peak antenna gain direction using a reference plane.

Figure 1:CDF curve of modulation efficiency (Full sphere)

Based on analysis above, the backscatter loss for OTA requirement could be defined as similar as min EIRP of spherical coverage in NR FR2, but with a different metric, for example:

The maximum backscatter loss is defined as the ratio of the input CW power at the device antenna to the backscatter power at the device antenna. The backscatter power is defined as mean filtered power measured over the duration of the D2R signal at the UE declared directionand. The basckscatter loss shall be met in Table 6.2.1.1-1 with the test parameters set in Table 6.2.1.1-2.

Note: UE declare the test direction in a reference plane.

Table 6.2.1.1-1: Maximum backscatter loss

| Operating band | Maximum backscatter loss |
| --- | --- |
| N8 | [10] |

Table 6.2.1.1-2: Test configuraton

| Test parameter | value |
| --- | --- |
| D2R channel bandwidth (kHz) | (2000*(1+Tb/(2Tc))/Tb)* 1.1/0.9 |
| CW frequency (MHz) | According to TS 38.192 |
| Filter centre frequency | CW frequency |
| Filter frequency offset | 1/Tc |
| Filter bandwidth | 2/Tb *1.1 |

Specify the backscatter loss using the text above.

Issue 3-3-1: SEM requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- The applicable frequency range for SEM: from the CBW edge to [OOB boundary]

- Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

Issue 3-3-1: spurious emission requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- OOB boundary

  - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth

  - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW

  - Other options are not precluded

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

For the SEM requirement, from the deployment scenario, operator would deploy a single carrier in uplink and the coexisting between other operators is not assumed in coexisting study phase. From this perspective, there is no need to specify the SEM requirement.

No SEM is required.

For the spurious requirements, it is the regulatory requirements and for 900MHz, the -36 dBm/100kHz applies. Such regulation requirement should apply to the A-IoT system. As the device is passive device and the “output” power is reflection of the received CW signal power, such backscatter power is relative to the received power and not a absolute power level defined in legacy NR UE. From this aspect, once the spurious emission is calculated using certain parameters, e.g maximum input CW power, the backscatter loss range, the spurious can be calculated in a predefined way. The boundary of it can also be decided.

For example, the CW maximum output is 33 dBm, the min distance between CW and device is 1meter, the received power on device antenna is 33-31.5 dB=1.5 dBm,   consider the OOK backscatter loss between 6 dB to 10 dB, the maximum backscatter power is -4.5 dBm. This is the reference signal power.  RAN1 defines the Tb (us) in a range below:

Tb=[266.67 133.33 66.67 33.33 16.67 8.33 4.17 1.39 ];

So the min bandwidth is 1/(266.67/2)=7.5 kHz.  Scaling this to the 100kHz measurement bandwidth, the reference power in 100kHz measurement bandwidth is -4.5 + 10log(100/7.5)= 6.7 dBm/100kHz.

The PSD curve of Tb=266.67 and Tc=1.04 us is illustrated in Figure 2 and Figure 3. The measurement bandwidth is 1 Hz is Figure 2 and 100kHz in Figure 3. To be easily identify the wanted signal, the sin wave upconversion is used as a reference.  It can be observed that 30 dB attenuation can be achieved at 30MHz offset. For Tc=0.69 case, 26 dB attenuation relative to the reference signal power is achieved at frequency offset of the 29 MHz. To meet the -36 dBm, a large frequency offset is needed. However, there is no regulatory guidance on the boundary and we think we need to follow the ETSI regulatory requirement on the OOB boundary.

The ERC 74-01 is the unwanted emission in spurious domain and the same -36dBm/100kHz applies to the short range device. This is quoted in Figure below. In the same standard, the 250% NB is recommended in specification and quoted in Figure 3. In the mentioned ITU-R F.1191, the necessary bandwidth is defined equal to OBW as one example.

ERC 74-01 recommend 250% necessary bandwidth for OOB boundary

ITU-R F.1191 recommend necessary bandwidth equal to OBW.

Figure 2: ERC 74-1 on spurious limit

Figure 3: ERC 74-1 on boundary recommendation

Figure 4: the unwanted emission and bandwidth in ITU-R F1191

The device spurious emission level depends on the input power CW level and this will be decided when dimensioning network. As such it is not related to the device capability but up to network planning, for example, limiting the CW input power on device by proper network planning can meet the -36 dBm/100kHz in any predefined frequency offset. Therefore, there is a need to lower the maximum input power to device 1 so spurious limit can be met with certain boundary (e.g 250% NB).

In Table below, we listed the OBW (99% Tx mean total power), D2R BW (defined last meeting), wanted signal level when the 250% boundary defined with OBW meet the -36 dBm/100kHz. It can be observed that the backscatter power should be maximum -2.45 dBm/100kHz for Tb=266.67 case and Tc=133.33 us case. Comparing to the 6.7 dBm/100kHz derived from 1 m distance to CWT node, this means using the 250%OBW boundary, there need additional 9.1 dB, this can be done either reduce the CWT power from 33 dBm to 23 dBm , or increase the device distance from 1 m to 4 m and the pathloss at 900MHz is 43.5 dB.

Reduce the CWT power to 23 dBm or increase min distance from device to CWT to 4 m for CWT output power of 33 dBm.

As the spurious requirement is regulatory requirement and we suggest using the OBW as necessary bandwidth so it can be tested properly for device spurious. In this regards, the device channel bandwidth will be defined differently with BS D2R CBW.  As the OBW too large for some Tb and Tc combination (e.g Tc=1.04 and 0.69 us) for n8 as it is larger than n8 uplink spectrum. We suggests not defining these combination for D2R channel bandwidth for system parameters.

Introduce the D2R channel BW equals to OBW for spurious test.

Introduce the D2R channel BW at device specification, exclude the Tc =1.04 and 0.69 case.

Introduce the maximum backscatter power of – 5.5 dBm (with 3 dB margin of -2.45 dBm)

Table 1: OBW, DR2 BW and Wanted signal level to meet -36dBm/100kHz @ 250%OBW frequency offset

|  | Tc=133.33 |  |  | Tc=66.7 |  |  | Tc=33.33 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb= 266.67 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 18.33 | 163.49 | -2.45 | 27.50 | 645.30 | -2.56 | 45.83 | 1370.35 | -2.56 |
|  | Tc=16.67 |  |  | Tc= 8.33 |  |  | Tc= 4.17 |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 82.50 | 2462.23 | 6.12 | 155.83 | 4997.19 | 6.12 | 302.50 | 9833.37 | 12.39 |
|  | Tc= 2.08 |  |  | Tc= 1.04, |  |  | Tc= 0.69 |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 595.83 | 19653.92 | 31.22 | 1182.49 | 37507.34 | 31.30 | 1778.31 | 53652.42 | 11.82 |
|  | Tc=66.67 |  |  | Tc=33.33 |  |  | Tc=16.67 |  |  |
| Tb=133.33 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 91.67 | 207.46 | 6.14 | 55.00 | 1539.63 | 4.85 | 45.83 | 2462.73 | 6.14 |
|  | Tc8.33 |  |  | Tc= 4.17 |  |  | Tc= 2.08 |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 165.00 | 5188.09 | 10.39 | 311.67 | 9840.29 | 11.33 | 605.02 | 19662.69 | 30.69 |
|  | Tc= 1.04 |  |  | Tc= 0.69, |  |  |  |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |  |  |  |
|  | 1191.70 | 37514.09 | 31.01 | 1778.38 | 53654.72 | 12.32 |  |  |  |
|  | Tc=33.33 |  |  | Tc=16.67 |  |  | Tc=8.33 |  |  |
| Tb= 66.67 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 73.33 | 359.83 | -1.24 | 109.99 | 2471.47 | 6.17 | 183.32 | 5490.93 | 8.55 |
|  | Tc=4.17 |  |  | Tc= 2.08 |  |  | Tc= 1.04 |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 329.98 | 9886.65 | 5.90 | 623.30 | 19677.76 | 22.11 | 1209.94 | 37513.23 | 21.82 |
|  | Tc= 0.69, |  |  |  |  |  |  |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |  |  |  |  |  |  |
|  | 1796.58 | 53663.66 | 14.86 |  |  |  |  |  |  |
|  | Tc=16.67 |  |  | Tc=8.33 |  |  | Tc=4.17 |  |  |
| Tb= 33.33 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 146.68 | 604.68 | 1.01 | 220.02 | 6107.25 | 7.59 | 366.70 | 10328.16 | 17.69 |
|  | Tc= 2.08 |  |  | Tc= 1.04, |  |  | Tc= 0.69 |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 660.07 | 19751.66 | 16.36 | 1246.79 | 37540.43 | 19.38 | 1833.52 | 53683.05 | 17.98 |
|  | Tc= 8.33 |  |  | Tc=4.17 |  |  | Tc=2.08 |  |  |
| Tb= 16.67 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 293.27 | 1206.55 | 3.88 | 439.91 | 10722.37 | 16.44 | 733.19 | 20666.82 | 19.44 |
|  | Tc= 1.04 |  |  | Tc=0.69, |  |  |  |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |  |  |  |
|  | 660.07 | 38802.00 | 12.76 | 1246.79 | 53778.75 | 24.89 |  |  |  |
|  | Tc= 4.17 |  |  | Tc=2.08 |  |  | Tc=1.04 |  |  |
| Tb= 8.33 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 586.90 | 2468.70 | 5.76 | 880.35 | 21656.32 | 15.31 | 1467.25 | 39436.84 | 22.42 |
|  | Tc= 0.69 |  |  |  |  |  |  |  |  |
|  | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |  |  |  |  |  |  |
|  | 2054.15 | 56457.06 | 21.02 |  |  |  |  |  |  |
|  | Tc= 2.08 |  |  | Tc=1.04 |  |  | Tc=0.69 |  |  |
| Tb= 4.17 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |
|  | 1172.40 | 4667.25 | 7.09 | 1758.59 | 41445.33 | 15.77 | 2344.79 | 56712.04 | 19.96 |
|  | Tc= 1.04 |  |  | Tc=0.69 |  |  |  |  |  |
| Tb= 1.39 | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) | D2R BW (kHz) | OBW (kHz) | Sig level (dBm/100kHz) |  |  |  |
|  | 1758.59 | 14855.72 | 13.58 | 3517.19 | 14933.20 | 20.71 |  |  |  |

Figure 5: PSD of Tb=266.67 us and Tc =1.04 us (measurement bandwidth 1Hz)

Figure 6:PSD of Tb=266.67 us and Tc =1.04 us (measurement bandwidth 100kHz)

Figure 7:PSD of Tb=266.67 us and Tc =0.69 us (measurement bandwidth 100kHz)

## Rx requirements

Issue 3-4-1: Reference sensitivity

Agreement in RAN4#115:

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

- FFS for the FRC, consider following as starting point:

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

We think -34 dBm REFSENS requirement is quite relaxed. Manufacture should declare a better REFSENS than this and network can use lower transmit power to save energy. The REFSENS is conducted requirement and EIS is the OTA requirement. If the dipole antenna type is assumed in the device, the total isotropic sensitivity (TIS) or the averaging the EIS in one or several directiosn ( elevation and azimuth angle) can be considered as the antenna gain difference for a donut-shaped antenna pattern is maximum 2.15 dBi.

Averaging EIS can be specified instead of the maximum EIS if a dipole antenna is assumed in device.

# Conclusions

In this contribution, we present our overview on the A-IoT device RF with below proposal and observations:

Observation 1 With reader de-sensitization of 20 dB to 30 dB, the backscatter loss should be 10 dB to 20 dB range.

Proposal-1: 10 dB backscatter loss should be specified for backscatter loss.

Proposal-2: The manufacturer declare the peak antenna gain direction using a reference plane.

Proposal-3:Specify the backscatter loss using the text above.

Proposal-4: No SEM is required.

Observation 2 ERC 74-01 recommend 250% necessary bandwidth for OOB boundary

Observation 3 ITU-R F.1191 recommend necessary bandwidth equal to OBW.

Observation 4 Reduce the CWT power to 23 dBm or increase min distance from device to CWT to 4 m for CWT output power of 33 dBm.

Proposal-5: Introduce the D2R channel BW equals to OBW for spurious test.

Proposal-6:Introduce the D2R channel BW at device specification

Proposal-7: Introduce the maximum backscatter power of – 5.5 dBm (with 3 dB margin of -2.45 dBm)

Proposal-8: Averaging EIS can be specified instead of the maximum EIS if a dipole antenna is assumed in device.

# References

- R4-2508116, WF on A-IOT device requirements, Huawei

- R4-2504399, A-IoT device requirement overview, Ericsson

- R4-2504400 ,A-IoT device RF requirement overview, Ericsson