3GPP TSG-RAN WG4 Meeting #116	R4-2511610

Bengaluru City, IN, Aug 25 – Aug 29, 2025

Agenda Item: 7.20.4.2

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. The WID [1] is divided into two parts to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “Other RRM requirements”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul.

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

## 2.1 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

In RAN4#115 meeting, it was agreed that the number of samples for CJT calibration report will be chosen based on achievable accuracy in terms of delay and frequency offsets. We provided the simulation result in [3] (also provided in appendix A) based on largest report resolution for both delay and frequency offsets (MD = 256, MFO = 256). From previous meetings, it is noticed that there is some misalignment in the collected results from companies, specifically between UE vendors’ results and NW venders’ results. However, we understand that this misalignment can be coming from the adapted algorithm by each company to produce these results. We also understand that this misalignment can be mitigated if some additional optimization is added to these algorithms. However, there is a trade-off between complexity and performance, optimizing the performance of the algorithm while compromising on the complexity might not be the best option always. This might also explain why the results are aligned among the UE vendors.

From our simulation results (provided in appendix A) for TO/FO estimation error, it can be observed that the accuracy of quantization (reported interval) is improved significantly when the number of samples is increased from 1 to 3. However, accuracy gain is not significant when number of samples 3 increase to 5 for both timing offset and frequency offset error estimation. In our view, there’s no strong reason to consider two more samples to define accuracy for 5 samples since there’s not much improvement in the accuracy from number of 3 samples to 5 samples.

Proposal 1: RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

Given this is the last meeting for core part, if RAN4 cannot reach consensus on the number of samples used for CJT reporting for delay and frequency offset, RAN4 not to define the RRM requirements for CJT.

Proposal 2: If RAN4 cannot reach consensus on the number of samples used for delay and frequency offset, RAN4 to decide not to define the corresponding RRM requirements for CJT.

According to WID [1] and last RAN4 agreement, we provided our view on the measurement period and scheduling restriction for FR1 on both FDD and TDD. Since delay offset and frequency offset are measured from periodic CSI-RS (Tracking RS) and aperiodic reporting on PUSCH. The legacy measurement period can be reused as baseline. E.g., measurement period = max(TReport, ceil(M*P)*TCSI-RS) for non-DRX. TReport could be zero since this is aperiodic reporting. The M (number of sample) = 3 based on our proposal 1 and reuse the legacy sharing factor(P).

Proposal 3: Reuse the legacy L1-RSRP CSI-RS measurement period as baseline for delay and frequency offset measurement.

- TReport = 0

- M (number of samples) = 3

- Reuse legacy sharing factor (P)

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration? Way forward: - Option 1: The scheduling restriction is specified as: - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting. - Option 2: No scheduling restrictions needed to be applied for CJT reporting. Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration? Way forward: - Option 1: Reuse the legacy measurement restriction in FR1 for delay and frequency offset measurement. - Measurement restriction does apply, - If UE does not support simultaneousRxDataSSB-DiffNumerology and - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols. - Measurement restriction does not apply, - If UE support simultaneousRxDataSSB-DiffNumerology or - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH. - Option 2: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

Since delay and frequency offset measurement is from periodic CSI-RS (Tracking RS) in legacy. The CSI-RS based L1 measurement restriction for FR1 in legacy could be reused as baseline in our understanding. The legacy measurement restriction is provided as reference on the following:

- Measurement restriction does apply,

  - If UE does not support simultaneousRxDataSSB-DiffNumerology and

  - SSB for RLM, BFD, CBD or L1-RSRP has different SCS than CSI-RS for L1-RSRP measurement and they are fully or partially overlapped on symbols.

- Measurement restriction does not apply,

  - If UE support simultaneousRxDataSSB-DiffNumerology or

  - SSB for RLM, BFD, CBD or L1-RSRP has the same SCS as CSI-RS for L1-RSRP measurement.

Proposal 4: Reuse the legacy CSI-RS based L1 measurement restriction as baseline in FR1 for delay and frequency offset measurement.

- Measurement restriction does apply,

  - If UE does not support simultaneousRxDataSSB-DiffNumerology and

  - SSB for RLM, BFD, CBD or L1-RSRP with different SCS as CSI-RS (TRS) for delay and frequency offset measurement and they are fully or partially overlapped on symbols in FR1.

- Measurement restriction does not apply,

  - If UE support simultaneousRxDataSSB-DiffNumerology or

  - SSB for RLM, BFD, CBD or L1-RSRP with the same SCS as CSI-RS (TRS)for delay and frequency offset measurement in FR1.

Follow similar principle as above measurement restriction, we reuse L1-RSRP scheduling restriction on FR1 in legacy as baseline. The legacy scheduling restriction is provided as reference on the following:

- There are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

- For UEs which support simultaneousRxDataSSB-DiffNumerology, there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement.

- For UEs which do not support simultaneousRxDataSSB-DiffNumerology, the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

  - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSIRS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

Proposal 5: Reuse the legacy L1-RSPR scheduling restriction in FR1 as baseline for delay and frequency offset measurement:

- There are no scheduling restrictions due to delay and frequency offset measurement performed on CSI-RS (tracking RS) with the same SCS as PDSCH/PDCCH in FR1.

- For UEs which support simultaneousRxDataSSB-DiffNumerology, there are no restrictions on scheduling availability due to delay and frequency offset measurement based on CSI-RS (tracking RS).

- For UEs which do not support simultaneousRxDataSSB-DiffNumerology, the following restrictions apply due to delay and frequency offset measurement based on CSI-RS (tracking RS).

  - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSIRS for CQI on symbols corresponding to the CSI-RS (tracking RS) indexes configured for delay and frequency offset measurement.

## 2.2 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

In this section we discuss the remaining RRM issues for Scenario#1, note that:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

For Scenario#1:

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: FFS Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG. - Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL. Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements? Way forward: FFS

The RACH requirements for 2TA for Scenario#1 can reuse the legacy timing requirement where the reference point for PRACH transmission triggered by PDCCH order,

- if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

- if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

Proposal 6: The RACH requirements for 2TA for Scenario#1 can reuse the legacy timing requirement where the reference point for PRACH transmission triggered by PDCCH order,

- if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

- if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

For Issue 2-2, the gradual timing requirements for two TAs can be updated similar to legacy requirements for (multi-DCI based multi-TRP operation with two TAs). The gradual timing adjustment applies for each TAG and shall be met for each TAG.

Proposal 7: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

For issue 2-3, since in Scenario#1 there is only single DL in asymmetric DL sTRP/UL mTRP deployment, there is no need to define MRTD requirement. On the other hand, for MTTD requirements we can reuse Rel 18 values for Rel 19.

Proposal 8: MTTD requirements for Scenario#1 can reuse Rel 18 values.

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Proposal 1: RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

Proposal 2: If RAN4 cannot reach consensus on the number of samples used for delay and frequency offset, RAN4 to decide not to define the corresponding RRM requirements for CJT.

Proposal 3: Reuse the legacy L1-RSRP CSI-RS measurement period as baseline for delay and frequency offset measurement.

- TReport = 0

- M (number of samples) = 3

- Reuse legacy sharing factor (P)

Proposal 4: Reuse the legacy CSI-RS based L1 measurement restriction as baseline in FR1 for delay and frequency offset measurement.

- Measurement restriction does apply,

  - If UE does not support simultaneousRxDataSSB-DiffNumerology and

  - SSB for RLM, BFD, CBD or L1-RSRP with different SCS as CSI-RS (TRS) for delay and frequency offset measurement and they are fully or partially overlapped on symbols in FR1.

- Measurement restriction does not apply,

  - If UE support simultaneousRxDataSSB-DiffNumerology or

  - SSB for RLM, BFD, CBD or L1-RSRP with the same SCS as CSI-RS (TRS)for delay and frequency offset measurement in FR1.

Proposal 5: Reuse the legacy L1-RSPR scheduling restriction in FR1 as baseline for delay and frequency offset measurement:

- There are no scheduling restrictions due to delay and frequency offset measurement performed on CSI-RS (tracking RS) with the same SCS as PDSCH/PDCCH in FR1.

- For UEs which support simultaneousRxDataSSB-DiffNumerology, there are no restrictions on scheduling availability due to delay and frequency offset measurement based on CSI-RS (tracking RS).

- For UEs which do not support simultaneousRxDataSSB-DiffNumerology, the following restrictions apply due to delay and frequency offset measurement based on CSI-RS (tracking RS).

  - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSIRS for CQI on symbols corresponding to the CSI-RS (tracking RS) indexes configured for delay and frequency offset measurement.

Proposal 6: The RACH requirements for 2TA for Scenario#1 can reuse the legacy timing requirement where the reference point for PRACH transmission triggered by PDCCH order,

- if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

- if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

Proposal 7: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

Proposal 8: MTTD requirements for Scenario#1 can reuse Rel 18 values.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2508272 WF on RRM requirements for NR_MIMO_Ph5_Part2 (MTK, RAN4#115)

[3] R4-2508292 Collection of simulation results for CJT calibration in NR_MIMO_Ph5_Part2

# Appendix A

Simulation assumption and results for CJT calibration:

Table 1. Simulation parameters for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

- Timing offset simulation results:

Table 2. Simulation result for timing offset error estimation.

| SCS | SNR | Timing offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 0.5CP | 29 | 19 | 18 |
| 15KHz | 0 | 0.5CP | 17 | 12 | 10 |
| 15KHz | 3 | 0.5CP | 10 | 7 | 6 |
| 30KHz | -3 | 0.5CP | 30 | 19 | 17 |
| 30KHz | 0 | 0.5CP | 16 | 11 | 10 |
| 30KHz | 3 | 0.5CP | 14 | 9 | 9 |

- Frequency offset simulation results:

Table 3. Simulation result for frequency offset error estimation.

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 65 | 39 | 29 |
| 15KHz | 0 | 350 Hz | 39 | 24 | 18 |
| 15KHz | 3 | 350 Hz | 26 | 16 | 12 |
| 30KHz | -3 | 350 Hz | 133 | 71 | 60 |
| 30KHz | 0 | 350 Hz | 83 | 48 | 37 |
| 30KHz | 3 | 350 Hz | 52 | 31 | 24 |