3GPP TSG-RAN WG4 Meeting # 116		 	    R4-2512144

Bengaluru, India, August 25th - 29th, 2025

Title: 	WF on RRM requirements for NR_XR_Ph3

Agenda Item:	7.26.1

Source: 	Nokia

Document for:	Approval

# Topic 1 - RRM core requirements

## Sub-topic 1-1 - Scenarios for measurement skipping

### Issue 1-1-1: UE power class

<Agreement>

- Measurement gap cancelation applies for all power classes, except for FR2 HST (PC6).

## Sub-topic 1-2 – Measurements with gaps

### Issue 1-2-1: Type 2 gaps

<Agreement>

- No further discussion needed. Discussion based on CR.

## Sub-topic 1-3 – XR skipping requirements impact

### Issue 1-3-1: Principles for measurement delay calculation

<Agreement>

- No further discussion needed.

### Issue 1-3-2: Formulas for measurement delay extension without DRX

<Agreement>

- Max( 200 ms, ( 8 + Lcancel) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

- where Lcancel is the number of cancelled MG occasions in the measurement period

### Issue 1-3-3: Formulas for measurement delay extension with DRX

<Agreement>

- Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+ Lcancel) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+ Lcancel) ´ DRX cycle ´ CSSFinter

  - Lcancel is the number of DRX cycles where at least one gap is cancelled

### Issue 1-3-4: Formulas for measurement delay extension for inter-RAT

<Agreement>

- T1 = ceil( Lcancel×Kgap_EUTRA)×MGRP

- Extend by 480 * ceil( T1 / 480 ) × CSSFinter-RAT

### Issue 1-3-5: Formulas for measurement delay extension of concurrent gaps

<Agreement>

- Max( 200 ms, ceil( (8 + Lcancel) * Kgap) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

- where Lcancel is the number of cancelled MG occasions in the measurement period

### Issue 1-3-6: Formulas for measurement delay extension of L1/L2 triggered mobility

<Agreement>

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil((M+Lcancel) * Kgap) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil((M+Lcancel) 1.5 * Kgap) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil((M+Lcancel) * Kgap) DRX cycle CSSFinter |

### Issue 1-3-7: Conditions for measurement skipping

<Agreement>

- No conditions are defined.

### Issue 1-3-8: Measurement gap configurations relevant to skipping

<Agreement>

- This issue is closed.

## Sub-topic 1-4 – Additional signalling aspects

### Issue 1-4-1: Skipping indication on frequency domain

<Agreement>

- No further discussion needed. Discussion based on CR.

## Sub-topic 1-5 – UE capabilities

### Issue 1-5-1: UE capabilities for XR – tables

<Agreement>

| 53.NR_XR_Ph3 | 53-1 | Recommended ratio of gap occasions for cancelation | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation | 64-1 | Yes | N/A | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Sub-topic 1-6 – CR handling

### Issue 1-6-1: Specification impact

- Background information:

  - Agreement from RAN4 #114 R4-2502700

Issue 1-3: L3 Measurements without gaps with scheduling restrictions Issue 1-4: L1 measurements with scheduling restrictions and procedures Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

<Agreement>

- For partially overlapped case

  - Capture clarification on the scheduling restriction clauses for L3 measurement without gap that no scheduling restriction applies on the canceled gaps

  - For L1 and L3 without gaps, clarification on the Kp that the calculation doesn’t include cancelled gaps on the Navailable

# Topic 2 - RRM performance requirements

## Sub-topic 2-2 – Test case list

### Issue 2-2-1: Scenarios for the definition of the test cases

<Way forward>

Discuss the proposals

- Proposal 1: it is proposed to define test cases for NR-SA.

- Proposal 2: it is proposed to define test cases for FR1 and FR2-1.

- Other proposals not precluded.

### Issue 2-2-2: Accuracy requirements

<Agreement> RAN4 confirms the agreement from RAN4 #113 in R4-2420103:

- Reuse existing measurement accuracy requirements for UE under XR operation.

### Issue 2-2-3: Per UE and per FR gaps

<Way forward>

Discuss the following options

- Option 1: Test cases are specified only for per-UE MGs

- Option 2: Test cases are specified for both per-UE and per-FR MGs

### Issue 2-2-4: Type 1 and Type 2 gaps

<Way forward>

Discuss the following options

- Option 1: Test cases are defined for both Type 1 and Type 2 gaps

- Option 2: Test cases are defined only for Type 1 gaps

### Issue 2-2-5: DRX proposals

<Way forward>

Discuss the following proposals:

- Proposal 1: Define the test cases for the cases without DRX and with DRX.

- Proposal 2: Test cases without DRX should have higher priority than with DRX.

  - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG.

  - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority.

  - Focus on non-DRX test cases for Type 2 MGs.

- Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both.

### Issue 2-2-6: Test target

<Way forward>

- RRM test cases should verify

  - Proposal 1: accuracy requirements

  - Proposal 2: delay extension with gap cancellation

  - Proposal 3: Reception/transmission without scheduling restrictions during cancelled gap occasions

  - Proposal 4: The UE processing timeline of receiving the DCI-based MG skipping indication

### Issue 2-2-7: List of test cases

<Way forward>

- Define test cases for

  - Proposal 1: Intra-frequency measurements with gaps

  - Proposal 2: Inter-frequency measurements with gaps

  - Proposal 3: Inter-RAT E-UTRAN FDD measurements

  - Proposal 4: Inter-RAT E-UTRAN TDD measurements

  - Proposal 5: Inter-frequency L1-RSRP with MG

  - Proposal 6: L3 and L1 measurements without gaps

## Sub-topic 2-3 – Test case configuration

### Issue 2-3-1: Type 2 test configurations

<Way forward>

- Proposal 1: For Type 2 MGs test cases, include 2 gaps with partial overlap.

- Proposal 2: For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e.

  - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

  - High priority MG pattern #2 with MGRP=80ms, offset=4ms

- Other options are not precluded

## Sub-topic 2-4 – MG cancellation model

### Issue 2-4-1: Cancelation model for XR test cases

<Way forward>

- Proposal 1: Measurement gap cancelation is determined randomly by the test equipment.

- Proposal 2: For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern.

- Other options are not precluded.

### Issue 2-4-2: Cancelation ratio for Type 1 gaps

<Way forward>

- Option 1:

  - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

  - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

- Option 2: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

### Issue 2-4-2: Cancelation ratio for Type 2 gaps

<Way forward>

- Option 1: For Type 2 MGs test cases, configure cancelation ratio as:

  - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%

  - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset

- Other options are not precluded

## Sub-topic 2-5 – DCI aspects

### Issue 2-5-1: Toffset between DCI and start of MG

<Way forward>

- Option 1: Define test cases for both 3ms offset and 5ms offset.

- Option 2: Define test cases for 5ms offset

### Issue 2-5-2: Types of DCI with cancellation indication

<Way forward>

- Proposal 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

- Other options not precluded

### Issue 2-5-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

<Way forward>

- Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

- Other options not precluded